### 7.1.4	H.323 URL scheme

One of the alias types defined by H.323 is the url-ID, which is intended to contain standard URL schemes.. In addition, URLs are used within Annex K/H.323 and may be used in other services added to H.323 in the future.

The H.323 URL is intended to be used for address resolution of of H.323 destinations by using the DNS technology widely deployed in Internet and other IP networks.   It is composed of two parts: the user and the hostport.  The user specifies an alias for the entity, such as a user or a service, without carrying an information about the location of the entity in the H.323 world.  The hostport, on the other hand, may specify the transport address of a Gatekeeper, a Gatekeeper Identifier, or the fully qualified domain name of the Gatekeeper, along with an optional port number for RAS communication.

The H.323 URL is defined in ABNF as shown below.  Note that it utilizes the Core Rules specified in section 6.1 of [52].

H323-URL			=	"H323:" user [ "@" hostport ]

user				=	 ( unreserved / escaped )

hostport			=	host [":" port]

host				=	hostname / IPv4Address

hostname			=	*( domainlabel "." ) toplabel [ "." ]

domainlabel		=	alphanum / alphanum *( alphanum / "-" ) alphanum

toplabel			=	ALPHA / ALPHA *( alphanum / "-" ) alphanum

IPv4Address		=	1*DIGIT "." 1*DIGIT "." 1*DIGIT "." 1*DIGIT

port				=	1*DIGIT

unreserved		=	alphanum / mark

alphanum			=	ALPHA / DIGIT

mark				=	"-" / "_" / "." / "!" / "~" / "*" / "'"

/	"(" / ")" / "&" / "=" / "+" / "$" / ","

escaped			=	"%" HEXDIG HEXDIG

The user string shall be present and the hostport string is optional. If an endpoint registers with a Gatekeeper and does not provide a hostport string, the Gatekeeper may append a hostport string to the URL when it returns the endpoint's aliases in an RCF message. The endpoint shall accept the modified alias and use it when sending subsequent requests to the Gatekeeper.