| Question(s): |  | Q13/16 |  | Meeting, date: |  |  | 01 December 2010 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SG, GSI, …: |  | SG16 | WP: | 2 | Intended type of document (R-C-TD): |  |  | C |
| Source: |  | NEC, NTT, OKI |  |  |  |  |  |  |
| Title: |  | Proposed updates to HSTP.CONF-H721 (ex H.IPTV-CONF.2) "Conformance testing specification for H.721" |  |  |  |  |  |  |
| Contact: | Kazunori TANIKAWA NEC Corporation Japan |  |  |  |  | Tel: +81 3 3798 2924 Fax: Email: k-tanikawa@cq.jp.nec.com |  |  |
| Contact: |  | Masahito Kawamori NTT Japan |  |  |  | Tel: +81 468 59 2517 Fax: +81 468 55 3495 Email: kawamori.masahito[at]lab. ntt.co.jp |  |  |
| Contact: |  | Hideki Yamamoto Oki Electric Industry Co., Ltd. Japan |  |  |  | Tel: +81 48 420 7012 Fax: +81 48 420 7138 Email: yamamoto436@oki.com |  |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |  |  |

Summary

This contribution proposes to modifications on [HSTP.CONF-H721] according to the results of discussions in Q13//16 e-meetings (10, 17&24 November, 2010).

- Introduction

It is necessary to updates some texts and tables of contributions on [HTSP.CONF-H721] proposed in the e-meetings.

- Discussion and Proposal

  - Security checklist

Firstly, a following security check item “Authenticated encrypted communication channel by mutual authentication with server-side SCP functionalities using PKI” was proposed at first e-meeting (10 November).  In addition, this requirement was decomposed to following three key functions according to clause 7.3 in [X.1191] ;

- Subscriber and/or TD authentication using PKI

- Server authentication using PKI

- Service access control for Linear TV (e.g., scrambling and encrypted)

Paying attention to the last bullet, this sentence also contains two security items.

- Encryption for contents

- Encryption for access control information (e.g., ECM)

The current security check list misses the last item “Encryption for access control information”.

Secondly, “Diffie-Hellman protocol [RFC 2539] (DH)” protocol should be deleted because both [H.721] and [X.1191] does not specify the details of key exchange protocols.

Proposed revisions are as follows:

Start proposed descriptions

Security Checklist

| Category | Functions/protocols | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| Service protection | Server-side SCP functions authentication using PKI for mutual authentication | 7.2.2, 9.4.1.1 | M*1 |  |  |
|  | Service access control to restrict acquisition and access to services by using SCP functionalities (e.g., scrambling and encrypted for Linear TV) | 7.2.2, 9.4.1.1 | M |  |  |
|  | Encryption for protecting access control information | 7.2.2 9.4.1.1 | M |  |  |
|  | Secure communication using SSL/TLS when IPTV terminal devices connect to security-related application servers (e.g., connection between portal and IPTV terminal devices) | 7.2.2 | M |  |  |
|  | Communicate with CRL server for update and management of CRL | 7.2.2 | M*1 |  |  |
|  | Client certificate in PKI path structure and root certificate | 9.4.1.1 | M*1 |  |  |
|  | Share the key for encryption by key exchange protocol | 9.4.1.1 | M |  |  |
|  | Acquire the newest CRL | 9.4.1.2 | O |  |  |
| Content protection | Request an individual set of rights and keys by transmitting to sever-side SCP functionalities a message which includes the lID that identifies the targeted rights and keys | 7.2.2, 9.4.2.1 | M |  |  |
|  | Rights and keys management for VOD (e.g., An Indivudual set of rights and keys are treated during playback and encrypted content is decrypted by using the keys) | 7.2.2, 9.4.2.2 | M*2 |  |  |
|  | Rights and keys management for Linear TV (e.g., Extracting a scramble key and supplying the key and information about the conditions for use of content to the renderer) | 7.2.2,9.4.2.3 | M*2 |  |  |
|  | AES encryption with CBC and OFB for a residual block for Linear TV | 7.2.2 | M*2 |  |  |
|  | CSA encryption for Linear TV | 7.2.2 | O*2 |  |  |
| Others | SCP client restricts the viewing of certain content and the associated information according to parental control information | 9.3.5.4 | O |  |  |
|  | Audio/Video output interfaces of the IPTV terminal device provide appropriate copy protection | 10.2 | M |  |  |

Note 1 - In case wherein X.509 base certificates are used as credentials for authentication, a revocation function is required [ITU-T X.1191].

Note 2 - It is required or recommended when a relevant IPTV service is provided.

[Editor’s note 22 Sept]: Categorization of this table seems to be pre-matured. Further study is required.

End proposed descriptions

  - Updated requirements levels

Statuses of proposed checklists in [HTSP.CONF-H721] were discussed at the last e-meeting (24 November, 2010).  Following modifications are proposed again based on the result of the meeting.

Start proposed modifications

LinearTV Checklist

Table I.1: Checklist concerning formats and streaming in Linear TV

| Category | Specifications | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| Monomedia | Video MPEG-2 | 7.1.1 | C*1. |  |  |
|  | Video H.264 (AVC) | 7.1.1 | C*1 |  |  |
|  | Audio MPEG2 AAC | 7.1.1 | C*2 |  |  |
|  | Audio MPEG1 Layer II | 7.1.1 | C*2 |  |  |
|  | Audio MPEG-4 HE AAC v1 | 7.1.1 | C*2 |  |  |
|  | Dolby AC-3 | 7.1.1 | C*2 |  |  |
|  | ARIB Captioning | 7.1.1 | O |  |  |
|  | ATSC Closed Captioning | 7.1.1 | O |  |  |
|  | EBU Teletext Subtitles | 7.1.1 | O |  |  |
|  | DVB Subtitling |  | O |  |  |
| Multiplex format | MPEG-2 TS | 7.1.1 | M |  |  |
|  | TTS | 7.1.1 | O |  |  |
| Streaming | RTP | 7.1.1 | M |  |  |

Note 1 - One of video formats is required.

Note 2 – One of audio formats is required

VoD Checklist

Table I.2: Checklist concerning formats and streaming in VOD

| Category | Specifications | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| Monomedia | Video MPEG-2 | 7.1.2 | C*3 |  |  |
|  | Video H.264 (AVC) | 7.1.2 | C*3 |  |  |
|  | Audio MPEG2 AAC | 7.1.2 | C*4 |  |  |
|  | Audio MPEG1 Layer II | 7.1.2 | C*4 |  |  |
|  | ARIB Captioning | 7.1.2 | O |  |  |
|  | ATSC Closed Captioning | 7.1.2 | O |  |  |
| Multiplex format | MPEG-2 TS | 7.1.2 | M |  |  |
|  | TTS | 7.1.2 | O |  |  |
| Streaming | RTP, RTSP | 7.1.2 | M |  |  |

Note 3 – At least one of the video formats are required.

Note 4 – At least one of the audio formats are required

Terminal Device Attachment and Initialization

Table I.3 Checklist concerning network attachment protocols

| Category | Protocols | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| Monomedia | IP, ICMP | 7.2.1.1 | M |  |  |
|  | IPv6, ICMPv6 | 7.2.1.1 | O |  |  |
|  | DHCP | 7.2.1.1 | M |  |  |
|  | DNS | 7.2.1.1 | M |  |  |

Service provider discovery and service attachment

Table I.4: Checklist concerning service discovery

| Category | Specifications | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| Multiplex Format | MPEG-2 TS | 7.2.1.3 | M |  |  |
|  | TTS | 7.2.1.3 | O |  |  |
| Streaming | RTP, RTSP | 7.2.1.3 | M |  |  |
| Multicast | IGMPv2 | 7.2.1.3 | C*5 |  |  |
|  | MLDv2 | 7.2.1.3 | C*5 |  |  |
| Portal | HTTP | 7.2.1.3 | C*6 |  |  |

Note 5- At least one of them are required when Linear TV services are provided.

Note 6 – HTTP is required when portal is used in IPTV services

Content delivery

Table I.5: Checklist concerning content delivery

| Category | Protocols | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| Multicast | IGMPv2 | 9.2.2 | C*7 |  |  |
|  | MLDv2 | 9.2.2 | C*7 |  |  |
| Unicast | RTP, RTSP | 9.2.3 | C*8 |  |  |
|  | HTTP | 9.2.3 | C*9 |  |  |

Note 7- At least one of them are required when Linear TV services are provided.

Note 8 – RTP/RTSP are required when VOD services are provided

Note 9 – HTTP is required when portal is used in IPTV services

Video decoding

Table I.6: Checklist concerning video decoding by codec

| Category | Implementations | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| H.262 | 1920x1080i MP@HL | 9.3.3.1.1 | C*10 |  |  |
|  | 1440x1080i MP@HL | 9.3.3.1.1 | C*10 |  |  |
|  | 1280x720p MP@HL | 9.3.3.1.1 | C*10 |  |  |
|  | 720, 544, 480x480i MP@ML | 9.3.3.1.1 | C*10 |  |  |
| H.264 | 1920x1080i HPorMP@Level4.0 | 9.3.3.1.2 | C*10 |  |  |
|  | 1440x1080i HPorMP@Level4.0 | 9.3.3.1.2 | C*10 |  |  |
|  | 1280x720p HPorMP@Level4.0 | 9.3.3.1.2 | C*10 |  |  |
|  | 720x480i HPorMP@Level3.0/3.1/3.2 | 9.3.3.1.2 | C*10 |  |  |
|  | 720x576i (the format used in Europe) | 9.3.3.1.2 | C*10 |  |  |

Note 10 - At least one of them are required.

Audio decoding

Table I.7: Checklist concerning audio decoding by codec

| Category | Implementations | Reference | Status | Support | Remarks |
| --- | --- | --- | --- | --- | --- |
| Audio format | MPEG-2 AAC | 9.3.3.2 | C*11 |  |  |
|  | MPEG-1 Layer II | 9.3.3.2 | C*11 |  |  |
|  | Audio MPEG-4 HE AAC v1 | 9.3.3.2 | C*11 |  |  |
|  | Dolby AC-3 | 9.3.3.2 | C*11 |  |  |

Note 11 - At least one of them are required.

End proposed modifications

  - Additions of texts for references to H.721

Modification of structure of [HTSP.CONF-H721] was accepted on 24 November.  Following texts concerning references to [H.721] are proposed:

Start proposed structure

7	Conformance for Basic IPTV services

7.1	Media formats and streaming methods for Linear TV

Media formats and streaming methods for Linear TV are specified in sub-clause 7.1.1 of [ITU-T H.721]. Corresponding check items such as Table I.1 in Appendix I shall be filled-in and done verdict.

7.2	Media formats and streaming methods for VOD

Media formats and streaming methods for VOD are specified in sub-clause 7.1.2 of [ITU-T H.721]. Corresponding check items such as Table I.2 in Appendix I shall be filled-in and done verdict.

7.3	Network attachment and initialization

Network attachment and initialization are specified in sub-clause 7.2.1 and 7.2.1.1 of [ITU-T H.721].  Service discovery is specified in sub-clause 7.2.1.2 and 7.2.1.3 of [ITU-T H.721].  For more details, refer to [ITU-T H.770] and [b-HTSP.CONF-H770].  Corresponding check-list such as Table I.3 Appendix I shall be filled-in and done verdict.

7.4	Service discovery and service consumption

Specifications of service discovery are described in sub-clause 7.2.1.2 and 7.2.1.3 of [ITU-T H.721].  [ITU-T H.770] consults with the details of service discovery.  Service navigation is specified in sub-clause 7.1.3 of [ITU-T H.721].  Corresponding check-list such as Table I.4 and I.8 Appendix I shall be filled-in and done verdict.

7.5	Content delivery

Specifications of content delivery are described in sub-clause 9.2 of [ITU-T H.721].  Corresponding check-list such as Table.I.5 in Appendix I shall be filled-in and done verdict.

7.6		Media decoding formats

Specifications of video decoding and audio decoding by codec implemented in IPTV terminal devices are described in sub-clause 9.3.3.1 and 9.3.3.2 of [ITU-T H.721].  Corresponding check-list such as Table.I.6 and I.7 in Appendix I shall be filled-in and done verdict.

8	Conformance for security

8.1	Service protection

Specifications of service protection are described in sub-clause 7.2.2 and 9.4.1 of [ITU-T H.721].  For details of service protection, refer to [ITU-T X.1191] and relevant ITU-T Recommendations (e.g., [ITU-T X.509]).  Corresponding check-list such as Table.I.8 in Appendix I shall be filled-in and done verdict.

8.2		Content protection

Specifications of content protection are described in sub-clause 7.2.2 and 9.4.2 of [ITU-T H.721].  For details of content protection, refer to [ITU-T X.1191]. Corresponding check-list such as Table.I.8 in Appendix I shall be filled-in and done verdict.

8.3	Other security items

Privacy, Parental control, copy control of output interfaces are described in sub-clause 7.2.3, 9.3.5.4 and 10.2 of [ITU-T H.721] respectively.  Corresponding check-list such as Table.I.8 in Appendix I shall be filled-in and done verdict.

9	Conformance for physical interface

9.1	Input interface

Specifications concerning input interface implemented in IPTV terminal devices are described in sub-clause 10.1 of [ITU-T H.721].  Corresponding check-list such as Table.I.9 in Appendix I shall be filled-in and done verdict.

9.2	Output interface

Specifications concerning output interface implemented in IPTV terminal devices are described in sub-clause 10.2 of [ITU-T H.721].  Corresponding check-list such as Table.I.9 in Appendix I shall be filled-in and done verdict.

- Conformance for QoS

QoS for IPTV terminal devices are described in sub-clause 7.2.4 and 9.2.4of [ITU-T H.721].  For details, refer to [ITU-T H.701] and [b-ITU-T HTSP.CONF-H701].  Corresponding check-list are treated in [b-ITU-T HTSP.CONF-H701] shall be filled-in and done verdict.

End proposed structure

________________