## H.22x Open Issue: Transport of several messages per PDU

Source: 	Editor of H.22x

Date:		February 2002

## The problem

The text in H.225.0 Annex G / H.22x regarding the sending of multiple messages in one transport PDU (and similar text in H.225.0 regarding the sending of H.245 messages) leaves room for interpretation. Clarification is needed to avoid interoperability problems.

## Excerpt from H.225.0 annex G (H.22x):

## 1.8(5.1)	Transport requirements

...

When messages are sent over the reliable transport service, multiple messages may be sent within the boundaries defined by the reliable transport protocol data unit (PDU) as long as whole messages are sent. (In IP implementations, as outlined in H.225.0 Appendix D (should be "IV" E.H.), this PDU is defined by TPKT.)

...

In UDP IP implementations, messages shall also be prefixed with TPKT headers, to enable multiple messages per packet. The UDP packet length field shall hold the total length of the payload, including all the messages and their TPKT headers.

## Excerpt from H.225.0:

## 6.1	General approach

...

When messages are sent on the reliable H.225.0 call signalling channel, only one whole message shall be sent within the boundaries defined by the reliable transport; there shall be no fragmentation of H.225.0 messages across transport PDUs. (In IP implementations as outlined in Appendix IV, this PDU is defined by TPKT.)

...

When messages are sent on the reliable H.245 control channel, more than one message may be sent within the boundaries defined by the reliable transport PDU as long as whole messages are sent; there shall be no fragmentation of H.245 messages across transport PDUs. (In IP implementations as outlined in Appendix IV, this PDU is defined by TPKT.)

## Possible interpretations

The following tries to identify some ways to interpret the text excerpts above.

a) Simple message concatenation

The transport PDU consists of a TPKT header containing the total length of the PDU, plus a payload of n concatenated octet fields, with each of them encoding a single message of the transported protocol as a PER complete encoding (PCE) according to X.691. There are no further header or length fields inside this payload (except if generated by the ASN.1 PER encoding process).

b) Embedded PDUs

The transport PDU consists of a TPKT header containing the total length of the PDU, plus a payload of n embedded "inner PDUs", where each inner PDU itself consists of a TPKT header containing the length of this inner PDU plus a single message of the transported protocol as a PER complete encoding (PCE) according to X.691.

c) PDU concatenation

This interpretation seems to make sense in the UDP case only since in the case of TCP it would mean only a single message per transport PDU, which is not what is said in the text.

The UDP datagram consists of the UDP header containing the total length of the datagram, plus a payload of n concatenated PDUs, where each PDU consists of a TPKT header containing the length of this PDU plus a single message of the transported protocol as a PER complete encoding (PCE) according to X.691.

d) Variant a) in the case of UDP

The UDP datagram contains the usual UDP header and one (or more) payload PDU(s) according to variant a).