Draft ITU-T Recommendation H.248.PCMSB

PCM Frame Spare Bit Package

Editor’s note: Text represents the merged output (from contributions AVD-2451 … 2455) of the Beijing WP2/16 meeting (May 2004). Some editorial enhancements are included.

# Summary

The H.248 Package provides support for Spare Bits Si and Sa4-Sa8, which are defined for the ITU-T G.704 2048 kbit/s Basic Frame Structure.

# Scope

This recommendation describes a relay mechanism of PCM frame spare bits, by using H.248 events and signals. The scope is limited on spare bits Si and Sa4-Sa8 of the 2048 kbit/s Basic Frame Structure [G.704]. These bits are typically designated for national and international use, specific point-to-point applications, etc. Further applications are for instance G.704 E1 frame based data link protocols, or spare bit based protocol procedures for access digital sections for ISDN primary rate at 2048 kbit/s [G.962].

This package is typically applicable for the termination referring a 2048 kbit/s Basic Frame Structure in Null Context.

NOTE – Time slot 0 (also known as channel 0) of a 2048 kbit/s frame represents generally a 64-kbit/s-channel. This bearer channel may principally be used for user plane traffic (“user channel”), or control plane traffic (“signaling channel”), or spare bit encoded information like G.704 basic frame recommendation. From H.248 perspective is this channel 0 representing an H.248 Termination of type “TDM”. Dependent on above mentioned use cases of channel 0, may the correspondent TDM Termination belong to the Null Context, or be associated with a “regular H.248 call Context”.

Editor’s note: Scope section will be revised in order to reflect comments from the experts meeting.

# References

[H.248.1]	ITU-T Recommendation H.248.1 Version 1 (03/2002), Gateway Control Protocol

[G.704]	ITU-T Recommendation G.704 (10/1998), Synchronous Frame Structures used at 1544, 6312, 2048, 8448 and 44 736 kbit/s Hierarchical Levels

[G.962]	ITU-T Recommendation G.962 (03/1993), Access Digital Section for ISDN Primary Rate at 2048 kbit/s

# Definitions

-

# Abbreviations

CW	Codeword

IC	Incoming

ID	Identifier

MF	Multiframe

MG 	Media Gateway

MGC 	Media Gateway Controller

OG	Outgoing

PCM	Pulse Code Modulation

SB, sb	Spare Bit

SMF	Sub-Multiframe

SSM	Synchronization Status Message [G.704]

Symbols (according ITU-T G.704):

Sa	Additional Spare Bit

Si	Spare Bit reserved for international use

# PCM Frame Spare Bit package

Editor’s note: IANA codepoint assignment is ongoing.

PackageID: pcmsb (0xnnnn)
Version: 1
Extends: none

This package defines H.248 methods for accessing Si-Bit and Sa4-Sa8 Bits of the 2048 kbit/s Basic Frame Structure according ITU-T G.704 [G.704]. Spare bit coding is defined in Annex A.

## Properties

None.

## Events

Table 1 is summarizing all considered events related to the ITU-T G.704 2048 kbit/s Basic Frame.

| Spare Bit (ITU-T G.704) | Event Identifier (EventID) |
| --- | --- |
|  | Spare Bit is z |
| 1 (Si) | "sb1_z" |
| 4 (Sa4) | "sb4_z" |
| 5 (Sa5) | "sb5_z" |
| 6 (Sa6) | "sb6_z" |
| 7 (Sa7) | "sb7_z" |
| 8 (Sa8) | "sb8_z" |

Table 1/H.248.PCMSB – List Event Identifiers

Event name: Spare bit SBy is detected as ‘z’ (see Table 1).

NOTE  SBy may be G.704 bit Si for y = 1, or Say for y = 4, 5, … 8.

EventID: sby_z, 0x00yz

Description: identifies Sa4 Bit with value z.

Value z represents a hexadecimal digit and is coded according Annex A. 
		Numerical codepoint is generally 0x00yz. ‘y’ corresponds to the spare bit, ‘z’ 
 		represents the value of the spare bit.

EventsDescriptor Parameters:

Strict Transition

ParameterID: strict (0x0001)

Type: enumeration

Default value: "state"

Possible values:	"exact"	(0x00),
 		"state"	(0x01), 
 		"failWrong"	(0x02)

"exact" means that only an actual state transition to is to be recognized;

"state" means that the event is to be recognized either if a state transition is detected or if the observed spare bit is already in this state;

"failWrong" means that if the state is already there, the command fails and an error is reported.

ObservedEventsDescriptor Parameters:

Initial State

ParameterID: trans (0x0002)

Type: Boolean

Default value: "True"

Possible values:

"False" means that the event was reported because the bit was already equal to requested value when the events descriptor containing this event was activated;

"True" means that the event represents an actual state transition.

The other events for the other spare bits are specified in exactly the same way. All event names and event identifiers are listed in Table 1.

## Signals

### Setting of Spare Bit SBx

This signal is setting a dedicated spare bit in time slot 0 of the G.704 2048 kbit/s Basic Frame structure. The basic structure for E1 is based on Sub-multiframes (SMF).

Signal Name: Spare bit SBy is set to ‘z’ (see Table 2).

NOTE  SBy may be G.704 bit Si for y = 1, or Say for y = 4, 5, … 8.

SignalID: sb4_z (0x014z)

Description: Sets Sa4 Bit to value z.

Value z represents a hexadecimal digit and is coded according Annex A. 
		Numerical codepoint is generally 0x01yz. ‘y’ corresponds to the spare bit, ‘z’ 
 		represents the value of the spare bit.

SignalType:

OO (On/Off)

Duration: –

Additional Parameters: none.

The other signals for the other spare bits are specified in exactly the same way. All signal names and signal identifiers are listed in Table 2.

| Spare Bit (ITU-T G.704) | Signal Identifier (SignalID) |
| --- | --- |
|  | Spare Bit is z |
| 1 (Si) | "sb1_z" |
| 4 (Sa4) | "sb4_z" |
| 5 (Sa5) | "sb5_z" |
| 6 (Sa6) | "sb6_z" |
| 7 (Sa7) | "sb7_z" |
| 8 (Sa8) | "sb8_z" |

Table 2/H.248.PCMSB – List of Signal Identifiers

## Statistics

None

## Procedures

### Signal Procedures

A spare bit signal must always be present on an H.248 TDM Termination belonging to a ITU-T G.704 E1 2048 kbit/s Basic Frame structure. Therefore, the setting of spare bit signals shall be considered to be state changes in the spare bit state, rather than as persistent signals in themselves. The state change shall be considered to be completed instantaneously by the MG. Consequently, there is no active signal to be terminated by any subsequent event detection or by a new signal descriptor..

The MG shall maintain the existing spare bit state on such a TDM Termination type until such time as the MGC sends a new spare bit signal to the MG to change state.

### Unexpected State of a Spare Bit

If the MGC sets an EventsDescriptor containing a Spare Bit transition event with the “strict” (0x0001) parameter set to “failWrong”, and the Spare Bit state is already what the transition implies, the execution of the command containing that EventsDescriptor fails. The MG SHALL include error code 541 “Unexpected initial Spare Bit state” in its response.

Editor’s note: proposed new error code 541 in contribution AVD-2453 (http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/AVD-2453.zip) is package specific, thus it’s not in scope of H.248.8. A text will be proposed in the next meeting for inclusion of the error code description in this package.

Editor’s note: see contribution AVD-2533

### Moving TDM Terminations from Null Context

H.248.PCMSB is used for “Channel 0” H.248 TDM Terminations.  These H.248 TDM Terminations may belong to the Null Context. In this case, any ADD command, which shifts a Channel 0 H.248 TDM termination to an H.248 call context, is not allowed. Those MGC commands, which request a termination to leave the Null Context, shall be responded by the MG with the error code '517 - Unsupported or invalid mode'.

Editor’s note: Error code 517 doesn’t match very well due to it’s scope on stream mode. A new error code will be proposed in next meeting.

Editor’s note: see contribution AVD-2533

Annex A

Spare Bit Coding

Editor’s note: see contribution AVD-2534

A.1	Sub-Multiframe based Codewords

ITU-T G.704 [G.704] is defining vertical codewords on a Sub-multiframe (SMF) basis on spare bits. A vertical codeword is a bit pattern of a single spare bit over multiple consecutive PCM frames. The four odd numbered frames in a G.704 E1 SMF are designated for spare bits. Consequently, a spare bit related codeword represents a 4-bit-pattern, also known as nibble.

NOTE 1 – A nibble is a group of four bits (G.804, G.8040/Y.1340).

NOTE 2 – The allocation of bits from a nibble pattern to the four frame positions within a Sub-multiframe is defined, for instance, by following two G.704 tables:
 	§2.3.3.2 Table 5B/G.704 – CRC-4 multiframe structure, or 
 	§2.3.4 Table 5C/G.704 – Synchronization Status Message (SSM) bit allocation for 2048 kbit/s.

- A.2	Hexadecimal Nibble Coding

A spare bit related nibble pattern must be coded with a single hexadecimal digit.

NOTE – All sixteen decimal values may represent valid codepoints. A codeword value is representing the state of the particular spare bit in section 6.2.

Most significance or least significance bit position is not relevant for that H.248 Package due to the Sub-Multiframe based application. Thus, transmission timing is not important. But in order to be consistence with H.248, the hexadecimal nibble coding shall be aligned with H.248.1 Annex B.3 hexadecimal octet coding.

Example:

An example for a specific codeword (CW), based on additional spare bit Sa4 is:

CWSMF,Sa4  = {Sa41, Sa42, Sa43, Sa44} for Sa4

| Nibble bit pattern | Hexadecimal coding |
| --- | --- |
| 1011 | D |

NOTE – Bit Sa44 is the MSB, bit Sa41 is the LSB.

Appendix I

2048 kbit/s Basic Frame of G.704 E1 Structure

Editor’s note: Appendix I is included for information purposes as long as H.248.PCMSB is a work item. May be removed in final Recommendation text.

Below excerpt from ITU-T Recommendation G.704 is providing the general time slot specification for the 2048 kbit/s Basic Frame Structure.

Table 5A/G.704 – Allocation of bits 1 to 8 of the frame

| Bit number Alternate frames | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame containing the frame alignment signal | Si | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | (Note 1) | Frame alignment signal |  |  |  |  |  |  |
| Frame not containing the frame alignment signal | Si | 1 | A | Sa4 | Sa5 | Sa6 | Sa7 | Sa8 |
|  | (Note 1) | (Note 2) | (Note 3) | (Note 4) |  |  |  |  |
| NOTE 1 – Si = Bits reserved for international use. One specific use is described in 2.3.3. Other possible uses may be defined at a later stage. If no use is realized, these bits should be fixed at 1 on digital paths crossing an international border. However, they may be used nationally if the digital path does not cross a border. NOTE 2 – The bit is fixed at 1 to assist in avoiding simulations of the frame alignment signal. NOTE 3 – A = Remote alarm indication. In undisturbed operation, set to 0; in alarm condition, set to 1. |  |  |  |  |  |  |  |  |

Table 5A/G.704 – Allocation of bits 1 to 8 of the frame (concluded)

NOTE 4 – Sa4 to Sa8 = Additional spare bits whose use may be as follows: i) Bits Sa4 to Sa8 may be recommended by ITU-T for use in specific point-to-point applications (e.g. transcoder equipments conforming to Recommendation G.761). ii) Bit Sa4 may be used as a message-based data link to be recommended by ITU-T for operations, maintenance and performance monitoring. If the data link is accessed at intermediate points with consequent alterations to the Sa4 bit, the CRC-4 bits must be updated so as to retain the correct end-to-end path termination functions associated with the CRC-4 procedure (see 2.3.3.5.4). The data-link protocol and messages are for further study. iii) Bits Sa5 to Sa7 are for national usage where there is no demand on them for specific point-to-point applications [see i) above]. iv) One of the bits Sa4 to Sa8 may be used in a synchronization interface to convey synchronization status messages, as described in 2.3.4. Bits Sa4 to Sa8 (where these are not used) should be set to 1 on links crossing an international border.

Appendix II

Example Signalling Flows

Editor’s note: Appendix II is included for information purposes as long as H.248.PCMSB is a work item. May be removed in final Recommendation text.

Four exemplary signaling flows are outlined:

- Registration with ‘Strict’ parameter and ‘Exact’ condition

- Registration with ‘Strict’ parameter and ‘State’ condition

- Registration with ‘Strict’ parameter and ‘FailWrong’ condition

- Setting of a Spare Bit nibble pattern

All H.248.PCMSB implementers must read the normative part of this Recommendation carefully before implementing from it. No one should use the examples in this appendix as stand-alone explanations of how to create protocol messages.

## II.1	Registration with ‘Strict’ Parameter and ‘Exact’ Condition

1)	MGC requests to subscribe for a spare bit event with parameter strict=exact

MGC Modifies the TDM Termination for event subscription:

MGC to MG:

MEGACO/1 [149.204.221.100]

Transaction = 15130 {

Context = 0 {

Modify = tdm/5/1/3/0 {

Events = 6903 {

pcmsb/sb5_0 {strict=exact}

}

}

}

}

2)	Response from MG

And the Modify is acknowledged.

MG to MGC:

!/1 [172.23.94.66] P=15130{C=-{MF=tdm/5/1/3/0}}

3)	MG notifies MGC the observed state transition

MG to MGC:

!/1 [172.23.94.66] T=1026{C=-{N=tdm/5/1/3/0{OE=6903{20700112T01305700 : pcmsb/sb5_0{trans=true}}}}}

4)	Response from MGC

And the Notify is acknowledged.

MGC to MG:

MEGACO/1

[149.204.221.100]

Reply = 1026

{

Context = - {

Notify=tdm/5/1/3/0

}

}

## II.2	Registration with ‘Strict’ Parameter and ‘State’ Condition

Precondition: spare bit Sa5 has the value ‘f’, or the nibble pattern for an E1 Sub-Multiframe corresponds to CWSMF,Sa5  = {Sa51, Sa52, Sa53, Sa54} = {1, 1, 1, 1}.

1)	MGC requests to subscribe for a spare bit event with parameter strict=state

MGC to MG:

MEGACO/1 [149.204.221.100]

Transaction = 1868 {

Context = 0 {

Modify = tdm/5/1/3/0 {

Events = 5868 {

pcmsb/sb5_f {strict=state}

}

}

}

}

2)	Response from MG

MG to MGC:

!/1 [172.23.94.66] P=1868{C=-{MF=tdm/5/1/3/0}}

3)	MG notifies MGC, that the state is already there

MG to MGC:

!/1 [172.23.94.66] T=1027{C=-{N=tdm/5/1/3/0{OE=5868{20700112T01442300 : pcmsb/sb5_f{trans=false}}}}}

4)	Response from MGC

MGC to MG:

MEGACO/1

[149.204.221.100]

Reply = 1027

{

Context = - {

Notify=tdm/5/1/3/0

}

}

## II.3	Registration with ‘Strict’ Parameter and ‘FailWrong’ Condition

Precondition: spare bit Sa5 has the value ‘f’, or the nibble pattern for an E1 Sub-Multiframe corresponds to CWSMF,Sa5  = {Sa51, Sa52, Sa53, Sa54} = {1, 1, 1, 1}.

1)	MGC requests to subscribe for a spare bit event with parameter strict=failWrong

MGC to MG:

MEGACO/1 [149.204.221.100]

Transaction = 3673 {

Context = 0 {

Modify = tdm/5/1/3/0 {

Events = 7470 {

pcmsb/sb5_f {strict=failWrong}

}

}

}

}

2)	MG sends response with error descriptor, because the state is already there

MG to MGC:

!/1 [172.23.94.66] P=3673{C=-{MF=tdm/5/1/3/0{ER=541{"Unexpected initial spare bit state"}}}}

## II.4	Setting of a Spare Bit Nibble Pattern

1)	MGC requests to set a specific nibble pattern for a dedicated spare bit

MGC to MG:

MEGACO/1 [149.204.221.100]

Transaction = 10033 {

Context = 0 {

Modify = tdm/5/1/3/0 {

Signals {

pcmsb/sb5_f

}

}

}

}

2)	Response from MG

MG to MGC:

!/1 [172.23.94.66] P=10033{C=-{MF=tdm/5/1/3/0}}

# Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.PCMSB Ed. 0.1 | Initial version, ITU-T SG16 Plenary Meeting, January 2004 Location: http://www.itu.int/md/meetingdoc.asp?type=mitems&lang=e&parent=T01-SG16-040120-D-0396 |
| H.248.PCMSB Ed. 0.2 | Input version to WP2/16 Experts Meeting, Beijing, May 2004 Editorial enhancements with respect to Ed. 0.1 Location: http://ftp3.itu.int/av-arch/avc-site/0405_Bei/AVD-2452.zip |
| H.248.PCMSB Ed. 0.3 | Output version to WP2/16 Experts Meeting, Beijing, May 2004 Location: http://ftp3.itu.int/av-arch/avc-site/0405_Bei/TD-43.zip |
| H.248.PCMSB Ed. 0.4 | Input version to WP2/16 Experts Meeting, San Jose, Aug/Sep 2004 Editorial enhancements with respect to Ed. 0.3 Location: http://ftp3.itu.int/av-arch/avc-site/0408_San/AVD-2530.zip |
| H.248.PCMSB Ed. 0.5 | Output version to WP2/16 Experts Meeting, San Jose, Aug/Sep 2004 Location: |

____________________

e.g., ITU-T G.704: MFE1 = SMF I + SMF II