ITU Telecommunication Standardization Sector	Document AVD-2532

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question(s):	3/16

Source*:	ALCATEL

Title:		Revised Text for H.248.PCMSB clause 2 “Scope”

Purpose:	Discussion

___________________

# Introduction

The proposed H.248 package is objecting transmission layer information. The original scope section was written from H.248 standpoint, but did miss the transmission network view. The text did raise a lot of valid questions and generated some confusion. Since the past meeting Alcatel received further feedback. The proposed improvements are accounted in this contribution.

# Proposal

The original scope of the H.248.PCMSB Package is unchanged, but a complete revised text with a new figure is proposed, in order to clarify the environment of this package.

[Begin Correction]

2	Scope

This package defines H.248 methods to allow the following functions for the Si and Sa4-Sa8 bits of the 2048 kbit/s Basic Frame Structure defined in ITU-T Recommendation G.704:

1. MGC instructs the MG to monitor and report changes in received spare bit states

2. MG reports changes in received spare bits to the MGC

3. MGC instructs the MG to change the state of spare bits being sent

In order to support various applications in an efficient manner, the coding allows changes to be specified either for individual bits or for the 4-bit codewords that occupy a sub-multiframe.

## Background from Transmission Perspective

ITU-T G.704 [G.704] specifies frame structures for synchronous transmission interfaces. This H.248 Package is exclusively scoping the 2048 kbit/s Basic Frame Structure according ITU-T G.704, section 2.3. This hierarchy level is traditionally called E1 in ITU-T, or P12 in revised ITU-T G.705 (10/2000).

Every G.704 frame defines in structured mode (see below) a number of 64 kbit/s channel time slots (see ITU-T G.704, chapter 5). These frame structures were originally developed for the digitization of the transmission infrastructure in telecommunication networks. Fixed telephone networks were the very first scope in that time, mobile networks didn’t play any significant role.

Thus, the prime scope were PCM signals, which is still reflected in G.704: Each 64 kbit/s channel time slot can accommodate, for example, a PCM-encoded voiceband signal conforming to G.711 or data information with a bit rate up to 64 kbit/s.

The term “PCM system” come in use due to that background, and is still common vocabulary, particularly for E1 usage in circuit-switched telephone networks.

## PCM Systems for E1

An E1 frame consists of 32 code words called time slots, and are numbered from 0 to 31. This general structure is often also known as PCM32 system. The abbreviation “PCM” points out that in telephone networks the dominating application of the individual E1 time slots is the PCM-based voice codec G.711. The number “3x” denotes how many channels of the G.704 frame are available for user traffic, i.e., typically PCM encoded traffic. A PCM31 comprises 31 time slots for user traffic (referred as channel 1 to 31), and one time slot for frame synchronization (= E1 TS0, channel 0). A PCM30 comprises 30 time slots for user traffic (referred as channel 1 to 15, and 17 to 31), one time slot for frame synchronization (channel 0), and one time slot for signaling traffic (channel 16).

The H.248.PCMSB package is scoping PCM30 and PCM31 systems, but not PCM32 interfaces (Figure 1).

## E1 Operation Modes

A PCM system corresponds to specific operation modes of a G.704 interface. E1 interfaces may be fundamentally operated either in unstructured mode or in structured mode. Unstructured mode is typically used for cell-, frame-, or packet-switched traffic (e.g., ATM-over-E1 according ITU-T G.804), and is out of scope of this H.248 Package. The structured mode is intended for circuit-switched traffic, and may be further subdivided in modes with or without multiframing. Both are in scope of H.248.PCMSB, but with a slightly different focus due to the different usage of spare bits in these modes.

### E1 Multiframes and Sub-Multiframes (SMF)

In case of structured H.248 MG E1 interfaces operated in multiframing mode, spare bits carried in the four odd numbered frames of the E1 SMF are scoped. The four even numbered frames are out of scope of package H.248.PCMSB. Thus, any G.704 CRC procedures are not considered, too.

### E1 Structure without Multiframing:

In case of structured H.248 MG E1 interfaces without multiframing, Annex A coding is simplified. In this mode there no SMF structure exists, spare bit Signals/Events degenerate to binary code words (hexadecimal character ‘0’ or ‘F’). Whereas Sa spare bits are carried only in the odd numbered frames, Si bit is carried in every frame.

## H.248 Package

This package defines H.248 methods for accessing Si-Bit and Sa4-Sa8 Bits of such a E1 frame, the 2048 kbit/s Basic Frame Structure according ITU-T G.704 [G.704]. These Spare Bits belonging to channel 0 in structured E1 frames. Other channel 0 bits or protocol control information is out of scope of this package.

This time-slot (channel 0) can not be used in such E1 operation modes as a bearer channel for user plane traffic and must remain in the Null Context. In some applications, it may be possible for a signaling channel to be defined using one or more of the spare bits of this time-slot as a data link, but such operation is outside the scope of this Recommendation.

From the H.248 perspective, this channel represents an H.248 Termination of type "TDM", however, it can not leave the Null Context or be associated with a "regular H.248 call Context".

Figure 1/H.248.PCMSB – Scope of this Package

…

[End Correction]

___________________

E1 frames carrying 64 kbit/s channels are commonly termed as “channelized E1”.

E1 frame: odd numbered frames do carry CRC-4 multi-framing information, which is a prerequisite to define Sa Code words per SMF, thus at least the usage of Si bit Signals are forbidden in this mode (setting of Si would destroy multiframing detection capability of the receiving side).

These bits are typically designated for national and international use, specific point-to-point applications, etc. Further applications are for instance G.704 E1 frame based data link protocols, or spare bit based protocol procedures for access digital sections for ISDN primary rate at 2048 kbit/s [G.962].