ITU Telecommunication Standardization Sector	Document AVD-2551

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question(s):	3/16

Source*:	ALCATEL

Title:		Comments to draft H.248 Package proposal for Call Information Recording

Purpose:	Discussion

___________________

Overview

1	Introduction	1

2	References	1

3	Comments	2

3.1	Package Scope	2

3.2	Charging Concept	2

3.3	Terminology	2

3.3.1	Call		2

3.3.2	CDR		3

3.4	Temporary Loss of MGC-MG Interconnection	3

4	Conclusion	3

# Introduction

This contribution addresses some questions to the package proposal (presented at the Beijing meeting in May) with regard to “call information recording” [1].

# References

- H.248 Package for Call Information Recording. ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2478, May 2004.
http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/AVD-2478.zip

# Comments

## Package Scope

It seems to be the case that the basic idea is that

- the MG shall be used as an (additional) backup system for the MGC, in order to face potential data lost during MGC failure periods, when for instance the primary MGC is switching over to the secondary MGC,

- the objected data is related to the generation of CDR (Call Data Record, E.422),

- thus the MG shall act as an additional protecting entity in order to improve the trustiness of charging, accounting, and/or billing process.

Meaning, in case of corrupted or lost CDR data in the primary MGC and secondary MGC, the MG is a tertiary instance for saving the most important CDR data. Of course, data scope is related to this MG only.

Question: Is this the intended scope by the authors of [1]?

## Charging Concept

The package defines time-related parameters only. Drawing the conclusion that time-based charging only shall be tackled by this H.248 package. It is common understanding that time-based charging is under full responsibility at the MGC level (e.g., by following charging principles for ISDN (ITU-T D.230) or SS7 (ITU-T D.212)).

Assuming that volume-based charging is used instead, or in addition to time-based charging, the picture may getting different because the H.248 MG would (probably) have the responsibility to ensure consistent data records for carried traffic volume. An example network scenario might be a packet-to-packet call, whereby the ephemeral H.248 Termination type doesn’t provide sufficient statistics for volume-based charging.

Question: May the package scope limited to time-based charging only?

## Terminology

### Call

The term call may mislead because call and bearer are separated in the H.248 model. E.g., the H.248 MG responsible for bearer connections behind calls.

Questions:

- Is “call” related to H.248.1 Connection Model?

- Might be “call” replaced by “Context”?

- Or might be “call” related with a specific H.248 Termination of a “call leg”?

- Or is “call” related to a call on call control level respectively to a session on session control level, i.e., beyond the scope of the MGC?

### CDR

Presuming that “call information recording” is clearly related to call data recording, it might be useful to use CDR term or to provide a reference. In order to be consistent concerning terminology, SANCHO delivered following results for CDR:

CDR	Call Data Record (E.422)
Call Detail Record (E.491, others)
Connection Detail Record (D.224, I.377)
Charging Data Record (Y.1741)

Question: If CDR shall be introduced, which abbreviation is primarily intended by the package authors?

## Temporary Loss of MGC-MG Interconnection

Question: Is this the solely concerning and motivation for your package proposal?

# Conclusion

Current scope of package proposal [1] seems to be rather broad. If the intended application is very specific, then it is proposed to provide more details in a scope clause.

___________________

Q.825: The set of functions needed to determine the price assigned to the service utilization.

Q.825: Set of functions required for Usage Metering, Charging and Billing.

Q.825: Administrative function to prepare bills to service customers, to prompt payments, to obtain revenues and to take care of customer reclaims.

H.248.1 cl. 3.3: “… could terminate bearer channels from a switched circuit network (e.g. DS0s) and media streams from a packet network (e.g. RTP streams in an IP network)”

http://www.itu.int/sancho/