ITU Telecommunication Standardization Sector	Document AVD-2534

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question:	3/16

Source*:	ALCATEL

Title:		H.248.PCMSB – Revised Spare Bit Coding

Purpose:	Discussion

___________________

# Introduction

H.248 Spare Bit coding is described in Annex A of H.248.PCMSB Package [1]. Annex A was introduced in the past May 2004 Rapporteurs meeting, and mainly motivated by the “codeword” based background of spare bits in G.704 multiframe structures.

The proposed scheme in [1] Annex A.2 was derived from H.248.1 Annex B.3. Some questions pop up after the May meeting.

This contribution proposes a revision of [1] Annex A.2.

# References

- PCM Frame Spare Bit Package (H.248.PCMSB) Edition 3. ITU-T WP2/16 Rapporteurs meeting, Document: TD-43, May 2004.	
http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/TD-43.zip

- Signalling Codepoints based on Codewords for H.248.PCMSB. ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2454, May 2004.	
http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/AVD-2454.zip

# Discussion

There are following questions and objections:

- H.248.1 Annex B is for text encoding, Annex B.3 describes the octet representation by hexadecimal digits

- H.248.1 Annex B.3 is for octets, H.248.PCMSB is objecting nibbles

- spare bits in a nibble codeword are unweighted, i.e., bit significance with respect to MSB/LSB is not relevant

Alcatel concludes that there’s no basic requirement for alignment of coding schemes in H.248.PCMSB Annex A.2 with H.248.1 Annex B.3.

Following revised text is proposed:

[Begin Correction]

Annex A

Spare Bit Coding

There are two structured E1 operation modes in scope of this package. The coding scheme for structured E1 with multiframing is defined in annex A.1. The coding scheme for structured E1 without multiframing is defined in annex A.2. An aligned coding scheme at the H.248 interface is defined.

A.1 Sub-Multiframe based Codeword Coding

G.704 defines vertical codewords using the spare bits on a Sub-multiframe (SMF) basis. A codeword is a 4-bit pattern formed from the four consecutive occurrences of the same spare bit within a single SMF.

The codeword shall be coded within events and signals as a single hexadecimal character, in which the most significant bit of that hexadecimal character shall be the bit in the first occurrence of the spare bit within the SMF, that is, in frame 1 or 9.

Note - The allocation of codewords is defined, for example, in Table 2.3.4/G.704, Synchronization Status Message (SSM) bit allocation for 2048 kbit/s.

Example:

An example for a specific codeword (CW), based on additional spare bit Sa4 is:

CWSMF,Sa4  = {Sa41, Sa42, Sa43, Sa44} for Sa4

| 4-bit pattern | Hexadecimal character |
| --- | --- |
| 1011 | B |

A.2 Single Bit Coding

There are no vertical codewords in mode “structured E1 without multiframing”. The 4-bit pattern degenerating to 1-bit codepoints. The single bit codepoint shall be coded with the same coding scheme as in annex A.1. The code set contains thus only two hexadecimal characters. The hexadecimal characters ‘0’ and ‘F’ shall be used for the spare bit coding of ‘0’ and ‘1’.

[End Correction]

# Conclusion

Correspondingly modified text is proposed by this contribution.

___________________