ITU Telecommunication Standardization Sector	Document AVD-2579

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question(s):	3/16

Source*:	ALCATEL

Title:		Comments to Draft H.248.STIMAL – Crosscheck from V5 Perspective

Purpose:	Discussion

___________________

# Introduction

Liasion [1] includes a new package proposal H.248.STIMAL [2]. This contribution provides initial comments from checking the H.248 package from V5 perspective.

# References

- Definition of a new Stimal Package. Liasion from ETSI TISPAN WG3, Document: AVD-2579, August/September 2004.	
http://ftp3.itu.int/av-arch/avc-site/0408_San/AVD-2579.zip

- Draft Proposal ITU-T H.248.STIMAL: Stimulus Analogue Lines Packages. Meeting ETSI TISPAN#3bis, Document: 03bTD77r1, July 2004.

- V Interfaces at the Digital Local Exchange (LE); V5.1 Interface for the Support of Access Network (AN); Part 1: V5.1 Interface Specification. ETSI EN 300 324-1 V2.1.1 (2000-04)

- V Interfaces at the Digital Local Exchange (LE); V5.2 Interface for the Support of Access Network (AN); Part 1: V5.2 Interface Specification. ETSI EN 300 347-1 V2.2.2 (1999-12)

# Comments

## Additional References

It is proposed to include latest V5.1 reference in the reference list: ETSI EN 300 324-1 [3].

## V5 Tables

Another improvement could be to reuse the tables of the PSTN protocol messages (EN 300 324-1, Table 16) and of the Information element identifier coding (EN 300 324-1, Table 17).

With the first table an overview of the mapping of messages to H.248 commands could be given. With the second table it can be indicated which information elements are implemented as events and/or signals in the stimulus analogue line package, which are not required to be supported for H.248 and where are corresponding mappings in H.248 used.

EN 300 324-1, Table 16: PSTN protocol message types

| Message type | Reference (clause) | Mapped |
| --- | --- | --- |
| Path establishment messages |  |  |
| ESTABLISH | 13.3.1 | H.248 (e.g. Notify, Modify) |
| ESTABLISH ACK | 13.3.2 | H.248 (Transaction Reply, Notify, Modify) |
| SIGNAL | 13.3.3 | H.248 (e.g. Notify, Modify) |
| SIGNAL ACK | 13.3.4 | H.248 (Transaction Reply) |
| Path clearing messages |  |  |
| DISCONNECT | 13.3.7 | “call finished” in stimal package |
| DISCONNECT COMPLETE | 13.3.8 | H.248 (Transaction Reply) |
| Other messages |  |  |
| STATUS ENQUIRY | 13.3.6 | H.248 Audit Value, Audit Descriptor |
| STATUS | 13.3.5 | No mapping. |
| PROTOCOL PARAMETER | 13.3.9 | H.248 (e.g. Modify) |
| NOTE: All other values of PSTN protocol message types are reserved. |  |  |

EN 300 324-1, Table 17: Information element identifier coding

| Name | Reference | Implemented as Event in Stimulus Lines Analogue Package | Implemented as Signal in Stimulus Lines Analogue Package | Mapped | Not required |
| --- | --- | --- | --- | --- | --- |
| SINGLE OCTET |  |  |  |  |  |
| Pulse-notification | 13.4.6.1 1 |  |  | X |  |
| Line-information | 13.4.6.2 1 | X |  |  |  |
| State | 13.4.6.3 1 |  |  |  | X |
| Autonomous-signalling-sequence | 13.4.6.4 1 | X | X |  |  |
| Sequence-response | 13.4.6.5 1 |  |  | X |  |
| VARIABLE LENGTH |  |  |  |  |  |
| Sequence-number | 13.4.7.1 3 |  |  |  | X |
| Cadenced-ringing | 13.4.7.2 3 |  |  | X (15 patterns) |  |
| Pulsed-signal | 13.4.7.3 3 to 5 | X | X |  |  |
| Steady-signal | 13.4.7.4 3 | X | X |  |  |
| Digit-signal | 13.4.7.5 3 |  | X |  |  |
| Recognition-time | 13.4.7.6 4 |  |  | X |  |
| Enable-autonomous-acknowledge | 13.4.7.7 4 to 6 |  |  | X |  |
| Disable-autonomous-acknowledge | 13.4.7.8 3 |  |  | X |  |
| Cause | 13.4.7.9 3to5 |  |  |  | X |
| Resource-unavailable | 13.4.7.10 3 to 8 |  |  | X |  |
| Enable-metering | 13.4.7.11 4 to 7 |  |  | X |  |
| Metering-report | 13.4.7.12 4 to 5 |  |  | X |  |
| Attenuation | 13.4.7.13 3 |  |  | X |  |
| NOTE: All other values are reserved. |  |  |  |  |  |

Alcatel proposes to include these tables in an Appendix, to facilitate the understanding with respect to replacement of V5 Access Nodes by H.248 MGs.

___________________