ITU Telecommunication Standardization Sector	Document AVD-2552

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question:	3/16

Source*:	ALCATEL

Title:	H.248.1 Version 3 – Proposal for Further Definitions

Purpose:	Discussion

___________________

# Introduction

H.248.1 Version 1 and Version 2 are dated on March 2002 respectively May 2002. Since that time H.248 work did progress and the number of H.248 Packages did increase. Some new Packages requiring a terminology which is not detailed enough contained in V1 and V2.

This contributions proposes the inclusion of some additional definitions in H.248.1 V3. The proposal is based on past contribution AVD-2458 [1].

- H.248 Media Gateways – Interworking with CAS-based Signalling Systems; Part I: Survey of Call Control Signalling Systems Types. ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2458, May 2004.	
http://ftp3.itu.int/av-arch/avc-site/0405_Bei/AVD-2458.zip

# Proposal

Following additional definitions and abbreviations are proposed.

[Begin Correction]

3.xx	Analog Line: This is a CAS interface type whereby an H.248 MG is peering an analog terminal equipment (“POTS terminal”).

3.xx	CAS Trunk: A ‘trunk’ with CAS. This CAS interface type is between a H.248 MG and a peer  public or private exchange (also known as STM switches).

…

[End Correction]

Additional abbreviations:

ALN	Analog Line

ALS	Analog Line Signalling

POTS	Plain Old Telephone Service

STM	Synchronous Transfer Mode

___________________