ITU Telecommunication Standardization Sector	Document AVD-2543

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question:	3/16

Source*:	ALCATEL

Title:	System (In)activity Checking – 	
Peer-to-Peer Operation State Monitoring between H.248 MGC-MG Pairs

Purpose:	Discussion

___________________

The capability of “Peer-to-Peer Operation State Monitoring” was frequently discussed in the past under various subjects like polling, keep alive, heartbeat, interconnection supervision, etc. within the MEGACOP/H.248 community. The last Rapporteurs meeting in Beijing did stimulate again this discussion by contribution [5]. Whereas the Inactivity Timer Package H.248.14 clarifies a “MGC polling by MG” mechanism, the opposite direction isn’t explicitly explained in any H.248 document. This seems to be one cause for requests concerning that capability. Another reason might be that signaling transport capabilities are misleadingly mixed with H.248 mechanisms.

This contribution tries to reflect again the H.248 history on that subject, summarizes the various capabilities, and tries to conclude an implementation guideline.

The text may be furthermore a baseline for an H-series Supplement document, in case that the majority feels that such a document may complement and help to understand that H.248 topic.

System (In)activity Checking 
–
Peer-to-Peer Operation State Monitoring between H.248 MGC-MG Pairs

1	Introduction	3

1.1	Purpose	4

1.2	Scope	4

2	References	4

2.1	H.248 Recomendations	4

2.2	H.248 Discussions on Polling, Keep Alive, Heartbeat, Inactivity etc.	4

3	Requirements	5

3.1	Fast Detection of (Un)reachability of Peer H.248 Node(s)	5

3.2	Fast Detection of (Un)availability of Peer H.248 Node(s)	5

4	Conventional Solution	5

5	Solutions supported by existing H.248 Mechanisms	6

5.1	Overview	6

5.2	H.248 Signalling Transport	7

5.2.1	H.248-over-UDP/IP	7

5.2.2	H.248-over-TCP/IP	8

5.2.3	H.248-over-SCTP/IP	8

5.2.4	H.248-over-AAL5/ATM	8

5.2.4.1	H.248-over- SAAL UNI	9

5.2.4.2	H.248-over-SAAL NNI	9

5.2.5	H.248-over-IP-over-ATM	9

5.2.6	Summary for Signalling Transport	10

5.3	H.248 Control Protocol	10

5.3.1	Polling Model	10

5.3.2	Polling of MG by MGC	10

5.3.3	Polling of MGC by MG	11

5.3.3.1	History	11

5.3.3.2	Inactivity Timer – Package H.248.14	11

5.3.4	Summary for H.248 Protocol	12

5.4	H.248 Applications	12

6	Conclusions	12

7	Appendix: History, Proposals and Discussions	13

# Introduction

H.248 nodes may be functionally partitioned in signaling transport and application functions besides the main control protocol (see Figure 1).

Figure 1/H.248 – Master Slave Model

H.248, as every other signaling system with equivalent availability and real-time objectives, has to support or to use certain basic mechanisms to tackle the requirements behind these targets. Fundamental requirements are for instance

- assured or reliable transport of signaling traffic,

- real-time transport of signaling traffic, or

- continuous monitoring the reachability and availability of the peer signaling partner.

Many call control and bearer control signaling systems are fulfilling these capabilities by using services of the underlying signaling transport protocol(s). For H.248 is the situation slightly different, because H.248 (and some other new control protocols) is designed in a “bearer independent way”, meaning that there are multiple possibilities for transport of H.248 signalling traffic (like H.248 over UDP, TCP, SCTP, or ATM).

There are basically two categories concerning these H.248 signalling transport stacks:

- 1st category: a protocol layering which is supporting above requirements, and a

- 2nd category: not providing these services to the H.248 gateway control protocol.

The 2nd category leaded to the need in defining certain mechanisms on H.248 gateway control protocol level, or alternatively, in the H.248 “served user entities” (shall be called H.248 application in this document, see Figure 1).

## Purpose

The document tries to summarize present situation and gives a survey on the “complete H.248 picture”.

## Scope

Reliable transport is a key feature of the first category signaling transport protocols. These mechanisms shall be briefly recalled again in this document. Scope of this contribution shall be the “continuous mutual monitoring” mechanism between two H.248 systems on H.248 gateway control protocol level. Common terminology is also “polling”, “heartbeat”, “keep alive”, or “watchdog” functionality.

The scope of this document is system (in)activity of H.248 nodes. Media inactivity is not addressed here (see [6]).

# References

## H.248 Recommendations

- ITU-T H.248.14, Inactivity Timer Package. March 2002.

## H.248 Discussions on Polling, Keep Alive, Heartbeat, Inactivity etc.

A selection of major contributions in chronological order.

- Draft of Proposed H.248 Annex M_KA: KeepAlive Package for H.248. ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2129, October 2001.	
http://ftp3.itu.int/av-arch/avc-site/0110_Dub/AVD-2129.zip

- H.248 Annex M.KA: KeepAlive Package. ITU-T SG16, Document: T01-SG16-020205-TD-WP2-0014, February 2002.	
http://ties.itu.int/u/tsg16/sg16/td/Geneva-5-02-2002/wp2/2-014.doc 
MEGACO discussion :	
http://www1.ietf.org/mail-archive/web/megaco/current/msg01579.html

- H.248 Annex M.6: Inactivity Timer Package. ITU-T SG16, Document: T01-SG16-020205-TD-WP2-0050, February 2002.	
http://ties.itu.int/u/tsg16/sg16/td/Geneva-5-02-2002/wp2/2-050.doc

- The dual heartbeat for MGC and MG to inspect the status of peer. ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2481, May 2004.	
http://ftp3.itu.int/av-arch/avc-site/0405_Bei/AVD-2481.zip

- Media (In)activity Checking – Monitoring Media Traffic in H.248 Media Gateways. ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2544, August/September 2004.	
http://ftp3.itu.int/av-arch/avc-site/0408_San/AVD-2544.zip

# Requirements

## Fast Detection of (Un)reachability of Peer H.248 Node(s)

There are strict reliability requirements for signaling networks (e.g., SS7). The availability of any signaling relation has to be at least in the range of 0.99999. This objective may be mapped on following statement: peer H.248 systems requiring a “high available communication link” for the H.248 signalling traffic. This requirement is related to network failures (in transmission, switching, or router equipment) between the two H.248 nodes.

This requirement is fulfilled in telephone signaling systems by continuously monitoring the availability of the “signaling paths”, e.g., by keep alive mechanisms.

## Fast Detection of (Un)availability of Peer H.248 Node(s)

An H.248 system is available when it is in a “normal operation state”. High service availability and network availability objectives requiring the fast detection of unavailable network elements. This is basically supported by pro-actively checking the network element availability.

# Conventional Solution

Following basic rule is traditionally applied in telephone signaling systems: in case that the signaling link is in the “available state”, then the signaling link termination node is considered to be “not unavailable”.

This simplified availability view is illustrated in Figure 2. The signalling links are terminated in the H.248 systems. A first-order approximation is to conclude “system availability” (blue dashed line) in case “availability of signaling transport service” situations. This approximation is temporarily sufficient for the scoped short monitoring periods.

Figure 2/H.248 – Simplified Availability Model

Of course, this conclusion is only an approximation, theoretically not fully correct, but practically often already sufficient and successfully applied in many systems in the past. Particularly, also due to the fact that the overall system availability framework is complemented by a set of other mechanisms, which are not necessarily related to the signaling feature.

Note: In small capacity systems are the “signaling transport service” and the “signaling protocol” often realized on the same processing element. In high capacity systems are these functions typically distributed on multiple processing elements, but often tightly coupled in the sense, that internal unavailability of individual processing elements is internally very fast communicated. In both cases is above rule considered to be a good approximation.

The conclusion is, both requirements of chapter 3 are considered to be fulfilled, in case of an existing capability for “continuously monitoring” the availability of the signaling transport service. This monitoring function is typically realized by distinguishing the load state of the “signaling transport path”:

- normal/high signaling load: 
 acknowledgement procedures and protocol timers are sufficient

- prolonged absence of signaling load: 
 a keep alive mechanism is running in idle phases, objective is continuously probing the reachability of the peer signaling partner.

Prominent representatives are for instance DSS1 layer 2 signalling (= ITU-T Q.921: LAPD), or SS7 level 2 signalling (= ITU-T Q.701-705: MTP2).

# Solutions supported by existing H.248 Mechanisms

## Overview

Figure 3 tries to summarize all present H.248 signalling transport alternatives. The figure outlines the asymmetrical nature of H.248 signalling, originating in the master-slave principle. This fact shall be indicated by the different types of H.248 applications: e.g., call control in MGC, and bearer control in MG. This asymmetry is not relaxing any of the requirements listed in chapter 3 for one side. E.g., less stringent requirements for MG in comparison to MGC.

Note: For instance in case of an “Emergency Standalone Application” (ESA) as an H.248 application in H.248 MGs. Such type of MGs requiring the capability for reachability and operation state monitoring of all associated primary and, in case of failover switching, secondary MGCs.

But the asymmetry is existent in certain H.248 mechanisms, - H.248 is per se a master-slave control asscociation -, whereas almost all H.248 signalling transport protocols are symmetrical, because either they are of type “end-to-end” (IETF UDP, TCP, SCTP), or they are of type “hop-to-hop” (ITU-T NNI MTP3B).

Figure 3/H.248 – Overview

## H.248 Signalling Transport

H.248 is architected in a transport independent way. The various options are discussed in this section with scope on the “reachability requirement” (see section 3.1).

### H.248-over-UDP/IP

UDP/IP per se doesn’t provide any support for reliable transport. That was one of the reasons in incorporating recommendations for Application Level Framing (ALF) procedures in H.248. ALF proposals for UDP/IP transport are covered in H.248.1 Annex D.1.

But ALF mechanisms for UDP are intended to improve transport reliability, they are not primarily objecting the scoped requirements of this document.

### H.248-over-TCP/IP

TCP transport is described in H.248.1 Annex D.2, but there’s the same conclusion as for UDP.

### H.248-over-SCTP/IP

SCTP transport is described in H.248.4. SCTP is basically scoping for transport of signaling traffic. SCTP (RFC 2960) is thus explicitly supporting such functions, called heartbeat.

Figure 4/H.248 – H.248 Signalling Transport over SCTP/IP

### H.248-over-AAL5/ATM

AAL5 is used in SAAL operation mode (Figure 5).

Figure 5/H.248 – H.248 Signalling Transport over ATM

#### H.248-over- SAAL UNI

The ITU-T Q.2110 Service Specific Connection Oriented Protocol (SSCOP) is designed for connection maintenance in the prolonged absence of data transfer. The mechanism is called keep alive, and configurable by a dedicated timer. This function verifies that the two peer SSCOP entities participating in a connection are remaining in a link connection established state even in the case of a prolonged absence of data transfer.

#### H.248-over-SAAL NNI

For narrowband SS7 signalling is MTP-2 providing a keep alive mechanism, based on FISU messages (Fill-In Service Unit). MTP-2 is replaced at broadband SS7 interfaces with MTP-3B by the SSCOP protocol. Thus, there’s the same keep alive procedure at SAAL UNI and NNI. From performance perspective is typically the value of Timer KEEP ALIVE different, e.g., 2 s at UNI, and 100 ms at NNI.

### H.248-over-IP-over-ATM

This is a scenario with AAL5 ATM connection(s) between MGC and MGs. But AAL5 is not operated in SAAL mode, rather in basic AAL5 data mode. Thus, it is up to the IP transport protocol whether an assured service for H.248 signalling is available or not.

### Summary for Signalling Transport

There are two categories of transport stacks for H.248 traffic with respect to a “keep alive capability”:

- With support: H.248 with H.248.4 (SCTP) or H.248.5 (ATM) transport

- Without: H.248 with H.248.1 Annex D.1 (UDP) or H.248.1 Annex D.2 (TCP) transport

The document doesn’t consider other evaluation criteria for the appropriateness of a specific H.248 transport option. But the conclusion here is that the first category technologies doesn’t impose any additional protocol mechanisms, above signaling transport layers, in order to ensure chapter 3 requirements.

## H.248 Control Protocol

This section outlines the mechanisms provided by the H.248 protocol itself.

### Polling Model

“Keep alive” mechanisms are principally polling procedures. Polling is generally a request-response scenario, which is periodically initiated. The “polling instance” (MG or MGC) is sending a “request message”, the “polled instance” (MGC or MG) is responding. The polling frequency is controlled by a protocol timer. Figure 6 shows a generic polling model.

Figure 6/H.248 – Generic H.248 Polling Model

### Polling of MG by MGC

There isn’t any package implementation necessary. In case that the MGC wants to poll the MG, then existing H.248 commands may be used. For example, an AuditValue command with an empty AuditDescriptor [1].

Figure 7/H.248 – MG Polling by MGC

Figure 7 outlines that this mechanism doesn’t require any specific implementation in the MG, and some kind of “polling functionality” in the MGC. This approach ensures that there aren’t any interoperability issues.

### Polling of MGC by MG

H.248.14 is the standardized mechanism for this “polling” direction.

#### History

The Inactivity Timer Package origins in discussions on the MEGACO mailing list in May 2001. A brief summary of the discussion is appended in chapter 7.

#### Inactivity Timer – Package H.248.14

The “polling request” is based on an H.248 event (see Figure 8). The complete mechanism is compelled and controlled by the MGC: the MGC may activate and deactivate the H.248.14 mechanism, and may control the “polling frequency” by setting timer mit. That’s why the package name was changed from “polling package” via “keep alive package” in “inactivity timer package”.

Figure 8/H.248 – MGC Polling by MG (H.248.14 Procedure)

H.248.14 is scoping “system inactivity” and not any kind of “media inactivity”. Due to that background the package is related to ROOT termination only, and may be not used for physical or ephemeral H.248 terminations.

### Summary for H.248 Protocol

Status:

- For the “MGC polling by MG” case exists a standardized mechanism by package H.248.14.

- For the “MG polling by MGC” case is no specific package needed. Furthermore H.248.14 is not intended for this case. There’s no explicit description in H.248.1 in case that the MGC wants to poll the MG, but there’s a hint in H.248.14.

## H.248 Applications

The “MG polling” case is already a specific “H.248 application” in the MGC. There might be further proprietary implementations in order to extend or complement previous mechanisms on H.248 protocol level. But there wasn’t any specific need identified up to now.

# Conclusions

Peer-to-peer operation state monitoring is an essential feature for high available networks. Basically this feature may be realized by continuously checking the MGC-MG interconnection.

In case that the used signaling transport technology (here H.248 transport according H.248.4 or H.248.5) is already providing such a capability (here “heartbeat” in case of SCTP for H.248-over-SCTP/IP, and “keep alive “ in case of SSCOP for H.248-over-ATM), it is recommended to use that mechanism, and other additional mechanism are not required.

In the other case of unreliable transport technologies (here H.248 over UDP/IP or TCP/IP), H.248 protocol level mechanisms are applicable (as described in section 5.3). Existing H.248 capabilities inclusive inactivity timer package H.248.14 are sufficient mechanisms.

Further need for additional mechanisms wasn’t identified up to now.

# Appendix: History, Proposals and Discussions

The H.248.14 Inactivity Timer Package was a result of a long debate of the H.248 community. There was initially a discussion on the MEGACO mailing list on the issue of MGC and MG polling each other to detect failures:

- Polling of MG by MGC, and vice versa,

- Polling of MGC by MG.

The discussion did start around May 2001. The then-chairman did summarize the first discussion end of July, by concluding six possible alternatives. Two proposals were based on new packages, a “Polling Package” and a “Timer Event Package”.

Both mechanisms were more or less equivalent, but the configuration management concept concerning the “polling timer” was different. There was also consensus that the “polling load” put on the MGC should be minimized. This requirement was resulting in the capability, that the MGC must be able to control the “polling frequency” in all associated MGs.

The first written proposal was a “Keep Alive Package”, presented in the Dublin Rapporteurs meeting in autumn 2001 [2]. The text was progressed in the subsequent SG16 plenary meeting in February 2002 ([3], [4]), and become finally the “Inactivity Timer Package” in March 2002 [1].

___________________

The period length typically depends on the network level and may range from multiple 100 ms till multiple seconds.

e.g., H.248.1 chapter 8: “Applications SHOULD implement an application level timer per transaction.”

see: ftp://ftp.ietf.org/ietf-mail-archive/megaco/2001-05.mail

Email subject: “Polls of MGC”

see http://www1.ietf.org/mail-archive/web/megaco/current/msg00123.html 
 	some reactions:
 	http://www1.ietf.org/mail-archive/web/megaco/current/msg00127.html
	http://www1.ietf.org/mail-archive/web/megaco/current/msg00128.html 	
	http://www1.ietf.org/mail-archive/web/megaco/current/msg00137.html