ITU Telecommunication Standardization Sector	Document AVD-2533

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question:	3/16

Source*:	ALCATEL

Title:		H.248.PCMSB – Revised Error Codes

Purpose:	Discussion

___________________

1	Introduction	1

2	References	1

3	Discussion	2

3.1	Error Case I: Unexpected Bit State	2

3.1.1	Issue	2

3.1.2	Proposed Alternative	2

3.1.3	Proposed Text Changes in H.248.PCMSB	2

3.2	Error Case II: Moving TDM Terminations representing E1 Channel 0	2

3.2.1	Issue	3

3.2.2	Proposed Alternative	3

3.2.3	Proposed Text Changes in H.248.PCMSB	3

4	Conclusion	3

# Introduction

The H.248.PCMSB Package [1] contains two error codes. Discussions in the past May 2004 Rapporteurs meeting did question the appropriateness of the proposed code points [2]. This contribution proposes alternatives.

# References

- PCM Frame Spare Bit Package (H.248.PCMSB) Edition 3. ITU-T WP2/16 Rapporteurs meeting, Document: TD-43, May 2004.	
http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/TD-43.zip

- New Error Code for Package H.248.PCMSB. ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2453, May 2004.	
http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/AVD-2453.zip

# Discussion

## Error Case I: Unexpected Bit State

This refers to section 6.5.2 in [1].

### Issue

Proposed (new) error code 541 in contribution AVD-2453 [2] was considered to be too package specific, thus out of scope of H.248.8. The controversial subjects were the words “spare bit”.

### Proposed Alternative

There are similarities with packages H.248 E.9 and H.248.25 concerning the underlying bit handling and state manipulations functions. H.248.8 error code #540 is exactly expressing the error situation from that perspective. Alcatel did propose #541 because of the word “hook” in #540, which we thought may mislead.

After reviewing again this issue, Alcatel proposes to reuse existing #540 in this error situation due to following reasons:

- Package usage is disjoint. In case of a H.248 TDM Termination configured with H.248.PCMSB, there’s no H.248.1 E.9, and vice versa.

- H.248.8 error code #540 describes accurately the error situation.

- Terminology is considered to be a minor point, particularly from protocol implementation. [The error text in the error descriptor is unambiguous due to the exclusiveness of the packages.]

- There’s certain alignment with other packages for “identical” error situations.

### Proposed Text Changes in H.248.PCMSB

In case that the experts accepting #540, following text changes are proposed:

[Begin Correction]

6.5.2 Unexpected Bit State of a Spare Bit

If the MGC sets an EventsDescriptor containing a Spare Bit transition event with the “strict” (0x0001) parameter set to “failWrong”, and the Spare Bit state is already what the transition implies, the execution of the command containing that EventsDescriptor fails. The MG SHALL include error code 540 “Unexpected initial Spare Bit state” in its response.

[End Correction]

## Error Case II: Moving TDM Terminations representing E1 Channel 0

This refers to section 6.5.3 in [1].

### Issue

Error code 517 (see AVD-2453 [2]) doesn’t match very well due to the scope on stream mode.

### Proposed Alternative

It’s proposed to use the more general code point #501 as MG reaction for the situation, described in H.248.PCMSB clause 6.5.3.

### Proposed Text Changes in H.248.PCMSB

In case that the experts accepting #501, following text changes are proposed:

[Begin Correction]

6.5.3 Moving TDM Terminations from Null Context

H.248.PCMSB is typically used may be applied for “Channel 0” H.248 TDM Terminations in PCM31 or PCM30 systems. For such type of E1 interfaces, Channel 0 of the referring a 2048 kbit/s Basic Frame Structure, in may belong to the H.248 Null Context. In this case, any ADD command, which shifts tries to move this a Channel 0 H.248 TDM termination to an H.248 call Context, is not allowed. Those MGC commands, which request a termination to leave the Null Context, shall be responded by the MG with the error code '517501 - Unsupported or invalid modeNot implemented'.

[End Correction]

# Conclusion

After a lot of discussions this year, Alcatel concludes that

- there’s neither the need for the definition of package specific error codes,

- nor the need for new H.248.8 error codes, and

- that existing H.248.8 error codes are sufficient for unambiguous description of H.248.PCMSB failure scenarios.

Correspondingly modified text is proposed by this contribution.

___________________