ITU Telecommunication Standardization Sector	Document AVD-2531

Study Group 16

Q.F,G,K,2-5/16 Rapporteur Meeting

San Jose (USA), 30 August – 3 September 2004

Question:	3/16

Source*:	ALCATEL

Title:		Package H.248.PCMSB Clarifications (Part II)

Purpose:	Discussion

___________________

1	Introduction	1

2	References	1

3	Discussion	2

3.1	Semi-Permanent Connections and H.248.21 Relation	2

3.2	E1 OAM Functions	2

3.3	Applicability for T1?	2

3.4	Reporting Control?	2

4	Conclusion	2

# Introduction

This H.248 Package was introduced in the January 2004 SG16 meeting. A first set of questions were clarified in the May 2004 Rapporteurs meeting, see clarifications part I in [2]. Additional questions were raised, particularly with regard to the circuit-switched transmission network interface. These PDH rsp. SDH/SONET specific questions shall be clarified in this contribution. The contribution incorporates some feedback, received since the May meeting

# References

- PCM Frame Spare Bit Package (H.248.PCMSB) Edition 0.3. ITU-T WP2/16 Rapporteurs meeting, Document: TD-43, May 2004.	
http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/TD-43.zip

- Package H.248.PCMSB Clarifications (Part I). ITU-T WP2/16 Rapporteurs meeting, Document: AVD-2451, May 2004.	
http://ftp3.itu.ch/av-arch/avc-site/0405_Bei/AVD-2451.zip

# Discussion

## Semi-Permanent Connections and H.248.21 Relation

There is no relation with H.248.21, "Semi-permanent Connection Handling Package". This is because time-slot 0 of the G.704 E1 frame is terminated in the E1 frame terminating network node, that is, in the Media Gateway, and can not be a part of a semi-permanent connection.

## E1 OAM Functions

There’s an “A” bit in E1 channel 0 for Remote Alarm. This bit, and potential other OAM type of functions, are out of scope of H.248.PCMSB package because this type of information is considered to be processed by Element Management Systems (EMS) and Network Management Systems (NMS), but not by H.248 MGCs.

## Applicability for T1?

T1 represents the G.704 Basic frame structure at 1544 kbit/s.

This frame structure is out of scope of H.248.PCMSB, because there aren’t any Spare Bits in this frame structure.

Note: there’s a 4 kbit/s data link defined for T1, but data transport isn’t based on Spare Bits.

## Reporting Control?

There was a question about “control of reports” from MGC side, particularly deactivating of MG “reporting” for spare bits not interested in.

Response: “reporting control” is a basic H.248 capability. The MGC registers only for interested spare bits.

# Conclusion

Contribution proposes clarifications for H.248.PCMSB.

___________________