ITU - Telecommunication Standardization Sector	Temporary Document __ (PLEN)

STUDY GROUP 16

Santiago, Chile, 17-28 May 1999

Question(s):	13

SOURCE*:	Rapporteur Q.13/16 (Dale L. Skran)

TITLE:	Updated Draft Recommendation H.323 Annex F “Simple Endpoint Types”

___________________

This document contains the final text for Decision at the SG16 meeting in Santiago (Chile) in May 1999 with change marks against the White Contribution for H.323 Annex F as found in COM-16-73.

H.323 Annex F: Simple Endpoint Types

Summary

This document introduces the notion of Simple Endpoint Types (SETs) that are designed to operate in a well-defined set scenarios and with a restricted functionality range — in contrast to full-fashioned H.323 multimedia systems — thereby explicitly reducing the overall system and hence the implementation complexity.  In particular, this document defines Simple Endpoint Types for audio communication (Audio SETs) that operate using a well-defined subset of H.323 protocols and are well-suited for IP Telephony applications while retaining interoperability with regular H.323 Version 2 devices.

Note: The subset of H.323 defined in this document is derived from the specific requirements for Audio SETs and also documents current practice as observed in the market place and at interoperability events.

# Introduction

Simple Endpoint Types — i.e. devices manufactured for a single purpose — may comprise a significant fraction of the overall set of H.323 capable end systems.  In contrast to full-featured H.323 devices (many implementations of which are PC-based), the so-called Simple Endpoint Types (SETs) may be implemented in inexpensive stand-alone boxes, the most prominent example being the simple  telephone.

Note: Sample application scenarios for such systems were found to include:

- Palmtop computer with audio communications capabilities (voice, file transfer, fax, ...)

- Telephone with an RJ-45 connector

- Text telephones (using ITU-T Recommendation T.140)

- Cellular IP phone

- Mobile system with integrated voice and data communications (UMTS, IMT2000)

All these systems have in common that they support a relatively fixed set of functionality: voice and/or rudimentary (i.e. not T.120) data communication facilities.  It is important to note that this functionality does not need to be extended for the respective system’s purpose: a telephone set without (an elaborate) display does not need to support video functionality, neither does it require data conferencing capabilities.

All of these systems have a limited amount of resources available (e.g. processing power, communication bandwidth, memory).

This Annex outlines the scope of SET devices in general and defines the procedural and protocol details of a Simple Audio Endpoint Type (Audio SET device).  In particular, this Annex defines the functional baseline for all types of Simple Endpoint Types; hence, further SETs are to be defined by referencing this Annex and then only specifying additions to the procedures and conventions set forth in this Annex.

This Annex defines a subset of H.323 functionality and any deviations from H.323 are explicitly identified.  Any procedures not explicitly described in this Annex are covered by the main body of H.323.

The development of SET devices has potential implications on other H.323 devices: in particular, MC(U)s and gateways should be aware of their potentially minimal support for H.323 version 2 functionality in order to provide SET devices with seamless access to enhanced H.323 services such as multipoint conferences and supplementary services.  Alternatively, external proxy devices may be provided to bridge the different functional ranges between SET devices and full-featured H.323 version 2 endpoints.  Interoperability issues are addressed in more detail in section 9.

# Specification Conventions

This document specifies only those services, procedures, protocol messages, etc. that are mandatory for the implementation of a SET device — which is a subset of the mandatory functionality of an H.323 version 2 (1998) system.  This implies that a  SET device shall not assume any functionality beyond what is specified mandatory in this Annex from another SET device.

In addition to the mandatory components, several sections of this document specify conditionally mandatory services, procedures, protocol messages, etc. based on the concept of functional blocks that are optional as a whole.  However, a  SET device that decides to implement a particular functional block, must support all the components defined as mandatory for this functional block; optional components may be supported.

All other features defined in H.323 are — by definition — optional, and their implementation in a  SET device is entirely at the discretion of the manufacturer.

# Scope

This Annex specifies rules on the use of H.323 that enable Simple Endpoint Types to be implemented in a simple fashion.  The following (non-exhaustive) list of Simple Endpoint Types is envisioned for standardization by the ITU-T:

- Simple telephone (Simple Audio Endpoint Type) — Defined in this document.

- Simple telephone with security capabilities — For further study.

- Text conversation terminal — For further study.

- Fax device — For further study.

The simple telephone is defined in this document.  Secure simple telephone, text terminal, and simple fax device are Simple Endpoint Types for further study.  The profiles for Simple Endpoint Types can be categorized as follows:

Figure 1: Venn diagram showing functional ranges of the various SET devices

Figure 1 is a schematic picture of the different Simple Endpoint Types that are being defined in the context of H.323 ‘profiles’, in a so-called Venn diagram. In this diagram, the relation between the SETs becomes is illustrated.  The wider ellipsis shows the context of a full H.323 compliant system.  As an example, the  simple telephone is put in the figure.  As it is clearly a subset of the full compliant H.323 system, it lays completely within its scope.  A secure simple telephone, containing additionally the security capabilities, comprises the capabilities of the simple telephone (e.g. same audio codecs, same call set-up, etc.).  The interoperability between a simple telephone SET implementation and a secure simple telephone will therefore be ensured.

The SET devices are defined in a way that enables them to interoperate seamlessly with one another and with H.323 version 2 (1998) devices supporting the FastConnect procedure as well as with all SET-aware H.323 endpoints.

Note that — although SET devices are defined with simple devices in mind — it is equally possible to build gateways based upon the respective SET specification.  No additional definitions are required for other types of devices.

# Normative References

All the normative references of H.323 version 2 (1998) and H.225.0 version 2 (1998) apply.

# Abbreviations

SET			— 	Simple Endpoint Type

Audio SET		—	Simple Audio Endpoint Type

Secure Audio SET	—	Secure Simple Audio Endpoint Type

Fax SET		—	Simple Facsimile Endpoint Type

Text SET		—	Simple Text Telephony Endpoint Type

# Simple (Audio) Endpoint Type – System Functionality Overview

The following characteristics apply to Simple Audio Endpoint Types (Audio SET devices):

Media capabilities:

- Voice-capability

- mandatory:	G.711 (A-law and u-law)

- suggested options:	G.723.1, G.729, GSM.

- suggested options:	audio redundancy encoding with any combinations of the above codecs.

- Audio SET devices shall only support symmetric audio operation.

- No data-capability.

- DTMF capability mandatory; transmission as H.225.0 Information messages mandatory; transmission as RTP payload for further study.

- No video capability.

- No T.120 capability.

- Media distribution: support for unicast mandatory.

The mandatory and optional media capabilities shall be defined separately for other Simple Endpoint Types.

Control capabilities:

The following minimum control capabilities shall equally apply to all Simple Endpoint Types.

- FastConnect sequence of H.323 version 2 mandatory.

Note: Audio SET devices are by default capable of participating in multipoint conferences — where they are obviously limited to audio communications.

Most other control capabilities are optional, in particular:

- UDP-based Faster-Connect Draft Recommendation H.323 Annex E optional.

- Supplementary services (solely based upon H.450.x) optional.

- Support for H.245 messages and procedures optional.

- Support of more than a single call / conference at a time is optional.

Some control capabilities are disallowed for Audio SET devices.

- MC functionality prohibited.

# Procedures for Simple Endpoint Types

This section specifies for all the protocols required by H.323 the detailed level of support by  SET devices in general and the specific requirements for Audio SET devices:

- Registration, Admission, and Status (RAS) signaling (H.225.0) — section7.1;

- Call signaling (H.225.0) — section7.2;

- Multimedia system control signaling (H.245) — section 7.3;

- Media packetization and transport (H.225.0, RTP) — section 7.4;

- Supplementary Services (H.450.x) — section 7.5 and 7.6;

- Multipoint conference operation — section 7.7;

- Loosely-coupled conferences (H.332) — section 7.8;

- Management Information Bases — section 7.9.

Security services as specified in ITU-T Recommendation H.235 to create Secure  SET devices are considered in section 8.

## RAS Signaling (H.225.0 RAS)

SET devices shall comply with the RAS procedures as defined in H.323 version 2 and H.225.0 version 2 with the following modifications applying.A SET device shall use the pre-granted ARQ procedures as specified in H.225.0 version 2 and shall be able to determine whether an incoming call request is received from its Gatekeeper.  A SET-aware Gatekeeper shall support pre-granted ARQ and shall pre-grant for placing and receiving calls with call routing through the Gatekeeper for SET devices (to be indicated in the preGrantedARQ component).  If a contacted Gatekeeper does not support pre-granted ARQ or does not provide the aforementioned pre-granting configuration, a SET device shall register with another Gatekeeper.

SET devices shall at a minimum support for the following RAS messages: transmission of GRQ, RRQ, URQ, UCF, and XRS and reception of GCF, GRJ, RCF, RRJ, URQ, UCF, URJ, and XRS.  SET devices may support additional RAS messages.

A SET device shall include the “set” component of the H.225.0 EndpointType when communicating with a Gatekeeper and set the bits as follows.

Bit 0:	=1	if the device has Audio SET functionality

Bit 1:	=0	if the device is not conference-aware

Bit 1:	=1	if the device is conference-aware

Use of the other bits will be defined by additional SET spefications.

## Call Signaling (H.225.0 Call Control)

SET devices shall comply with the call control procedures defined in H.323 version 2 and H.225.0 version 2. SET devices shall not close the call signaling channel after call establishment.

SET devices shall implement the FastConnect procedures as specified in H.323 version 2.  When originating a call, a  SET device shall place a call using FastConnect.

SET devices shall support H.225.0 Information messages in the call signaling channel.  Such messages should be used for – but are not restricted to – conveying user input in the Keypad Information Element.

SET devices should use the Status Enquiry and Status messages of H.225.0 to estimate round-trip times to its peer.

SET devices may implement UDP-based call setup as outlined in H.323 Annex E.  If UDP-based call setup is implemented, a SET device should attempt to call another endpoint via UDP-based call setup first.

Implementation of supplementary services based upon H.450.x is optional for SET devices. SET devices shall be able to safely ignore H.225.0 Facility messages that they do not understand.

A SET device shall include the “set” component of the H.225.0 EndpointType when exchanging call signaling PDUs with its peer.  The bits of the “set” components shall be set as defined in section 7.1.

## Multimedia System Control Signaling (H.245)

### H.245 Control Channel

The FastConnect procedure shall be used for connection establishment.  Repeated transmission of the fastStart element in H.225.0 call signaling messages shall be used to reconfigure or reroute media streams.

SET devices shall not open a separate H.245 connection:

- They shall restrict H.245 signaling to the OpenLogicalChannel structure in the FastConnect sequence along with implicit Master Slave Determination.

- If further H.245 signaling is required, they shall perform tunneling as defined in H.225.0 version 2.

SET devices shall use the syntax of H.245 version 4 or higher (1998).

No specific procedures are defined for H.245 messages.  If SET devices implement H.245 functionality, they shall adhere to the procedures defined in H.323, H.225.0, and H.245.

### Master-Slave Determination

SET devices shall implicitly assume the slave role in any communication relationship without an H.245 control channel.

In case an H.245 tunnel is established, following the rules of section 6.2.8.4 of H.323 version 2, the SET device shall indicate a value of 40 for the terminalType.  This ensures that in case a SET device connects to a full H.323 version 2 (1998) device, the latter will win the Master-Slave-Determination.

### Terminal Capability Exchange

Although SET devices are by definition restricted in their supported functional range, a capability exchange procedure cannot be circumvented to allow for a minimum of diversity in the devices.  However, the range of possible capabilities that may be signaled by a  SET endpoint is restricted to what is defined in the following, and the capability exchange procedures shall adhere to the rules set forth in this section.

The Capability Exchange procedure for media types and transmission modes shall be carried out following the rules of the FastConnect procedure using multiple Open Logical Channel structures  as a selection of possibilities offered by the caller out of which the callee chooses a subset to send and receive.

The following subsection list which capabilities need to be understood on the receiving (called) side and which may be transmitted on the sending (calling) side for Audio SET devices.

#### Audio Capability

- G.711 (u-Law, A-Law, 56 kbit/s, 64 kbit/s)
The following alternatives shall be supported:

AudioCapability.g711Alaw64k	 20	number of frames

AudioCapability.g711Alaw56k	 20	number of frames

AudioCapability.g711Ulaw64k	 20	number of frames

AudioCapability.g711Ulaw56k	 20	number of frames

- G.723.1 (silence suppression or not, low and high rate)
A SET supporting G.723.1 must at a minimum support:

AudioCapability.g7231

maxAl-sduAudioFrames	 1	number of frames

silenceSuppression	True/False as appropriate

- G.729 (plain or Annex A)
A SET supporting G.729 must at a minimum support:

AudioCapability.g729	 1	number of frames

AudioCapability.g729AnnexA	 1	number of frames

- GSM (full rate, enhanced full rate, half rate).
A SET supporting GSM must at a minimum support

AudioCapability.gsmFullRate	GSMAudioCapability,

AudioCapability.gsmHalfRate	GSMAudioCapability,

AudioCapability.gsmEnhancedFullRate	GSMAudioCapability

with GSMAudioCapability defined as appropriate for each of these rates:

GSMAudioCapability.audioUnitSize	 1	number of frames

GSMAudioCapability.comfortNoise	True/False as appropriate

GSMAudioCapability.scrambled	True/False as appropriate

#### Video Capability

Audio SET devices do not support video.

#### Data Capability

Audio SET devices do not support data.

#### Conference Capability

SET devices are assumed to be proxied into centralized conferences with centralized data distribution (see section 7.6).

#### User Input Capability

SET devices shall support transmission of DTMF as Keypad Information Elements in the H.225.0 call signaling connection (e.g. using Information messages).

#### Security Capability

Security for  SET devices — i.e. the definition of Secure  SET devices — is for further study.  Refer also to section 8.

#### maxPendingReplacementFor

Shall be supported by Audio SET devices.  A value equal to ‘1’ shall be implicitly assumed:

maxPendingReplacementFor  	= 1

Hence, the maxPendingReplacementFor parameter shall not be signaled explicitly.

#### nonStandardCapability

Use of non-standard capabilities — on the top level of the capability structure as well as within the aforementioned capability categories — should be avoided as far as possible.

#### Additional Rules for the Use of Capabilities

For Audio SET devices, audio capabilities shall only be signaled via the FastConnect procedure and repeated exchange of OpenLogicalChannel structures using the FastConnect.

Video capabilities, data capabilities, conference capabilities, security capabilities, and h233encryption capabilities shall not be used.

The values of the MultiplexCapability table entry of an Audio SET device shall be assumed as follows:

maximumAudioDelayJitter	 250ms

receiveMultipointCapability,

transmitMultipointCapability, and

receiveAndTransmitMultipointCapability

multicastCapability	TRUE/FALSE as appropriate, default FALSE1)

multiUnicastConference	TRUE/FALSE as appropriate, default FALSE1)

mediaDistributionCapability

centralizedControl	TRUE

distributedControl	FALSE

centralizedAudio	TRUE

distributedAudio	TRUE/FALSE as appropriate, default FALSE1)

centralizedVideo	FALSE

distributedVideo	FALSE

centralizedData 	ABSENT

distributedData	ABSENT

mcCapability

centralizedConferenceMC	FALSE

decentralizedConferenceMC	FALSE

rtcpVideoControlCapability	ABSENT

mediaPacketizationCapability	ABSENT

...,

transportCapability	ABSENT

redundancyEncodingCapability	Audio redundancy encoding only (if any)

logicalChannelSwitchingCapability	FALSE

t120DynamicPortCapability	FALSE

1) Multicast, multi-unicast, and distributed audio may be supported by Conference-aware Audio SET devices.

Capabilities signaled from the remote side that are not understood shall be ignored.

### Logical Channel Signaling Messages

The opening of logical channels shall adhere to the FastConnect specifications of H.323 version 2.

In addition,  SET devices shall support reconfiguration of media streams at any time during a call . Open Logical Channel structures shall be tunneled in H.225.0 call signaling messages following the procedures defined in H.225.0 version 2 (1998) and H.323 version 2 (1998) re-using the fastStart element of the H.225.0 call signaling message.  Open Logical Channel structures outside the FastConnect procedure shall be used to alter media stream parameters – to provide a basis for supplementary services.  Such Open Logical Channel structures shall be interpreted upon reception as follows.

- If the logical channel number matches a currently open logical channel, the respective channel shall be reconfigured following the principles of the FastConnect procedure if the dataType component is not “null”.  If the dataType component is “null” – indicating a “NullChannel” – the respective logical channel shall be considered closed and media transmission on this logical channel shall cease.

- If the logical channel number does not match a currently open channel, a new logical channel shall be opened following the princicples of the FastConnect procedure.

In the following, the restrictions on Open Logical Channel request are outlined:

OpenLogicalChannel

forwardLogicalChannelNumber	LogicalChannelNumber

forwardLogicalChannelParameters

portNumber	ABSENT

dataType	a valid audio data type (section 7.3.3.1)

multiplexParameters	CHOICE: h2250LogicalChannelParameters

forwardLogicalChannelDependency	ABSENT,

replacementFor	used if another Logical Channel is to be replaced

reverseLogicalChannelParameters

dataType	a valid audio data type (section 7.3.3.1)

multiplexParameters	CHOICE: h2250LogicalChannelParameters

reverseLogicalChannelDependency	LogicalChannelNumber OPTIONAL,

replacementFor	used if another Logical Channel is to be replaced

separateStack	ABSENT

encryptionSync	ABSENT for Audio SET devices; FFS.

To the H2250LogicalChannelParameters structure, the following restrictions apply:

H2250LogicalChannelParameters

nonStandard	should be ABSENT

sessionID	INTEGER(0..255)

associatedSessionID	ABSENT

mediaChannel	TransportAddress – should be a unicast address

mediaGuaranteedDelivery	ABSENT

mediaControlChannel	PRESENT – reverse RTCP channel

mediaControlGuaranteedDelivery	FALSE

silenceSuppression	as appropriate

destination	typically ABSENT

dynamicRTPPayloadType	as appropriate,

mediaPacketization	as appropriate; may only specify the payload format used

rtpPayloadType

payloadDescriptor	should refer to an rfc-number

payloadType	(dynamic) payload type value to be used

transportCapability

nonStandard            	should be ABSENT

qOSCapabilities	should be ABSENT (may only contain RSVP parameters)

mediaChannelCapabilities	should be ABSENT (may indicate “ip-udp”)

redundancyEncoding	optional; only audio redundancy is allowed

source		typically ABSENT

## Media Exchange

For media exchange, SET devices shall follow the H.323 and H.225.0 procedures using RTP / UDP / IP to convey the media streams.  The appropriate media packetization formats shall be used.

## Supplementary Services (H.450.x)

Support of any of supplementary services according to the H.450 series of ITU-T Recommendations is optional.

Note: If H.450.x functionality is not provided by a  SET device, the SET device should implement the message rejection functionality (Interpretation APDU) of H.450.1 to enable its peer to quickly determine non-availability of supplementary services on side of the  SET device.  If H.450.1 message rejection is not implemented the peer has to rely on a timeout.

A baseline for supplementary services to be supported by SET devices is for further study.

## Third-party initiated pause and rerouting

Support for third-party initiated pause and rerouting is similar to the procedures outlined in H.323 version 2 (1998) section 8.4.6 – with the following modifications applying.

### Initiating side

To re-route a call connecting to a SET device its peer (typically a Gatekeeper) shall transmit a NullChannel specification in the fastStart element in a message of the call signaling channel.

Subsequently, the initiating entity shall again transmit the (for the new peer) appropriate OpenLogicalChannel structures – similar to the capability negotiation and media stream establishment in the FastConnect procedure – and include the new transport addresses to redirect the media stream sourced by the  SET device.  The OpenLogicalChannel structures are carried in an H.225.0 call signalling message.

The OpenLogicalChannel structure should offer the same audio encodings that were offered in the initial call.

### Receiving side ( SET device)

Upon reception of a NullChannel specification in a fastStart element, a SET device shall stop transmitting the media stream(s) immediately and shall be prepared to handle interruptions in the received media stream(s).  The  SET device shall expect a repeated exchange of capability and transport addresses following the principles of the FastConnect procedure.

Upon reception of an OpenLogicalChannel structure carried in an H.225.0 call signaling message, the  SET device shall select an acceptable media encoding from the selection offered by the initiating entity – following the rules of the FastConnect procedure.  The  SET device shall then start transmitting its media stream(s) to the transport address(es) newly indicated in the OpenLogicalChannel structures.

## Conference-Mode Operation

SET devices may participate in multipoint conferences in either of two ways:

- by being proxied into a conference through a dedicated external device — such as a SET-aware MC combined with a suitable MP or a SET-specific proxy as outlined in section 7.7.1 as the default mode of operation for  SET devices; or

- by implementing the necessary procedures of the H.225.0 and H.245 protocols as outlined in this section. This mode of operation is defined in section Error: Reference source not found.

### Conference-unaware SET Devices

The default mode of operation for  SET devices does not require any awareness of conferencing functionality in a  SET device itself.  Instead, an external entity is assumed that bridges between a full-featured H.323 device and the  SET device.  This logical entity may be a stand-alone proxy device or may be part of an MC(U), a Gateway, or a Gatekeeper.

Note: The functionality of a logical bridging entity may include the following:

- concealing the existence of conference-related H.245 commands and responding appropriately in the direction of the full-featured H.323 device;

- adapting H.245 capability and logical channel signaling including multipoint mode commands;

- mixing several incoming audio streams and providing a single stream to the  SET device;

- translating transport addresses for the audio stream;

- transcoding audio streams; and

- offering access to conference control functions via simple input means (such as DTMF signaling) to the  SET device.

### Conference-aware SET Devices

The specification of conference-aware SET devices is for further study.

Nevertheless, SET devices may follow the full procedures for conference-mode operation defined in the H.323 Series of Recommendations.

## Support for Loosely-coupled Conferences (H.332)

Support for loosely-coupled conferences according to ITU-T Recommendation H.332 is optional:

- Participation as a member of the panel is optional; it is provided either if conference-mode operation and media distribution via multicast are supported, or if an appropriate MC/MP combination hides all the conference commands from the  SET device and only presents a single audio-stream.

- Participation as a member of the audience is optional; it is possible if the SET device supports multicast reception of information and is capable of receiving and interpreting H.332 session announcements.

## Management Information Bases (MIBs)

Implementation of Management Information Bases is optional for  SET devices.  If MIBs are included in the implementation, the following H.323-related MIBs should be implemented:

- Call signaling,

- Terminal entity,

- RAS,

- Real-time Protocol (RTP).

Details are for further study.

# Security Extensions

Plain  SET devices are not capable of supporting H.235 security services.  Secure  SET devices, however, define a simple extension to SET devices covering security functionality using a subset of the mechanisms specified in H.235.

The details of Secure  SET devices are for further study.

# Interoperability Considerations

This Annex specifies a  SET device as a well-defined subset of the total H.323 functionality.

SET devices should always be used in conjunction with SET-aware Gatekeepers.  The SET-aware Gatekeeper shall perform pre-granted ARQ and shall employ the Gatekeeper-routed call model to ensure full interoperability with other H.323 version 1 (1996) and version 2 (1998) devices.

In addition, SET-awareness may be built into MC(U)s or gateways to achieve seamless interoperability.

The following table presents an overview of interoperability achieved between Audio SET devices and other H.323 endpoints.

|  | H.323 version 1 | H.323 version 2 | H.323 version 2 with Fast Connect | SET device |
| --- | --- | --- | --- | --- |
| H.323 version 1 |  |  |  |  (GK) |
| H.323 version 2 |  |  |  |  (GK) |
| H.323 version 2 with Fast Connect |  |  |  |  (*) |
| SET device |  (GK) |  (GK) |  (*) |  |
| (GK) — indicates that a SET-aware Gatekeeper is needed for interoperation. (*) — optional redirection of media channels requires repeated execution of FastConnect in both endpoints |  |  |  |  |

Table 1/Annex F: Interoperability of SET devices with other H.323 devices

# Implementation Notes (Informative)

This section provides informative text on simple encoding most of the necessary H.245 messages without requiring specific ASN.1 en/decoders.

Note that all these messages are transmitted as tunneled H.245 messages; i.e. the resulting bit patterns are encoded as a single OCTET STRING of the SEQUENCE in the fastStart component of a H323-UU-PDU.  In the tables shown below, the leftmost octet (octet #0) of the first row (word #0) is placed in the first octet of the octet string, followed by octet #1 of the first row, and so on.  Octet #3 of word #n is followed by octet #0 of word #(n+1).

If numbers are to be encoded, 2-complement encoding is used for numbers that may be negative.  Otherwise, simple binary encoding is used.  Encoding of numbers spanning multiple octets is done in a way that the most significant bit of the encoded value is located in the first octet of the value (network byte order).

## Open Logical Channel

The OpenLogicalChannel structures are used by SET devices during the FastConnect procedure to indicate their capabilities and simultaneously open media channels in both directions and to reconfigure media streams during a conference.  By definition, the OpenLogicalChannel structures contain only either forward logical channel parameters or backward logical channel parameters.

### Forward Logical Channel Parameters

An Open Logical Channel structure containing only ForwardLogicalChannel parameters may be coded in three different ways, depending on the audio type (Au-Type) and the X bit.

#### G.711 and G.729

The most common structure is the following (G.711, G.729, G.729 Annex A):

|  |  | Octet #0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #1 |  |  |  |  |  |  |  |  | Octet #2 |  |  |  |  |  |  |  |  | Octet #3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 6 | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| 0 | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  | Logical Channel Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | X |  |  |  |
| 4 | AuType |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | # samples |  |  |  |  |  |  |  |  | 0x80 |  |  |  |  |  |  |  |  | length=0x0A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 0x04 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  | session id |  |  |  |  |  |  |  |  | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| 12 | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | RTCP: UDP port number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Logical Channel Number:

This field contains the number of the H.245 logical channel – 1.

X bit:	Used to distinguish between basic and extended audio types.  If X=0, Au-Type (see next field) applies; otherwise (X=1), the extended audio types described below apply (primarily GSM) along with a different packet structure.

Au-Type:	Identifies the audio codec to be used.  The following values are acceptable for AuType.  The leftmost bit is placed in bit 1 of octet #3 above, the rightmost in bit 5 of octet #4.

| No | Codec description | AuType value |
| --- | --- | --- |
| 1 | G.711 A law 64 kbit/s | 0001 |
| 2 | G.711 A law 56 kbit/s | 0010 |
| 3 | G.711 u law 64 kbit/s | 0011 |
| 4 | G.711 u law 56 kbit/s | 0100 |
| 5 | G.723.1 | 1000 |
| 6 | G.729 | 1010 |
| 7 | G.729 Annex A | 1011 |
| 8 | GSM and others (see below) | X=1 |

samples:	For codecs 1, 2, 3, 4, 6, and 7 this component contains the number of samples – 1 per audio packet as defined in H.245.

session id:	Contains the session id parameter to be used in conjunction with RTP/RTCP.

M bit:	Multicast address bit: indicates that the following address is a multicast address.  While many address types are defined besides IPv4 (including IPv6 and IPX), the structures shown here are only valid for IPv4 addresses.

RTCP IP address/port: Contains the transport address for the RTCP receiver reports to be sent to.

#### G.723.1 Codec

For G.723.1, the structure is slightly modified as follows:

|  |  | Octet #0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #1 |  |  |  |  |  |  |  |  | Octet #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| 0 | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Logical Channel Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | X |  |  |  |
| 4 | AuType |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | # samples |  |  |  |  |  |  |  |  | S |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | length=0x0A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x04 |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | session id |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RTCP: port number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The meaning of the fields is identical to the meaning defined for the above format.  In addition, the following fields are relevant:

S bit:	Indicates support for silence suppression if S=1.

#### GSM

For GSM – identified by bit #1 of octet #3 set to X=1 – the structure looks as follows:

|  |  | Octet #0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #1 |  |  |  |  |  |  |  |  | Octet #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| 0 | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Logical Channel Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | X |  |  |  |
| 4 | Ext. AuType |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0x03 |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # samples |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | C |  | S |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0x80 |  |  |  |  |  |  |  |  | length=0x0A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | session id |  |  |  |  |  |  |  |  | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | RTCP: IP addr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RTCP: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | UDP port number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The fields have the same meaning as in the above packet formats.  In addition, the following fields are defined for GSM:

Ext. Audio Type:	Identifies the extended audio codec:

GSM Full Rate	= 000 0011

GSM Half Rate	= 000 0100

GSM Enhance Full Rate = 000 0101

C bit:			C=1 indicates support / use of comfort noise

S bit:			S=1 indicates support / use of scrambling

### Reverse Logical Channel Parameters

Open Logical Channel Message containing ReverseLogicalChannel parameters are encoded as described in this subsection.

#### G.711, G.729

The most common structure is the following (G.711, G.729, G.729 Annex A):

|  |  | Octet #0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #1 |  |  |  |  |  |  |  |  | Octet #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 6 | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| 0 | 0x40 |  |  |  |  |  |  |  |  |  |  |  |  |  | Logical Channel Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 0x04 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x01 |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | X |  |  |  |
| 8 | AuType |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | # samples |  |  |  |  |  |  |  |  | 0x80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | length=0x11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 0x14 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  | session id |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| 16 | RTP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | RTP: port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RTCP: port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | RTCP: port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The fields have the same meaning as above.  In addition, the following fields are defined:

RTP IP address/port:	Target transport address for the RTP audio stream to be sent to.

RTCP IP address/port:	Target transport address for RTCP sender reports to be sent to.

#### G.723.1

For G.723.1, the structure differs slightly from the above as follows:

|  |  | Octet #0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #1 |  |  |  |  |  |  |  |  | Octet #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 |  | 6 |  |  | 5 |  | 4 |  |  | 3 |  | 2 |  | 1 |  | 0 |
| 0 | 0x40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Logical Channel Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 0x04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x01 |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 1 |  | 0 |  |  | 0 |  | 1 |  |  | 1 |  | X |  | 0 |  |
| 8 | AuType |  |  |  |  |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | # samples |  |  |  |  |  |  |  |  | S |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | length=0x11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x14 |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | session id |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | RTP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | RTP IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RTP: port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | M |  |  | 0 |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  |
| 24 | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | RTCP: port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### GSM

For GSM – identified by bit #1 of octet #7 set to X=1 – the structure looks as follows:

|  |  | Octet #0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet #3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |
| 0 | 0x40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Logical Channel Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 0x04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | X |  |  |  |
| 8 | Ext. Au-Type |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 0x03 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # samples |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | C |  | S |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0x80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | length=0x11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0x14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | session id |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | RTP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | RTP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RTP: port number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | RTP: port number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | M |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | RTCP: IP address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RTCP: port number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Ext. Au-Type:	Identifies the extended (GSM) audio codec to be used as follows:

GSM Full Rate	= 000 0011

GSM Half Rate	= 000 0100

GSM Enhance Full Rate = 000 0101

————————