ITU - Telecommunication Standardisation Sector           Document

File:  H323AnnexFComments.doc

STUDY GROUP 16			Original : English

Study Group Meeting                                     _

Santiago May 17-28 1999

Question 13/16

SOURCE:  Ericsson, Sweden

TITLE:   Minor modifications to H.323 Annex F

PURPOSE:  Proposal

# Summary

H.323 Annex F contain a couple of wordings that is contradicting or confusing. This is a proposal to modify the Annex before decision.

Background:

The SET is an important concept for a simple terminal defined in H.323 Annex F. It is intended to be both a general template for SET specifications and a specific Audio SET specification. The following needs for modifications were found when the SET concept was to be applied on the Text SET.

# Proposal

In Section 6, Media capabilities, “Data-capability optional” should be “No data capability”.

(the deleted line mentions facsimili as an example, but the section is valid for the Audio SET only that has no fax capability)

In section 7.1 RAS Signaling,

This is a suggestion to add Bit 2 = 1  if the device has Text SET functionality.

It must also be investigated if the meaning of Bit 0 =1 should be  “if the device has Audio SET functionality”. This may be needed for a Text SET to be able to answer and initiate Audio-only calls, which is a desirable function.

In section 7.6.1. Initiating side, lthe ast sentence should end:

“encodings that were offered in the initial call”.

The encodings prior to call rerouting may be not suitable for the terminal that is connected after the rerouting.

In section 7.6.2 Receiving side,  Last paragraph, second line, should read: “,the SET device shall select acceptible media encoding”

Next sentence should be “The SET device shall then start transmitting its media stream(s)…”

This is to allow e.g. a rerouting from an Audio SET to a Text SET and then add the Text Media stream.

Contact: Gunnar Hellström,  Ericsson
Tel +46 751 100 501  Fax +46 556 002 06 		e-mail  gunnar.hellstrom@omnitor.se