3GPP_TSG_SAWG2#12 		TSG-S2#12 S2-000425

Tokyo, Japan, March 6th – 9th, 2000

Agenda Item: 	Release 2000

Source: 	AT&T, BT, Ericsson, France Telecom, Japan Telecom, Lucent, Motorola, Nokia, Nortel, Rogers Wireless, SBC, T-Mobil, Telenor, Telia, TIM/Cselt

Title: 	Proposal for a Single Call Control Protocol to be used within the Release 2000 Reference Architecture

Document for:	Discussion and Revision to 23.821

# Introduction

The proposed architecture within 23.821 covers both the CS Domain and the PS Domain.  The PS Domain has been enhanced to support multi-media capabilities with the addition of the CSCF and associated call control functionalities.

The contribution highlights the reasons as to why the R2000 architecture should only support a single call control protocol between the terminal and the CSCF (this is for the provision of MultiMedia type services delivered via the PS domain).

# Proposal

This contribuition proposes that the R2000 architecture should support a single call control protocol between the terminal and the CSCF. The call control is to be used for the provision of MultiMedia type services delivered via the PS domain.

The selection and application of a single call control between the terminal and the CSCF will provide the following benefits:

-	Interoperabiltity between operators (offering MultiMedia services within a PS domain) would be simplified, allowing for visited customers to have access to MuliMedia services via the PS domain in the ‘roamed to’ network.

-	Commonality of terminals and mass market terminal provision.  The use of a standardised selected CC protocol in the UE will ensure compatibility for users and the common operation of terminals (compliance to standards) within networks, this will also ease mass market volume production and offer cost per unit benefits.

The selection of a single call control protocol between the UE and the CSCF does not preclude an interworking function at the boundary of the R00 network  for interworking with other call control protocols.

Therefore this contribution proposes that:

- For multi media type services delivered via the PS Domain within the R00 architecture a single Call Control protocol shall be used between the UE and the CSCF.

- The following text is included in Section 8 of TR23.821 Architecture Principles for Release 2000

‘For MultiMedia type services delivered via the PS Domain within the R00 architecture, a single call control protocol shall be used between the user equipment UE and the CSCF.’