Telecommunication Standardization  Sector                 APC-1352R2 (TD-22)

Original: English

Experts Group for Video Coding and Systems in           		April 13-17, 1998

ATM and Other Network Environments

STUDY GROUP 16

CONTRIBUTION

Source:	Jim Toga, Intel Corporation  (Editor)

email:	jim.toga@intel.com

voice:	+1 (503) 264-8816

fax:		+1 (503) 264-3485

Date: April 10, 1998

TITLE:

Draft: H.323 Implementers Guide - Rev 2

PURPOSE:   Draft

ITU - Telecommunication Standardization Sector    Temporary Document xxx

STUDY GROUP 16						Original: English

____________________________

Geneva xxxx  1998

Question : 12,13,14/16

SOURCE :	Rapporteur for former Q.13/16 (D. Skran)

TITLE : 	H.323 Implementers Guide

This document provides a text for correction and clarifications to H.323 (2/98), H.225.0 (2/98), H.235 (2/98) H.45X (2/98) and H.245 (2/98) which have been produced based on the discussions at interregnum Rapporteurs meetings.

Editor:		Jim Toga

Intel

+1 503 264 8816 (voice)

+1 503 264 3485 (fax)

jim.toga@intel.com

| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | April 1998 Original: English |

Question:  13/16

Geneva 1998

STUDY GROUP 16 – CONTRIBUTION

| SOURCE: | Rapporteur Q13/16 |  |
| --- | --- | --- |
| CONTACT: | Dale Skran Ascend Communications 621 Shrewsbury Avenue Shrewsbury, NJ. 07702 USA | Tel: +1 (908) 450-1636 Fax: +1 (908) 972-6389 E-Mail: dskran@ascend.com |
| TITLE: | Implementers Guide for the ITU-T H.323 Recommendation Series Version 2 — Packet-Based Multimedia Communication Systems. |  |
| VERSION: | 1 |  |
| STATUS: | Changes presented at ITU-T SG16 Rapporteurs meeting Yokuska, Japan |  |
| DATE: | April 1998 |  |
| Abstract: | This document is a compilation of reported defects identified with the 1998 decided editions of the ITU-T H.323 (H.225.0, H.235, H.245 and H.45x.) series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementers. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.323 series Recommendations. |  |

Contact Information

| ITU-T Study Group 16 / Question 13 Rapporteur | Dale Skran Ascend Communications 620 Tinton Avenue Building A, Second Floor Tinton Falls, NJ. 07724 USA | Tel: +1 (908) 578-3101 Fax: +1 (908) 578-3131 E-Mail: dale.skran@ascend.com |
| --- | --- | --- |
| ITU-T Study Group 16 / Question 14 Rapporteur ITU-T Recommendation H.323 Editor | Gary Thom Delta Information Systems 300 Welsh Road, Bldg 3 Horsham, PA. 19044-2273 USA | Tel: +1.215.657.5270 Fax: +1.215.657.5273 E-Mail: gthom@delta-info.com |
| Implementers Guide Editor ITU-T Recommendation H.235 Editor | James Toga Intel Corporation 2111 NE 25th Avenue JF3-212 Hillsboro, OR. 97124-5961 USA | Tel: +1.503.264.8816 Fax: +1.503.264.3485 E-Mail: jim.toga@intel.com |
| ITU-T Recommendation H.225.0 Editor | Glen Freundlich Lucent Technologies 11900 N. Pecos St. Westminster, CO 80234 USA | Tel: +1.303.538.2899 Fax: E-Mail: ggf@dr.lucent.com |
| ITU-T Recommendation H.245 Editor | Mark Reid Video Server Incorporated | Tel: +1 (781) 505-2368 Fax: +1 (781) E-Mail: mreid@videoserver.com |

Document History

| Revision | Date | Description |
| --- | --- | --- |
| A | April 1998 | Initial version — completed at the ITU-T Study Group 16 Rapportuers meeting, Yokuska, Japan April 1998 |

Table of Contents

1.	Introduction	5

2.	Scope	5

3.	Policies for Updating This Document	5

4.	Defect Resolution Procedure	5

5.	References	5

6.	Nomenclature	6

7.	Technical and Editorial Corrections	7

7.1	Technical and Editorial Corrections to ITU-T Recommendation H.323	7

7.2	Technical and Editorial Corrections to ITU-T Recommendation H.225.0	8

7.2.1	8

7.2.2	H.225.0 Non Standard Message	9

7.16	Non-Standard Message	9

7.2.1	ANNEX H - H.225.0 Message Syntax (ASN.1)	9

7.3	Technical and Editorial Corrections to ITU-T Recommendation H.245	12

7.4	Technical and Editorial Corrections to ITU-T Recommendation H.235	12

9.	Multipoint Procedures	13

9.1	Authentication	13

7.4.1	Diffe-Hellman exchange with optional Authentication	13

7.4.2	Corrections to ANNEX A	14

7.4.2	Corrections to ANNEX B	15

7.4.3	Corrections to Appendix I	17

7.3	Technical and Editorial Corrections to ITU-T Recommendation H.45X	17

8.	Implementation Clarifications	18

8.1	Token Usage in H.323 systems	18

8.2	H.235 Random Value  Usage in H.323 systems	18

# 1.	Introduction

This document is a compilation of reported defects identified with the 1998 decided editions of the ITU-T H.323 series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementers. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.323 series Recommendations.

The first version of the guide was produced following the April 1998 ITU-T Study Group 18 meeting.  Wide distribution of this document is expected and encouraged.

# 2.	Scope

This guide resolves defects in the following categories:

- editorial errors

- technical errors such as omissions or inconsistencies

- ambiguities

In addition the Guide may include explanatory text found necessary as a result of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories. Proposals for new features should be made in the normal way through contributions to the ITU-T.

# 3.	Policies for Updating This Document

This document is managed by the ITU-T Study Group 16 Question 3 Rapporteur’s group.  It can be revised at any recognized Q3/16 Rapporteur’s group meeting provided the proposed revisions are reached through consensus by the members of the group.  A revision history cataloguing the evolution of this document is included.

# 4.	Defect Resolution Procedure

Upon discovering technical defects with any components of the H.323 Recommendations series, please provide a written description directly to the editors of the affected Recommendations with a copy to the Q3/13 or Q3/14 Rapporteur.  The template for a defect report is enclosed.  Contact information for these parties is included in this document.  Return contact information should also be supplied so a dialogue can be established to resolve the matter and an appropriate reply to the defect report can be conveyed.  This defect resolution process is open to anyone interested in H.323 series Recommendations.  Formal membership in the ITU is not required to participate in this process.

# 5.	References

This document refers to the following H.323 series Recommendations:

	ITU-T Recommendation H.323 (1998),  Packet-Based Multimedia Communications Systems

	ITU-T Recommendation H.225.0 (1998), Call Signaling Protocols and Media Stream Packetization for Packet Based Multimedia Communications Systems

	ITU-T Recommendation H.245 (1998), Control Protocol for Multimedia Communication

	ITU-T Recommendation H.235 (1998), Security and Encryption for H Series (H.323 and other H.245 based) multimedia terminals.

# 6.	Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

| Symbol | Description |
| --- | --- |
| [Begin Correction] | Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| [End Correction] | Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| ... | Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity. |
| --- SPECIAL INSTRUCTIONS --- {instructions} | Indicates a set of special editing instructions to be followed. |

# 7.	Technical and Editorial Corrections

## 7.1	Technical and Editorial Corrections to ITU-T Recommendation H.323

| Description: | An inconsistency in the use of channel addressing within the FastStart procedure has been discovered. This information will be contained in the revision 3 of H.323 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.323 document that was submitted for approval in 1998. As a part of the “Fast Connect Procedure”, it is the responsibility of the called endpoint to decide about the coupling of proposed incoming and outgoing media streams for each SessionId. According to RTP/RTCP specifications, media streams in the same session, must use a common RTCP channel. Additionally, many RTP/RTCP implementations mandate adjacent odd/even port pairs to be allocated. This requirement mandates that for any and all fastStart structures that are proposed in a SETUP message, those with common sessionID values shall also have common mediaChannelControl values. This change may affect the functionality or local operation of an implementation. |
| --- | --- |

[Begin Correction]

8.1.7.1  Proposal, Selection, and Opening of Media Channels

...

In an OpenLogicalChannel which proposes a channel for transmission from the called endpoint to the calling endpoint, the reverseLogicalChannelParameters element shall be included and contain parameters specifying the characteristics of the proposed channel. The forwardLogicalChannelParameters element must also be included (because it is not optional), with the dataType element set to nullData, multiplexParameters set to none, and all optional elements omitted. Alternative proposals for the same receive channel shall contain the same sessionID value in H2250LogicalChannelParameters. All alternative OpenLogicalChannel > structures, that propose a channel for transmission from the called > endpoint to the calling endpoint, shall contain the same > sessionID and the same mediaChannel value. The other H2250LogicalChannelParameters and dataType within reverseLogicalChannelParameters shall be set to correctly describe the receive capabilities of the calling endpoint associated with this proposed channel. The calling endpoint may choose to not propose any channels for transmission from the called endpoint to the calling endpoint, such as if it desires to use H.245 procedures later to establish such channels.

> In the SETUP message, each OpenLogicalChannel which proposes a channel > for transmission from the calling endpoint to the called endpoint, > shall contain mediaControlChannel element (indicating the reverse RTCP > channel) into the H2250LogicalChannelParameters element of the > forwardLogicalChannelParameters structure.

> In the SETUP message, each OpenLogicalChannel which proposes a channel > for transmission from the called endpoint to the calling endpoint, > shall contain mediaControlChannel element (indicating the RTCP channel > going in the same direction) into the H2250LogicalChannelParameters > element of the reverseLogicalChannelParameters structure. > All mediaControlChannel elements inserted by the calling endpoint for > the same sessionID for both directions shall have the same value.

Upon receipt of a SETUP message containing fastStart, determining that it is willing to proceed with the Fast Connect procedure, and reaching the point in the connection at which is is ready to begin media transmission, the called endpoint shall choose from amongst the proposed OpenLogicalChannel structures containing reverseLogicalChannelParameters elements for each media type it wants to transmit, and from amongst the proposed OpenLogicalChannel structures specifying forwardLogicalChannelParameters (and omitting reverseLogicalChannelParameters) for each media type it wants to receive. If alternative proposals are presented, only one OpenLogicalChannel structure shall be selected from amongst each alternative set; alternatives within a set have the same sessionID. The called endpoint accepts a proposed channel by returning the corresponding OpenLogicalChannel structure in any Q.931 message sent in response to SETUP, up to and including CONNECT. The called endpoint may choose to not open media flow in a particular direction or of a particular media type by not including a corresponding OpenLogicalChannel structure in the fastStart element of the Q.931 response.

When accepting a proposed channel for transmission from called endpoint to calling endpoint, the called endpoint shall return the corresponding OpenLogicalChannel structure to the calling endpoint, inserting a valid mediaControlChannel element (indicating the reverse > RTCP channel) into the H2250LogicalChannelParameters element of the > reverseLogicalChannelParameters structure. > All mediaControlChannel elements inserted by the called endpoint for > the same sessionID for both directions shall have the same value. . The called endpoint may begin transmitting media on the accepted channel according to the parameters specified in reverseLogicalChannelParameters immediately after sending the Q.931 response containing fastStart, unless mediaWaitForConnect was set to TRUE in which case it must wait until after sending the CONNECT message.

When accepting a proposed channel for transmission from the calling endpoint to the called endpoint, the called endpoint shall return the corresponding OpenLogicalChannel structure to the calling endpoint, inserting a valid mediaControlChannel element (indicating the RTCP channel going in the same direction) into the H2250LogicalChannelParameters element of the forwardLogicalChannelParameters structure. The called endpoint shall then prepare to immediately receive media flow according to the parameters specified in forwardLogicalChannelParameters. The calling endpoint may begin transmitting media on the accepted and opened channels upon receipt of the Q.931 response containing fastStart, and may release any resources allocated to reception on proposed channels that were not accepted.

[End Correction]

## 7.2	Technical and Editorial Corrections to ITU-T Recommendation H.225.0

| Description: | An error in the labeling of the Information element as described in H.225.0 This information will be contained in the revision 3 of H.225.0 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.225.0 document that was submitted for approval in 1998. This change does not affect the functionality or operation of the protocol |
| --- | --- |

[Begin Correction]

7.3.6	Information

This message may be sent to provide additional information.  It may be used to provide information for call establishment (e.g. overlap sending) or miscellaneous call-related information. It may be used to deliver proprietary features.

This message may be sent by an H.323 entity; its processing on receipt is optional.

This message follows Table 3-7/Q.931 with the following modifications:

| Information element | H.225.0 status(M/F/O) | Length in H.225.0 |
| --- | --- | --- |
| Protocol discriminator | M | 1 |
| Call reference | M | 3 |
| Message type | M | 1 |
| Sending complete | O | 1 |
| Display | O | 2-82 |
| Keypad facility | O | 2-34 |
| Signal | O | 2-3 |
| Called party number | O | 2-35 |
| User-to-User | M | 2-131 |

Table 9/H.225.0

Information Message Content

The user-to-user information element contains the Information-UUIE defined in the H.225.0 Message Syntax. The Information-UUIE includes the following:

protocolIdentifier  - set to the version of H.225 supported

callIdentifier - a globally unique call identifier set by the originating endpoint which can be used to associate RAS signaling with the modified Q.931 signaling used in H.225.0

[End Correction]

### H.225.0 Non Standard Message

| Description: | An error in the description of the requestSeqNum contained within the Non Standard message has been detected. This information will be contained in the revision 3 of H.225.0 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.225.0 document that was submitted for approval in 1998. The Non Standard message has no well defined or corresponding response message and therefore there can be no correlation between sequence numbers on incoming and outgoing messages from a particular endpoint. If H.323 applications are to utilize this message and need to be able to detect lost or duplicated messages, the implementation must supply its own sequencing information within the body of the message. The corrected text is shown below. |
| --- | --- |

[Begin Correction]

## 7.16	Non-Standard Message

The NonStandardMessage structure is as follows:

requestSeqNum - this is a monotonically increasing number unique to the sender.

...

[End Correction]

### 7.2.1	ANNEX H - H.225.0 Message Syntax (ASN.1)

| Description: | A number of errors have been detected in the ASN.1 syntax of H.225.0 and are shown in the corrected text below. This information will be contained in the revision 3 of H.225.0 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.225.0 document that was submitted for approval in 1998. The syntax has been changed to appropriately label the information elements as described in the previous correcting section. The Progress message has inappropiate extention markers. These should have been placed at the end of the currently defined message. The FastStart token as previously defined, did not include the ASN.1 syntax to enforce the presence of required fields within this token. This correction does not change any protocol that is transmitted. An error in the syntax for security tokens is present in H.225.0 version 2. The typographical error provides a circular reference, which although is not detrimental to implementations, does not provide the intended option. In order to allow H.323 implementations to utilize the generic H.235 token format in an application specific manner, the ASN.1 is changed to account for the typographical error. Note that this is a required change whether the functionality is used or not. |
| --- | --- |

[Begin Correction]

...

H323-UU-PDU ::= SEQUENCE

{

h323-message-body   CHOICE

{

setup			Setup-UUIE,

callProceeding		CallProceeding-UUIE,

connect			Connect-UUIE,

alerting		Alerting-UUIE,

userInformation	Information-UUIE,

releaseComplete	ReleaseComplete-UUIE,

facility		Facility-UUIE,

...,

progress		Progress-UUIE,

empty			NULL		-- used when a FACILITY message is sent,

-- but the Facility-UUIE is not to be invoked

-- (possible when transporting supplementary

-- services messages)

},

nonStandardData	NonStandardParameter OPTIONAL,

...,

h4501SupplementaryService	SEQUENCE OF OCTET STRING OPTIONAL,

-- each sequence of octet string is defined as one

-- H4501SupplementaryService APDU as defined in

-- Table 3/H.450.1

h245Tunneling			BOOLEAN,

-- if TRUE, tunneling of H.245 messages is enabled

h245Control			SEQUENCE OF OCTET STRING OPTIONAL,

-- each octet string may contain exactly

-- one H.245 PDU

nonStandardControl		SEQUENCE OF NonStandardParameter OPTIONAL

}

Information-UUIE	::=SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

...,

callIdentifier		CallIdentifier

}

Progress-UUIE ::= SEQUENCE

{

protocolIdentifier		ProtocolIdentifier,

destinationInfo		EndpointType,

h245Address			TransportAddress OPTIONAL,

callIdentifier			CallIdentifier,

h245SecurityMode		H245Security OPTIONAL,

tokens				SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens			SEQUENCE OF CryptoH323Token OPTIONAL,

fastStart			SEQUENCE OF OCTET STRING OPTIONAL,

...

}

FastStartToken ::= ClearToken (WITH COMPONENTS {..., timeStamp PRESENT, dhkey PRESENT, generalID PRESENT -- set to ‘alias’ –- })

EncodedFastStartToken ::= TYPE-IDENTIFIER.&Type (FastStartToken)

CryptoH323Token::= CHOICE

{

cryptoEPPwdHash SEQUENCE

{

alias		AliasAddress, -- alias of entity generating hash

timeStamp	TimeStamp, -- timestamp used in hash

token 		HASHED	{ EncodedPwdCertToken –- generalID set to ‘alias’ -- }

},

cryptoGKPwdHash  SEQUENCE

{

gatekeeperId	GatekeeperIdentifier, -- GatekeeperID of GK generating hash

timeStamp	TimeStamp, -- timestamp used in hash

token		HASHED 	{ EncodedPwdCertToken  -- generalID set to Gatekeeperid -- }

},

cryptoEPPwdEncr	ENCRYPTED

{ EncodedPwdCertToken –- generalID set to Gatekeeperid --},

cryptoGKPwdEncr	ENCRYPTED

{ EncodedPwdCertToken –- generalID set to Gatekeeperid --},

cryptoEPCert		SIGNED { EncodedPwdCertToken –- generalID set to Gatekeeperid -- },

cryptoGKCert   	SIGNED { EncodedPwdCertToken –- generalID set to alias -- },

cryptoFastStart	SIGNED { EncodedFastStartToken },

nestedcryptoToken	CryptoToken,

...

}

...

[End Correction]

## 7.3	Technical and Editorial Corrections to ITU-T Recommendation H.245

## 7.4	Technical and Editorial Corrections to ITU-T Recommendation H.235

### 7.4.1	Key Escrow usage

| Description: | A minor inconsistency has been discovered in the Recommendation H.235 Section 6.6.1. This information will be contained in the revision 2 of H.235 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.235 document that was submitted for approval in 1998. This change does not affect behavior or implementations in any way. |
| --- | --- |

[Begin Correction]

6.6.1	Key Escrow

Although not specifically required for operation, this recommendation contains provision for entities utilizing the H.235 protocol to support the facility known as trusted third party (TTP) within the signalling elements.

[End Correction]

### 7.4.1	H.235 Control Channel references

| Description: | A typographical error has been discovered in section 8 of the Recommendation H.235. This information will be contained in the revision 2 of H.235 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.235 document that was submitted for approval in 1998. This change does not affect intended behavior or implementations in any way, the uncorrected text is misleading and in error. |
| --- | --- |

[Begin Correction]

## 8.2	Unsecured H.245 Channel Operation

Alternatively, the H.245 channel may operate in an unsecured manner and the two entities open a secure logical channel with which to perform authentication and/or shared-secret derivation.  For example TLS or IPSEC may be utilized by opening a  logical channel with the datatype containing a value for  h235Control. This channel could then be used to derive a shared secret which protects any media session keys or to transport the EncryptionSync.

[End Correction]

### 7.4.1	Diffe-Hellman exchange with optional Authentication

| Description: | A minor section reference has been discovered in the Recommendation H.235 Section 9. This information will be contained in the revision 2 of H.235 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.235 document that was submitted for approval in 1998. |
| --- | --- |

[Begin Correction]

# 9.	Multipoint Procedures

## 9.1	Authentication

Authentication shall occur between an endpoint and the MC(U) in the same manner that it would in a point to point conference.   The MC(U) shall set the policy concerning level and stringency of authentication.  As stated in section 6.6 the MC(U) is trusted; existing endpoints in a conference may be limited by the authentication level employed by the MC(U).  New ConferenceRequest/ConferenceResponse commands, allow endpoints to obtain the certificates of other participants in the conference from the MC(U).  As outlined in H.245 procedures, endpoints in a multipoint conference may request other endpoint certificates via the MC, but may not be able perform direct cryptographic authentication within the H.245 channel.

...

[End Correction]

### 7.4.1	Diffe-Hellman exchange with optional Authentication

| Description: | Two errors have been discovered in the labeling of parameters of arguments in the Diffe-Hellman exchange described in the Recommendation H.235 Section 10.2. This information will be contained in the revision 2 of H.235 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.235 document that was submitted for approval in 1998. Phase 1: As this correction affects implementations, which utilize this mechanism to provide authentication during the Diffe-Hellman exchange. Note that if these optional parameters are not utilized (denoted by italics below and in the original recommendation) no implementation changes are needed. Phase 2: The identifier (generalID) passed from in the second exchange (e.g. Response) should be that of the recipient of the Response message (e.g. EPA). |
| --- | --- |

[Begin Correction]

10.2	Diffie-Hellman with optional Authentication

...

...

[End Correction]

### 7.4.2	Corrections to ANNEX A

| Description: | An omission in the ASN.1 syntax for H.235 has been discovered. Specifically, an identifier is missing from the ClearToken structure. This information will be contained in the revision 2 of Recommendation H.235 to be published by the ITU-T. However, this information appears incorrectly in the final H.235 document that was submitted for approval in 1998. The absence of this identifier will not allow multiple ClearTokens included in a single RAS message to be associated with individual uses. Additionally, ClearTokens may be defined for different uses that have the same format and these need to be differentiated by the tokenOID. |
| --- | --- |

[Begin Correction]

ClearToken		::= SEQUENCE  -- a `token' may contain multiple value types.

{

tokenOID  	OBJECT IDENTIFIER,

timeStamp		TimeStamp OPTIONAL,

password			Password OPTIONAL,

dhkey			DHset OPTIONAL,

challenge	 	ChallengeString OPTIONAL,

random			RandomVal OPTIONAL,

certificate		TypedCertificate OPTIONAL,

generalID		Identifier OPTIONAL,

nonStandard 		NonStandardParameter OPTIONAL,

...

}

[End Correction]

### 7.4.2	Corrections to ANNEX B

| Description: | A number of typographical errors have been discovered in Annex B. Their corrected values are shown below. This information will be contained in the revision 2 of Recommendation H.235 to be published by the ITU-T. However, this information appears incorrectly in the final H.235 document that was submitted for approval in 1998. |
| --- | --- |

[Begin Correction]

- 2. Signalling and Procedures

...

One purpose of H.225.0 exchanges as they relate to H.323 security, is to provide a mechanism to set up the secure H.245 channel.  Optionally, authentication may occur during the exchange of H.225.0 messages.  This authentication may be certificate or password based, utilizing encryption and/or hashing (i.e. signing).  The specifics of these modes of operation are described in sections (4.2-4.3)

...

[End Correction]

[Begin Correction]

- 4.1 Introduction

This annex will not explicitly provide any form of message privacy between gatekeepers and endpoints. There are two types of authentication that may be utilized.  The first type is symmetric encryption based that requires no prior contact between the endpoint and Gatekeeper.  The second type is subscription based and will have two forms, password or certificate.   All of these forms are derived from the procedures shown in sections [change these to document cross-references] 10.2, 10.3.2, 10.3.3 and 10.3.4. In this annex, the generic labels (EPA and EPB) showed in the aforementioned sections will represent the Endpoint and Gatekeeper respectively.

...

[End Correction]

[Begin Correction]

- 4.2 Endpoint-Gatekeeper Authentication (Non-Subscription Based)

This mechanism may provide the Gatekeeper with a cryptographic link that a particular endpoint, which previously registered, is the same one that issues subsequent RAS messages.  It should be noted that this might not provide any authentication of the Gatekeeper to the endpoint, unless the optional signature element is included.  The establishment of the identity relationship occurs when the terminal issues the GRQ as outlined in H.323 section [change to cross-reference ] 7.2.1.   The Diffie-Hellman exchange shall occur in conjunction with the GRQ and GCF messages as shown in the first phase of section .  This shared secret key shall now be used on any subsequent RRQ/URQ from the terminal to the gatekeeper.  If a Gatekeeper operates in this mode and receives a GRQ without a token containing the DHset or an acceptable algorithm value, it shall return a securityDenial reason code in the DRJ.

...

[End Correction]

[Begin Correction]

- 5.1 Gateway

As stated in section [change to cross reference] 6.6, an H.323 Gateway should be considered a trusted element.  This includes protocol gateways (H.323-H.320 etc.…) and security gateways (proxy/firewalls).  The media privacy can be assured between the communicating endpoint and the gateway device; but what occurs on the far side of the gateway should be considered insecure by default.

[End Correction]

[Begin Correction]

- 4.2 Endpoint-Gatekeeper Authentication (Non-Subscription Based)

...

Terminal (xRQ):

- The terminal shall provide all of the information in the message as described in the appropriate H.225.0 sections.

- The terminal shall encrypt the GatekeeperIdentifier (as returned in the GCF) using the shared secret key that was negotiated. This shall be passed in a clearToken (see section 10.2) as the generalID.

The 16 bits of the random and then the requestSeqNum shall be XOR’d with each 16 bits of  the GatekeeperIdentifier.  If the GatekeeperIdentifier does not end on an even 16 boundary, the last 8 bits of the GatekeeperIdentifier shall be XOR’d with the least significant octet of the random value and then requestSeqNum.  The GatekeeperIdentifier shall be encrypted using the selected algorithm in the GCF (algorithmOID)  and utilizing the entire shared secret.

...

[End Correction]

### 7.4.3	Corrections to Appendix I

| Description: | A typographical error has been discovered with respect to a section reference of encryption key generation. This information will be contained in the revision 2 of H.235 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.235 document that was submitted for approval in 1998. The referenced section was incorrectly labeled to a nonexistent section heading. |
| --- | --- |

[Begin Correction]

4.2 Password

...

- The encryption key is constructed from the user’s password using the procedure described in section 10.3.2 of H.235.  The resulting  octet “string” is then used as the DES key to encrypt the challenge.

...

[End Correction]

## Technical and Editorial Corrections to ITU-T Recommendation H.45X

### 7.3.1	H.450.3 ASN.1 Correction

| Description: | A typographical error has been discovered in the ASN.1 definitions presented in H.450.3, Chapter 11. This information will be contained in the revision 2 of H.450.3 Recommendation to be published by the ITU-T. However, this information appears incorrectly in the final H.450.3 document that was submitted for approval in 1998. The corrected, included element is shown below. |
| --- | --- |

[Begin Correction]

H225InformationElement FROM H225-generic-parameters-definition

...

[End Correction]

# 8.	Implementation Clarifications

## Token Usage in H.323 systems

There has been some confusion on the usage of individual CryptoH323Tokens as passed in RAS messages.  There are two main categories of CryptoH323Tokens; those used for H.235 procedures and those used in an application specific manner. The use of these tokens should be according to the following rules:

- All H.235 defined (e.g. cryptoEPPwdHash, cryptoGKPwdHash, cryptoEPPwdEncr, cryptoGKPwdEncr, cryptoGKCert, and cryptoFastStart). shall be utilized with the procedures and algorithms as described in H.235.

- Application specific or proprietary use of tokens shall utilize the nestedcryptoToken for their exchanges.

- Any nestedcryptoToken used should have an tokenOID (object identifier) which unambigously identifies it.

## 8.2	H.235 Random Value  Usage in H.323 systems

The random value that is passed in xRQ/xCF sequence between endpoints and Gatekeepers may be updated by the Gatekeeper.  As described in section 4.2 of H.235 this random value may be refreshed in any xCF message to be utilized by a subsequent xRQ messages from the endpoint.  Due to the fact that RAS messages may be lost (including xCF/xRJ) the updated random value may also be lost.  The recovery from this situation may be the reinitalizing of the security context but is left to local implemtaion.

Implementations that require the use of multiple outstanding RAS requests will be limited by the updating of the random values used in any authentication.  If the updating of this value occurs on every response to a request, parallel requests are not possible.  One possible solution, is to have a logical ‘window’ during which a random value remains constant.  This issue is a local implementation matter.

H.323 Recommendation Series Defect Report Form

| DATE: |  |
| --- | --- |
| CONTACT INFORMATION NAME: COMPANY: ADDRESS: TEL: FAX: E-MAIL: |  |
| AFFECTED RECOMMENDATIONS: |  |
| DESCRIPTION OF PROBLEM: |  |
| SUGGESTIONS FOR RESOLUTION: |  |

Note – Attach additional pages if more space is required than is provided above