ITU - Telecommunication Standardization Sector

STUDY GROUP 16					Original: English

Question:	Q.13 / 16					September 20, 1997

SOURCE: 	M. Korpi / Siemens AG

Tel.: +49 89 722 34570

Fax.: +49 89 722 23977

Email: korpim@sbs.de

TITLE: 		Draft new Recommendation H.450.2 (determined)

Call Transfer Supplementary Service for H.323

PURPOSE:

This new Recommendation H.450.2 specifies the Call Transfer supplementary service (SS-CT), and is applicable to various basic services supported by H.323 Multimedia Terminals. SS-CT is based on the equivalent supplementary service for Private Integrated Services Networks (PISN) specified in ISO/IEC 13865 and 13869.

History of changes:

Changes after March 1997 SG16 meeting in Geneva:

After determination of draft H.450.2 (TD-61 PLEN) in the March Geneva SG16 meeting, the following modifications have been applied to the document.

- Clause 3: Definitions added

- Clause 4: List of Acronyms added

- Clause 6: NFE and Interpretation APDU setting for various Call Transfer operations added

- Clause 10.3.3: Interaction with H.323 Ad Hoc Conference added

- Clause 10.3.4: Gatekeeper actions added

- Clause 11.3, 11.4, 11.5 and 11.6 and 13: Primitives aligned between clauses 11.x and 13.

- Clause 11.3 to 11.6: Primitive parameters added.

- Clause 12: Automatic tagging applied to ASN.1 modules

- Clause 12: ASN.1 module reference changed

- Clause 12: ASN.1 extensibility: Usage of the syntactic flag (ellipsis) for extensibility (acc. to X.680 Amendment 1 “Rules for extendsibility”)

- Clause 12: CTSetupArg: transferringNumber added

- Clause 12: AliasAddress changed to SEQUENCE OF EndpointAddress where applicable

Changes after Q.12-14 Rapporteurs meeting in Hertzliya June 10-13, 1997 (APC-1177):

- Clause 3: Definition of “secondary call” modified (result of the discussion on APC-1205 item 4.2 Hertzliya / PictureTel)

- Clause 10.5 Further description of call transfer interaction with Ad-hoc Conference added

- Clause 10.6.1, 11.1.1.2, 11.4, 12 and 13: Gatekeeper procedures added - gatekeeper may perform call transfer for the transferred endpoint (APC-1222 and TD-35 Hertzliya / Lucent)

- Clause 12/ASN.1: New operation callTransferComplete added for informing the transferred endpoint about the call transfer in case transfer is performed by the gatekeeper

- Clause 13:  Note added, that in case of conflicts between the text and SDLs, the text shall take precedence

- Clause 13.5: Clearing primitive for secondary call by transferred-to endpoint added (APC-1205 sect 4.2.2)

Changes after Q.12-14 Rapporteurs meeting in Sunriver September 8-11, 1997 (APC-1253):

- Clause 5, 6, 10.6.1, 11 and 12: Scenarios added for transfer with secondary call existing with gatekeeper performing call transfer using transfer by join methods (APC-1292 from Mitel, APC-1297 PictureTel)

- Clause 12: Type BMP-String added to call transfer operations CTUpdateArg, CTActiveArg and CTCompleteArg in order to enable signalling of textual information (e.g. Name information) in addition to the number (to be inline with the corresponding modification agreed for H.450.3 in Hertzliya)

- Clause 12/ASN.1: Redefinition of the Extension construct in order to shorten notation.

- Type Extension in H.450.1 is a parameterized type. Extension is therefore to be parameterized in H.450.2 as well.

- Clause 12/ASN.1:  As an alternative to the ExtensionSeq, type NonStandardParameter as defined in H.225.0  may be used for extending H.450.2 supplementary service APDUs with proprietary information.

- Clause 10.3: Note added regarding the two procedures used in the ISO QSIG Call Transfer standard and how these procedures apply to H.450.2 (APC-1297, PictureTel).

- Clause 5: Usage of H.450.2 for transferring of T.120 calls is for further study.

- Clause 10.6.1, 10.6.2 and Figures 6, 15, 16, 17 and 18/H.450.2 modified according to TD-27v2 Sunriver (usage of empty terminal capability set messages for closing logical channels in case of gatekeeper performing call transfer); source BT

- Editorial changes.

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	DRAFT H.450.2

TELECOMMUNICATION	(Sept 20, 1997)
STANDARDIZATION  SECTOR	
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

Call Transfer Supplementary Service for H.323

DRAFT  ITU-T  Recommendation  H.450.2

FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union.  The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.450.2 was prepared by the ITU-T Study Group 16 (1997-2000) and was approved by the WTSC (Place, Month xx-xx, 199x).

___________________

  ITU  199x

All rights reserved.  No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Contents

SUMMARY

1 Scope

2 References

3 Terms and definitions

4 Abbreviations and Acromyms

5 Description

6 Messages and Information elements

7 Actions at the transferring endpoint (user A)

7.1 Normal procedures if only the primary call exists

7.2 Normal procedures if also the secondary call exists

7.3 Exceptional procedures

8 Actions at the transferred endpoint (user B)

8.1 Normal procedures

8.2 Exceptional procedures

9 Actions at the transferred-to endpoint (user C)

9.1 Normal procedures

9.2 Exceptional procedures

10 Additional procedures

10.1 Interworking with non-H.323 terminals

10.2 Hold and consultation

10.3 Transfer without rerouting (i.e. transfer by join)

10.4 Further actions at Transferred and Transferred-To endpoint

10.5 Interactions with H.323 Ad Hoc Conference

10.6 Gatekeeper actions

11 Dynamic Description of Transfer with Rerouting

11.1 Transfer with Rerouting without secondary call

11.2 Transfer with Secondary call

11.3 Communication between Transferring signalling entity TRGSE and TRGSE user

11.4 Communication between Transferred signalling entity TRDSE and TRDSE user

11.5 Communication between Transferred-To signalling entity TRTSE and TRTSE user

11.6 Peer-to-peer communication for Transfer with Rerouting

12 Operations in support of SS-CT

13 Specification and Description Language (SDL) Diagrams

13.1 Call Transfer Block diagram

13.2 Call transfer Substructure Diagram

13.3 Transferring Signalling Entity SDLs

13.4 Transferred Signalling Entity SDLs

13.5 Transferred-To Signalling Entity SDLs

# SUMMARY

Recommendation H.450.2 describes the procedures and the signalling protocol for the Call Transfer supplementary service (SS-CT) in H.323 (Packet Based Multimedia Communications Systems) networks.

SS-CT is a supplementary service which enables the served user A to transform an existing call (user A - user B) into a new call between user B and a user C selected by user A. User A may or may not have a call established with user C prior to Call Transfer.

This Recommendation makes use of the "Generic functional protocol for the support of supplementary services in H.323" as defined in Recommendation H.450.1

The procedures and the signalling protocol of this Recommendation are derived from the Call Transfer supplementary service specified in ISO/IEC 13865 and 13869.

Call Transfer Supplementary Services for H.323

(Place, 199x)

1	Scope

This Recommendation describes the Call Transfer supplementary service (SS-CT), which is applicable to various basic services supported by H.323 Multimedia Endpoints. SS-CT is based on the equivalent supplementary service for Private Integrated Services Networks (PISN) specified in ISO/IEC 13865 and 13869.

# 2	References

The following ITU-T Recommendations, and other references contain provisions which, through reference in this text, constitute provisions of the Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

ISO/IEC 13865 Information technology - Telecommunications and information exchange between systems - Private Integrated Services Network  Specification, functional model and information flows - Call Transfer Supplementary Service.

ISO/IEC 13869 Information technology - Telecommunications and information exchange between systems - Private Integrated Services Network - Inter-exchange signalling protocol - Call Transfer supplementary service.

ITU-T Rec. I.112,: Vocabulary of terms for ISDNs

ITU-T Rec. I.210,: Principles of Telecommunication Services Supported by an ISDN and the Means to Describe Them.

ITU-T Recommendation H.323, Packet based Multimedia Communications Systems.

ITU-T Recommendation H.225.0, Media Stream Packetization and Synchronization on Non-Guaranteed Quality of Service LANs

ITU-T Recommendation H.245, Control Protocol for Multimedia Communication

ITU-T Recommendation H.450.1, Generic functional protocol for the support of supplementary services in H.323

# 3	Terms and definitions

For the purposes of this Recommendation, the following definitions apply.

Endpoint; terminal; user

see Rec. H.323

Transferring endpoint; User A

User/Endpoint transferring a call with a terminal B, to a user C.

Transferred endpoint; User B

The remote endpoint / user of the call with user A which is transferred by A to a user C.

Transferred-to endpoint; User C

The endpoint / user, to which a user B, originally having a call to user A, is transferred to.

Primary call

Call between the transferring user A and the transferred user B.

Secondary call

Call between the transferring user A and the transferred-to user C associated with the Primary call (this association is an implementation matter within endpoint A)

Transferred call

Call between transferred user B and transferred-to user C which results from successful completion of call transfer.

# 4	Abbreviations and Acromyms

For the purpose of this Recommendation the following abbreviations are used.

| ASN.1 | Abstract Syntax Notation No. 1 |
| --- | --- |
| APDU | Application Protocol Data Unit |
| CT | Call Transfer |
| GK / GKp / GKs | Gatekeeper / Gatekeeper on primary call / Gatekeeper on secondary call |
| MC | Multipoint Controller |
| PISN | Private Integrated Services Network |
| SCN | Switched Circuit Network |
| SDL | Specification and Description Language |
| SS-CT | Supplementary Service Call Transfer |
| TRDSE | Transferred Signalling Entity |
| TRGSE | Transferring Signalling Entity |
| TRTSE | Transferred-to Signalling Entity |

# 5	Description

Call Transfer (SS-CT) is a supplementary service which enables the served user (user A or transferring user) to transform an existing call with user B (primary call) into a new call between user B and a user C (transferred-to user) selected by user A. User A may or may not have a call established with user C (secondary call) prior to transfer. Each call can either be an incoming call to user A or an outgoing call from user A. On successful completion of SS-CT user B and user C can communicate with each other and user A will no longer be able to communicate with user B or user C.

The initial call between user A and user B (primary call) must be answered before transfer can be initiated. On invocation of call transfer, if a call between user A and user C exists (secondary call), the transferred-to endpoint is informed of the pending call transfer, and transfer only proceeds if this endpoint is able to participate. In this case the transferred-to endpoint returns a temporary identifier to be used in the subsequent transfer procedure to identify the secondary call.

The transferring endpoint (user A) requests the transferred endpoint (user B) to call the transferred-to endpoint (user C). The transferred endpoint then establishes a call to the transferred-to endpoint (transfer by rerouting) and includes the temporary identifier for the secondary call if this call exists. User B’s Gatekeeper may establish the call if it is handling the Call Transfer.

The primary call is retained until the first acknowledgment has been received from the transferred-to endpoint (user C), and is then released. This means that the primary call remains in place if call transfer fails before that stage. If the secondary call exists it is retained until the new call request arrives at the transferred-to endpoint and is then released.

Upon answer from user C (if no secondary call exists) or successful association of user C with the new call (if user C was already involved in the secondary call) user B and user C can communicate with each other. The media exchange capabilities of the new call should be equal to those of the primary call as far as possible.

If call transfer fails after the primary call was released the action taken is for further study.

Note: Use of Call Transfer Supplementary service for transferring only a subset of H.323 media streams or for  transferring T.120 calls is for further study.

# 6	Messages and Information elements

The APDUs of call transfer operations are transported within User to User information elements in call control and FACILITY messages as defined in the ITU-T Recommendation H.450.1. The call transfer operations are described in clause 12.

When conveying the invoke APDU of the operations defined in clause 12, the destinationEntity data element of the NFE shall contain the value “endpoint”.

When conveying the invoke APDU of operations callTransferAbandon, callTransferUpdate, callTransferComplete, callTransferActive and subaddressTransfer, the Interpretation APDU shall contain value discardAnyUnrecognizedInvokePdu.

When conveying the invoke APDU of operation callTransferSetup, the Interpretation APDU shall contain value clearCallIfAnyInvokePduNotRecognized.

When conveying the invoke APDU of operation callTransferIdentify or callTransferInitiate, the Interpretation APDU shall be omitted or shall contain value rejectAnyUnrecognizedInvokePdu.

# 7	Actions at the transferring endpoint (user A)

In the procedures below, 'clearing' or 'release' of a call means the release of all resources associated with the call, as specified in Rec. H.323 (i.e all logical channels, H.245 control channel, call signalling channel, call reference etc.).

## 7.1	Normal procedures if only the primary call exists

In order to initiate call transfer, the transferring endpoint shall send a callTransferInitiate invoke APDU in a FACILITY message to the transferred endpoint, using the call reference of the primary call, start timer CT-T3 and enter state CT-Await-Initiate-Response. The invoke APDU shall contain element rerouteingNumber with the address of user C as selected by user A. Element callIdentity shall be empty.

On receipt of a callTransferInitiate return result APDU on the primary call the transferring endpoint shall stop timer CT-T3, clear the primary call and enter state CT-Idle.

If the primary call is released while in state CT-Await-Initiate-Response, the transferring endpoint shall stop timer CT-T3 and enter state CT-Idle.

## 7.2	Normal procedures if also the secondary call exists

In order to initiate call transfer, the transferring endpoint shall send a callTransferIdentify invoke APDU in a FACILITY message to the transferred-to endpoint, using the call reference of the secondary call, start timer CT-T1 and enter state CT-Await-Identify-Response.

On receipt of a callTransferIdentify return result APDU on the secondary call the transferring endpoint shall stop timer CT-T1, send a callTransferInitiate invoke APDU in a FACILITY message to the transferred endpoint, using the call reference of the primary call, start timer CT-T3 and enter state CT-Await-Initiate-Response. The invoke APDU shall contain elements rerouteingNumber and callIdentity with the values received in callTransferIdentify return result.

On receipt of a callTransferInitiate return result APDU on the primary call the transferring endpoint shall stop timer CT-T3, clear the primary and secondary call if not already cleared, and enter state CT-Idle.

If the primary and/or secondary call is released while in state CT-Await-Identify-Response or CT-Await-Initiate-Response, the transferring endpoint shall stop the relevant timer, as an option clear the other call if not already cleared, and enter state CT-Idle.

## 7.3	Exceptional procedures

On receipt of a callTransferIdentify reject or return error APDU while in state CT-Await-Identify-Response the transferring endpoint shall stop timer CT-T1 and enter state CT-Idle.

On receipt of a callTransferInitiate reject or return error APDU while in state CT-Await-Initiate-Response the transferring endpoint shall send a callTransferAbandon invoke APDU in a FACILITY message on the secondary call if this call exists, stop timer CT-T3 and enter state CT-Idle.

On expiry of timer CT-T1 or CT-T3 the transferring endpoint shall send a callTransferAbandon invoke APDU in a FACILITY message on the secondary call if this call exists, and enter state CT-Idle.

# 8	Actions at the transferred endpoint (user B)

## 8.1	Normal procedures

On receipt of a callTransferInitiate invoke APDU on the primary call the transferred endpoint shall determine whether it can participate in the call transfer. If it can it shall use the address contained in element rerouteingNumber to initiate call establishment towards the transferred-to endpoint, include in the SETUP message a callTransferSetup invoke APDU with the same value of callIdentity as received in the callTransferInitiate invoke APDU, and enter state CT-Await-Setup-Response. The callTransferSetup invoke APDU may also include the address of user A in element transferringNumber.  Optionally timer CT-T4 may be started.

On receipt of an ALERTING or CONNECT message with a callTransferSetup return result APDU from the transferred-to endpoint the transferred endpoint shall stop timer CT-T4 if running, send a RELEASE COMPLETE message with a callTransferInitiate return result APDU on the primary call to the transferring endpoint and enter state CT-Idle. Further connection establishment between user B and user C shall follow standard H.323 procedures, whereby  media properties may be inherited from the primary call or automatically negotiated between endpoints B and C.

## 8.2	Exceptional procedures

If on receipt of a callTransferInitiate invoke APDU the transferred endpoint cannot participate in call transfer a callTransferInitiate return error APDU shall be sent in a FACILITY message on the primary call.

On expiry of timer CT-T4, or on receipt of a clearing indication on the new call while in state CT-Await-Setup-Response, the transferred endpoint shall send a callTransferInitiate return error APDU in a FACILITY message on the primary call, stop timer CT-T4 if running, and enter state CT-Idle. If a callTransferSetup return error APDU was received from the transferred-to endpoint the same error value shall be used in the callTransferInitiate return error APDU.

If the primary call is cleared while in state CT-Await-Setup-Response the transferred endpoint shall stop timer CT-T4 if running, clear the new call and enter state CT-Idle.

# 9	Actions at the transferred-to endpoint (user C)

## 9.1	Normal procedures

On receipt of a callTransferIdentify invoke APDU on the secondary call the transferred-to endpoint shall determine whether it can participate in the call transfer. If it can it shall send a callTransferIdentify return result APDU in a FACILITY message on the secondary call, start timer CT-T2 and enter state CT-Await-Setup. The return result APDU shall contain user C’s address in element rerouteingNumber and a local identifier for the secondary call in element callIdentity.

On receipt of a SETUP message with a callTransferSetup invoke APDU from the transferred endpoint the transferred-to endpoint shall, if in state CT-Await-Setup, stop timer CT-T2, check the destination address information of the SETUP message and element callIdentity received in the callTransferSetup invoke APDU, and continue with call establishment if they correctly identify the secondary call. The secondary call shall be cleared using normal H.323 procedures.

If the callTransferSetup invoke APDU is received in state CT-Idle, call setup shall continue if element callIdentity is empty and if call transfer to user C is acceptable.

Either the ALERTING or the CONNECT message on the new call shall include a callTransferSetup return result APDU.

## 9.2	Exceptional procedures

If on receipt of a callTransferIdentify invoke APDU the transferred-to endpoint cannot participate in call transfer a callTransferIdentify return error APDU shall be sent in a FACILITY message on the secondary call.

If on receipt of a callTransferSetup invoke APDU with an empty element callIdentity the transferred-to endpoint cannot accept call transfer to user C the endpoint shall return a RELEASE COMPLETE message with a callTransferSetup return error APDU.

If on receipt of a callTransferSetup invoke APDU with a non-empty element callIdentity the transferred-to endpoint cannot match the identifier with a secondary call the endpoint shall return a RELEASE COMPLETE message with a callTransferSetup return error APDU.

If in state CT-Await-Setup a callTransferAbandon invoke APDU is received on the secondary call, or if the secondary call is cleared, the transferred-to endpoint shall stop timer CT-T2 and enter state CT-Idle.

On expiry of timer CT-T2 the transferred-to endpoint shall enter state CT-Idle.

# 10	Additional procedures

## 10.1	Interworking with non-H.323 terminals

In the specification of procedures above the term “endpoint” means any entity that processes H.225 call signalling Examples  are “terminal” or “gateway”.

If user B or user C are non-H.323 terminals the gateway on the route to user B or user C will act as transferred endpoint or transferred-to endpoint, respectively.

## 10.2	Hold and consultation

Call transfer procedures do not demand a particular hold state for the primary or secondary call, i.e. user B or user C may or may not be put on hold prior to call transfer, depending on the capabilities of terminal A and the specific implementation.

With existing (e.g. PISN) implementations, call transfer is frequently performed by user A putting user B on hold, setting up the secondary call to user C (“consultation call”) and then invoking transfer. This method is also shown in an example in subclause 11.6.

## 10.3	Transfer without rerouting (i.e. transfer by join)

for the .6.1).

ISO/IEC 13869 defines two procedures for call transfer, 1) transfer by re-routing and 2) transfer by join. In H.450.2, where the transferring endpoint typically is the terminal rather than a private integrated network exchange, the transfer by re-routing procedure was chosen as the primary procedure (it is not expected from a H.323 terminal to join two calls in which it no longer participants).

However, the transfer by join procedure may be used in H.450.2 if a gatekeeper performs the transfer (e.g. if the terminals do not support H.450.2 call transfer by re-routing) or in case of gateway interworking with the corresponding SCN transfer by join procedure (see also subclauses 10.6.1 and 11.2.2.2).

## 10.4	Further actions at Transferred and Transferred-To endpoint

Together with the callTransferSetup invoke APDU a callTransferUpdate invoke APDU may be sent in the SETUP message from the transferred endpoint to the transferred-to endpoint. Within the argument, optional elements redirectionNumber and basicCallInfoElements may be conveyed. If a callTransferUpdate invoke APDU is received by the transferred-to endpoint, this information may be conveyed to the transferred-to user.

Together with the callTransferSetup return result APDU a callTransferUpdate invoke APDU may be sent in the ALERT or CONNECT message from the transferred-to endpoint to the transferred endpoint. Within the argument, optional elements redirectionNumber and basicCallInfoElements may be conveyed. If a callTransferUpdate invoke APDU is received by the transferred endpoint, this information may be conveyed to the transferred user.

If during CT-Idle, a FACILITY message containing a subaddressTransfer invoke APDU is received, the endpoint may relay the subaddress on to the local user.

## 10.5	Interactions with H.323 Ad Hoc Conference

Call transfer may be used in the context of conference.

An H.323 endpoint A (that does not have the Active MC) may transfer its connection to the conference to another endpoint D. For this purpose, the endpoint A shall send a FACILITY message containing a callTransferInitiate invoke APDU and conferenceGoal = “invite” towards the conference entity B containing the Active MC (may be the GK or another endpoint). The entity B may then issue a new SETUP message to endpoint D, containing a callTransferSetup invoke APDU and conferenceGoal = “invite”.  After successful establishment of the H.225 channel between entity B (Active MC) and the new endpoint D, the call between entity B and endpoint A (transferring endpoint) is released following normal call transfer procedures. Establishment of the H.245 channel and of logical channels between entity B and endpoint D follows subclause 8.4.3 of H.323.

An H.323 endpoint A that contains the active MC may transfer an endpoint B to an endpoint C, if the call between the endpoints A and B is a two-party conference call at the time of call transfer request.

An H.323 endpoint that has the active MC shall reject a call transfer request, if the active MC serves a multipoint conference at the time of call transfer request.

## 10.6	Gatekeeper actions

In the case of the gatekeeper routed model, the gatekeeper shall either transparently transport (see also FIGURE 4/H.450.2) or act on the operations defined in clause 12 (see subclause 10.6.1).

### 10.6.1	Call Transfer performed by Gatekeeper

A gatekeeper may decide to become the destination for a callTransferInitiate invoke APDU. In this case, the gatekeeper shall perform the actions applicable for the transferred endpoint with the additions described in this subclause.6/H.450.2).

A gatekeeper may decide to become the destination for a callTransferIdentify invoke APDU. In this case, the gatekeeper shall perform the actions applicable for the transferred-to endpoint with the additions described in this subclause.

A gatekeeper may be able to act as the destination for both sides of the call transfer, in which case it performs actions appropriate for both transferred and transferred-to endpoints.

When a gatekeeper is handling the call transfer signalling on behalf of an endpoint, the gatekeeper may choose to relay call transfer APDUs (callTransferComplete, callTransferActive, callTransferUpdate) to the endpoint.

#### 10.6.1.1 	Gatekeeper Actions on the Primary Call

The callTransferInitiate invoke APDU sent from the transferring endpoint A to endpoint B may be intercepted by the GK, i.e. the callTransferInitiate invoke APDU does not reach the transferred endpoint B. The GK shall perform further procedures as defined for a transferred endpoint in clause 8. This includes the sending of a SETUP message with a callTransferSetup invoke APDU and an optional callTransferUpdate invoke APDU to the transferred-to endpoint C, the reception of the ALERTING or CONNECT message with a callTransferSetup return result APDU and optionally a callTransferUpdate invoke APDU as well as the sending of a RELEASE COMPLETE message on the primary call to endpoint A including a callTransferInitiate return result APDU.

endpoint C exchanges terminal capabilities with the gatekeeper/MC (the gatecapabilitIn addition to the call transfer procedures defined in clause 8, the gatekeepershall may send a FACILITYFacility message with a callTransferCTComplete invoke APDU to endpoint B and, where a secondary call is present, to endpoint C to inform endpoint Bendpoint B them that it has it has they have been transferred (“joining”).Argument redirectionNumber shall contain the number of the transferred-to user C.  The argument endDesignation shall have the value “primaryEnd”, argument callStatus shall be set to “answered” (if available). The CTComplete invoke apdu shall be sent latest after the GK has passed on the OpenLogicalChannel message from endpoint B to endpoint C. The callTransferCTComplete invoke APDU shall be sent upon reception of an ALERTING or a CONNECT message from the transferred-to endpointterminal C.  When sent to endpoint B, aArgument redirectionNumber shall contain the number of the transferred-to user C argument endDesignation shall have the value “primaryEnd”, argument callStatus shall be set to either “answered” (if  sent in response to a CONNECT) or “alerting” (if sent in response to an ALERTING). When sent to endpoint C, argument redirectionNumber shall contain the number of the transferred user B, argument endDesignation shall have the value “secondaryEnd”, and argument callStatus shall be set to “answered”.

If the transferred-to user initially responded to the SETUP with an ALERTING me, a FACILITY message cont callTransferCTActive invokePDUapdu may be sent to endpoint B ued-to terminal.

(or its gatekeeper) exchanges terminal capabilities with the gatper/MC (the gatekeeperl represent the ointThe gatekeeper performs ster-slave determination with endpoint C. If the gatekeeper is not acting as an MC, then the terminalType field shall represent endpoint B.

terminal capability set (one which indicates that the remote entity has not receive capabilities) to endpoints A & B, causing A & B to close their logical channels.  When endpoint B receives a non-empty terminal capability set from the gatekeeper it shall reset its H.245 associated state to that which it is in when H.245 has just completed terminal capability set exchange in the initial call establishment sequence.  Endpoint B shall take part in master/slave determination and open appropriate logical channels.  send a RequestChannelClose with reason “reopen” to endpoint B. The RequestChannelClose shall stimulate endpoint B to close its logical channels  ndpoint B send OpenLogicalChannel to endpoint Cndpoint C responds with an OpenLogicalChannelAck that contains the address where endpoint C will accept the media stream. Endpoint C opens channels

#### 10.6.1.2 Gatekeeper Actions on the Secondary Call

The callTransferIdentify invoke APDU sent from the transferring endpoint A destined to endpoint C may be intercepted by a GK, i.e. the callTransferIdentifyCTIdentityDU does not reach the transferred-to endpoint C. The gatekeeper shall perform further procedures as defined for a transferred-to endpoint in clause 9. This includes sending ALERTING and CONNECT messages (as appropriate) in response to the SETUP with callTransferSetup invoke APDU received from endpoint B to reflect the state of the secondary call. In addition, the gatekeeper shall perform procedures specified in this subclause.

The gatekeeper must ensure that it is able to receive and handle call signalling from the transferred-to endpoint (or its gatekeeper)routed sign

If media channels associated with the secondary call are already open at the time sending the callTransferSetup return result APDU (typically in case of transfer into active but may also apply to transfer into alerting), they  need to be closed. Therefore, tthe gatekeeper shall instruct endpoint C to close its media channels to endpoint A usingthe H.323 procedures for third party re-routing.  This involves the gatekeeper sending an empty terminal capability set (one which indicates that the remote entity has not receive capabilities) to endpoints A & C causing them to close their logical channels.   by sending a RequestChannelClose with reason “h450service” to endpoint C. Once the media E COMPLETE message to release he call signalling channel.

If no media channels associated with the secondary call are open at the time sending the callTransferSetup return result APDU (may be true e.g. in case of transfer into alerting) ALERTING message to endpoint B, the gatekeeper shall exchange t A (if H.245 channel was established prior between endpoints A and B) and send a RELEASE COMPLETE to endpoint A to release the call signalling channel..  The CONNECT message received from endpoint C shall be sent towards the transferredndpoint B.

, a FACILITY message containing callTransferComplete invoke APDU may be recemiIn addition to the call transfer procedures defined in clause 9, the gatekeeper may send a FACILITY message with a callTransferCompleteCTComplete invoke APDU to endpoint C to inform it that it has been transferred (“joining”). The callTransferCompleteCTComplete invoke APDU shall be sent to the transferred-to endpoint C at the same time as the first of either ALERTING or CONNECT message containing callTransferSetup return result APDU is sent to the transferred endpoint B. Argument redirectionNumber shall contain the number of the transferred user B, argument endDesignation shall have the value “secondaryEnd”, and argument callStatus shall be set to  “answered”.

If terminal capabilities had been exchanged with the transferring endpoint prior to the invocation of call transfer, when H.245 control signalling is established with the transferred terminal (terminal B or its gatekeeper) the gatekeeper shall send a pabilitySet representing endpoint C’s capabilities.

# 11	Dynamic Description of Transfer with Rerouting

The following subclauses show examples of call transfer scenarios by describing the operational model and corresponding signalling flows.

The following notation is used:

N11.1	Transfer with Rerouting without secondary call

FIGURE 1/H.450.2

Model for Transfer with Re-routing without secondary call

FIGURE 2/H.450.2

Signalling flow for Transfer with Re-routing without secondary call

### 11.1.2	Operational model for Transfer using gatekeeper routed model

#### 11.1.2.1	Gatekeeper is transparent for call transfer operations

FIGURE 3/H.450.2

Operational model for transfer using gatekeeper routed model -

GK is transparent for call transfer operations

FIGURE 4/H.450.2

Signalling flow for transfer using gatekeeper routed model -

GK is transparent for call transfer operations

11.1.2.2	Call transfer supplementary service performed by gatekeeper:

FIGURE 5/H.450.2

Operational model for transfer using gatekeeper routed model -

GK intercepts call transfer APDUs

FIGURE 6/H.450.2

Signalling flow for transfer using gatekeeper routed model -

GK intercepts call transfer

## 11.2	Transfer with Secondary call

### 11.2.1 Operational Model for Transfer if no gatekeeper is involved in the service

FIGURE 7/H.450.2

Operational model for transfer with secondary call

FIGURE 8/H.450.2

Signalling Flow  for Transfer with Secondary call

The r

### 11.2.2 Operational Model for Transfer using gatekeeper routed model

It is possible to have a mixed model where one gatekeeper involved in the transfer handles the call transfer for its endpoint (as described in subclause 11.2.2.2) while another gatekeeper involved in the same transfer allows transparent operation (subclause 11.2.2.1).

#### 11.2.2.1 Gatekeeper is transparent for Call Transfer Operations

FIGURE 9/H.450.2 and FIGURE 10/H.450.2 show signalling paths where each end of both the primary and secondary calls is gatekeeper routed. Where an end of a call uses direct signalling, that end reduces to the model shown in subclause 11.2.1 without a gatekeeper.

FIGURE 9/H.450.2

Operational Model for Call Transfer with Consultation - GK is transparent for call transfer operations

FIGURE 10/H.450.2

Signalling Flow for Call Transfer with Consultation - Gatekeeper is transparent for call transfer operations

11.2.2.2 Call Transfer Supplementary Service performed by Gatekeeper

This subclause gives examples on call models for call transfer with secondary call with one or more gatekeepers performing the call transfer supplementary service.  In  FIGURE 11/H.450.2, FIGURE 12/H.450.2 and FIGURE 13/H.450.2, there may be one ore more gatekeepers on the call signalling path. The call transfer signalling may be handled by any gatekeeper on the path. For that reason, only the gatekeeper handling the call transfer is shown (as GKp on the primary call, and GKs on the secondary call). Other gatekeeers on the call signalling path which are not shown are transparent for call transfer APDUs.

Note that in FIGURE 12/H.450.2 and FIGURE 13/H.450.2 appropriate signalling to establish gatekeeper routing at the transferred-to endpoint (user C) is required.

For some selected call models, signalling flows are given in subsequent figures below.

FIGURE 11/H.450.2

Call Transfer with Consultation - Primary Call Routed through Gatekeeper

FIGURE 12/H.450.2

Call Transfer with Consultation - Secondary Call Routed through Gatekeeper

FIGURE 13/H.450.2

Call Transfer with Consultation - Primary and Secondary Calls Routed through Different Gatekeepers

FIGURE 14/H.450.2

Call Transfer with Consultation - Primary and Secondary Calls Routed through Common Gatekeeper

FIGURE 15/H.450.2

Call Transfer with Consultation - Distinct Gatekeeper: Secondary Call Connected

(based on operational model given in Figure 13)

FIGURE 16/H.450.2

Call Transfer with Consultation - Distinct Gatekeeper: Secondary Call Alerting

(based on operational model given in Figure 13)

FIGURE 17/H.450.2

Call Transfer with Consultation - Common Gatekeeper: Secondary Call Connected

(based on operational model given in Figure 14)

FIGURE 18/H.450.2

Call Transfer with Consultation - Common Gatekeeper: Secondary Call Alerting

(based on operational model given in Figure 14)

## 11.3	Communication between Transferring signalling entity TRGSE and TRGSE user

### 11.3.1	Table of primitives

Communication between the TRGSE and TRGSE user, is performed using the primitives shown in Table 1/H.450.2.

TABLE 1/H.450.2

TRGSE Primitives

| generic name | Type |  |  |  |
| --- | --- | --- | --- | --- |
|  | request | indication | response | confirm |
| CTInitiate | PARAMETERS | not defined 1 | not defined | - 2 |
| CTIdentify 3 | - | not defined | not defined | PARAMETERS |
| CTAbandon | not defined | - | not defined | not defined |

1	“not defined” means that this primitive is not defined	
2	“-“ means no parameters

3  Only applicable in case of Transfer with secondary call

### 11.3.2	Primitive definition

The definition of these primitives is as follows

- The CTInitiate.request primitive is used to make the transfer request

- The CTInitiate.confirm primitive is used to indicate the result of the transfer request

- The CTIdentify.request primitive is used to request a call identity from TRTSE

- The CTIdentify.confirm primitive is used to confirm the call identity of TRTSE

- The CTAbandon.indication primitive is used to indicate transfer unsuccessful notification to the TRTSE

### 11.3.3	Parameter definition

CTInitiate.req parameters

- callIdentity: call identifier of secondary call

- reroutingNumber: address of user C

- extension: Placeholder for vendor-specific extensions

- CTIdentify.conf  parameters

- callIdentity: call identifier of secondary call

- reroutingNumber: address of user C

- extension: Placeholder for vendor-specific extensions

### 11.3.4	States

The following states are used to specify the allowed sequence of primitives between the TRGSE and TRGSE user

CT-Idle

Supplementary service Call Transfer is not operating

CT-Await-Identify-Response

A callTransferIdentify invoke APDU has been sent to the TRTSE. This state is used during transfer by rerouting with secondary call.

CT-Await-Initiate-Response

A callTransferInitiate invoke APDU has been sent to the TRDSE. This state is used during transfer by rerouting.

## 11.4	Communication between Transferred signalling entity TRDSE and TRDSE user

### 11.4.1	Table of primitives

Communication between the TRDSE and TRDSE user, is performed using the primitives shown in Table 2/H.450.2.

TABLE 2/H.450.2

TRDSE Primitives

| generic name | Type |  |  |  |
| --- | --- | --- | --- | --- |
|  | request | indication | response | confirm |
| CTInitiate | not defined 1 | PARAMETERS | -2 | not defined |
| CTSetup | PARAMETERS | not defined | not defined | - |
| SubaddressTransfer | PARAMETERS | PARAMETERS | not defined | not defined |
| CTUpdate | PARAMETERS | PARAMETERS | not defined | not defined |
| CTComplete | PARAMETERS 3 | PARAMETERS 4 | not defined | not defined |
| CTActive | PARAMETERS 3 | PARAMETERS 4 | not defined | not defined |

1	“not defined” means that this primitive is not defined	
2	“-“ means no parameters

3	Applicable only for a gatekeeper, if it performs the call transfer  (see subclause 10.6.1)

4	Applicable for the transferred endpoint, if a gatekeeper performs the call transfer procedures

### 11.4.2	Primitive definition

The definition of these primitives is as follows

- The CTInitiate.indication primitive is used to notify about transfer iniate request

- The CTInitiate.response primitive is used to indicate success of the transfer

- The CTSetup.request primitive is used to request call establishment from TRTSE

- The CTSetup.confirm primitive is used to indicate success of call establishment to TRTSE

- The CTUpdate.request primitive is used to request sending of identification information to the TRTSE

- The CTUpdate.indication primitive is used to notify TRDSE User about TRTSE identification

- The SubaddressTransfer.request primitive is used to request transmission of TRDSE subaddress to the TRTSE

- The SubaddressTransfer.indication primitive is used to notify TRTSE subaddress to the TRDSE user

- The CTComplete.request primitive is used to request sending of call transfer information from the TRDSE located in gatekeeper to the TRDSE located in the transferred terminal endpoint (i.e. gatekeeper performs call transfer for the transferred endpoint).

- The CTComplete.indication primitive is used to indicate call transfer information from the TRDSE to the TRDSE user (i.e. gatekeeper has performed call transfer for the transferred endpoint)

- The CTActive.request primitive is used to request sending of call transfer active information from TRDSE located in a gatekeeper to the TRDSE located in the transferred terminal endpoint (i.e. gatekeeper performs transfer  ringing for the transferred endpoint with secondary call existing)

- The CTActive.indication primitive is used to indicate to the transferred user that the TRTSE user has become active (only if gatekeeper performs call transfer)

### 11.4.3	Parameter definition

CTInitiate.ind parameters

- callIdentity: call identifier of secondary call

- reroutingNumber: address of user C

- extension: Placeholder for vendor-specific extensions

- CTSetup.req parameters

- callIdentity: call identifier of secondary call

- transferringNumber: address of user A (optional)

- extension: Placeholder for vendor-specific extensions

- SubaddressTransfer.xxx parameters

- redirectionSubaddress: Party Subaddress

- extension: Placeholder for vendor-specific extensions

- CTUpdate.xxx parameters

- redirectionNumber: address of remote party (B or C)

- redirectionInfo: name information of remote party (B or C)

- : H.225 Information element (optional)

- extension: Placeholder for vendor-specific extensions

- CTComplete.xxx parameters

- redirectionNumber: address of new connected party (party C)

- basicCallInfoElements: H.225 Information element (optional)

- redirectionInfo: name information of new connected party (party C)

- callStatus: reflects the H.225 call state of the new connected party

- extension: Placeholder for vendor-specific extensions

CTActive.xxx parameters

- connectedAddress: address of user C

- basicCallInfoElements: H.225 Information element (optional)

- connectedInfo: name information of user C (optional)

- extension: placeholder

### 11.4.4	States

The following states are used to specify the allowed sequence of primitives between the TRDSE and TRDSE user

CT-Idle

Supplementary service Call Transfer is not operating.

CT-Await-Setup-Response

A callTransferSetup invoke APDU has been sent to the TRTSE. This state is used during transfer by rerouting.

CT-Await-Connect

The Primary Call has been transferred to an alerting TRTSE, and the TRDSE has been notified. A CONNECT message indicating answering by the TRTSE is awaited.

## 11.5	Communication between Transferred-To signalling entity TRTSE and TRTSE user

### 11.5.1	Table of primitives

Communication between the TRTSE and TRTSE user, is performed using the primitives shown in Table 3/H.450.2.

TABLE 3/H.450.2

TRTSE Primitives

| generic name | Type |  |  |  |
| --- | --- | --- | --- | --- |
|  | request | indication | response | confirm |
| CTSetup | not defined 1 | PARAMETERS | - 2 | not defined |
| CTUpdate | PARAMETERS | PARAMETERS | not defined | not defined |
| CTAbandon | not defined | - | not defined | not defined |
| CTIdentify 3 | not defined | - | PARAMETERS | not defined |
| SubaddressTransfer | PARAMETERS | PARAMETERS | not defined | not defined |
| CTComplete | PARAMETERS4 | PARAMETERS5 | not defined | not defined |

1	“not defined” means that this primitive is not defined

2  “-“ means no parameters

3  Only applicable in case of Transfer with secondary call

if a GKs actsed-to endpoint; only usGKs (if available

### 11.5.2	Primitive definition

The definition of these primitives is as follows

- The CTSetup.indication primitive is used to notify about setup request

- The CTSetup.response primitive is used to indicate acceptance of the call

- The CTUpdate.request primitive is used to request update of identification information

- The CTUpdate.indication primitive is used to notify about updated identification information

- The CTIdentify.indication primitive is used to request a call identification

- The CTIdentify.response primitive is used to give response to identify request

- The CTAbandon.indication primitive is used to notify about unsuccessful transfer attempt

- The SubaddressTransfer.request primitive is used to request transmission of subaddress to the TRDSE

- The SubaddressTransfer.indication primitive is used to notify TRDSE subaddress to the TRTSE user

- The CTComplete.request primitive may be used by a GKs to request sending of call transfer information to the  transferred-to user.

- The CTcomplete.indication primitive is used to indicate call transfer information to the transferred-to endpoint (may erred-to e

### 11.5.3	Parameter definition

CTSetup.ind parameters

- callIdentity: identifier of secondary call

- transferringNumber: address of user A (optional)

- extension: Placeholder for vendor-specific extensions

- SubaddressTransfer.xxx parameters

- redirectionSubaddress: Party Subaddress

- extension: Placeholder for vendor-specific extensions

- CTUpdate.xxx parameters

- redirectionNumber: address of party C

- redirectionInfo: name information of party B or C

- basicCallInfoElements: H.225 Information element (optional)

- extension: Placeholder for vendor-specific extensions

- CTComplete.xxx parameters

- redirectionNumber: address of new connected party (party B)

- basicCallInfoElements: H.225 Information element (optional)

- redirectionInfo: name information of new connected party (party B)

- callStatus: reflects the H.225 call state of the new connected party

- extension: Placeholder for vendor-specific extensions

### 11.5.4	States

The following states are used to specify the allowed sequence of primitives between the TRTSE and TRTSE user

CT-Idle

Supplementary service Call Transfer is not operating

CT-Await-Setup

A callTransferIdentify return result APDU has been sent to the TRGSE. This state is used during transfer by rerouting.

## 11.6	Peer-to-peer communication for Transfer with Rerouting

### 11.6.1	Messages

The FACILITY message may be used to request or acknowledge a supplementary service. For more information see ITU-T Recommendation H.450.1.

The SETUP message is sent by a calling entity to indicate its desire to set up a connection to a called entity. For more information see ITU-T Recommendation H.225.0 subclause 7.3.11.

The CONNECT message is sent by the called entity to the calling entity to indicate acceptance of the call by the called entity. For more information see ITU-T Recommendation H.225.0  subclause 7.3.3.

The RELEASE COMPLETE message is sent by a terminal to indicate release of the call if the reliable call signalling channel is open. For more information see ITU-T Recommendation H.225.0  subclause 7.3.10.

### 11.6.2	Timers

CT-T1 - Timer CT-T1 shall operate at the TRGSE during state CT-Await-Identify-Response. Its purpose is to protect against the absence of a response to the CTIdentify.request.

CT-T2 - Timer CT-T2 shall operate at the TRTSE during state CT-Await-Setup. Its purpose is to protect against failure of completion of the call transfer operation.

CT-T3 - Timer CT-T3 shall operate at the TRGSE during state CT-Await-Initiate-Response. Its purpose is to protect against failure to establish the new connection.

CT-T4 - Timer CT-T4 may optionally operate at the TRDSE during state CT-Await-Setup-Response. Its purpose is to protect against failure to establish the new connection.

Editors Note: Default value ranges for timers are for further study.

### 11.6.3	Counters

No counter variables are used for the call transfer supplementary service.

### 11.6.4	Message Flow Examples

#### 11.6.4.1	Transfer with rerouting (without consultation)

Description from user point of view:

User A (transferring party):

communicating with B;selects C; Request B to connect to C (Media inherited); transfer accepted; idle

User B (transferred party):

communicating with A; receives request of transfer; confirms media; communicating with C;

User C (transferred-to party):

idle; receives notification of incoming call; accepts call; confirms media; communicating with B

TABLE 4/H.450.2

Transfer with rerouting (without consultation)

| Row No | User/ Application action |  | a) Primitive b) State c) Timer |  | Note | H.225, H.323, H.245, H.450 | Note | a) Primitive b) State c) Timer | User/ Application action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MM Terminal A |  |  |  | Network |  |  | MM Terminal B/Gateway |  |
| 2 | Active Basic Call between TE A and TE B Capabilities exchanged H.225 connection still exists UDP path for audio open |  |  |  |  |  |  |  |  |
| 3 | Request terminal B to transfer call to C enter state: CT-Await-Initiate-Response |  | a) Transfer-ring/TRGSE <CTInitiate.req> b)CT-Await_Ini-tiate-Response c) CT-T3 |  |  | H.225 <FACILITY> H4501SupplementaryService APDU: invoke callTransferInitiate rerouteingNumber=address C callIdentity (=empty) |  | a) Transferred TRDSE a)<CTInitiate.ind> b) CT-Idle c) None | Receive transfer_ request |
| 4 | MM Terminal B / Gateway |  |  |  |  | Network |  | MM Terminal C |  |
| 5 | establish TCP path for H.225 call signalling |  |  |  |  |  |  |  |  |
| 6 | Request for call estab-lishment to User C Select Media User C address |  | a) Transferred TRDSE <CTSetup.req> <CTUpdate.req> b)CT-Await_Setup_Response c) T4 (optional) |  |  | H.225 <SETUP> H4501SupplementaryService APDU: invoke - callTransferSetup - callTransferUpdate - redirectionNumber |  | a) Transferred to TRTSE <CTSetup.ind> <CTUpdate.ind_ind> b) CT-Idle c) None | Receive H.225 setup.indi-cation |
| 7 | Indication to Appl/User B of MM Terminal C general availability |  | a) Transferred TRDSE <CTSetup.conf> <CTUpdate.ind> b) CT-Idle c) None |  |  | H.225 <CONNECT> H4501SupplementaryService APDU: returnResult - callTransferSetup invoke - callTransferUpdate - redirectionNumber |  | a) Transferred to TRTSE <CTSetup.resp> <CTUpdate.req> b) CT-Idle c) None | Call accept by User C or by Application C |
| 8 | MM Terminal A |  |  |  |  | Network |  | MM Terminal B / Gateway |  |
| 9 | close logical channels |  |  |  |  |  |  |  |  |
| 10 | Receive H.225 release.indication | a) Transferring TRGSE <CTInitiate_ conf> b) CT-Idle c) None |  |  |  | H.225 <RELEASE COMPLETE> H4501SupplementaryService APDU: returnResult - callTransferInitiate |  | a) Transferred TRDSE <CTInitiate_ resp> b) CT-Idle c) None | release call to TE A |
| 11 | MM Terminal B / Gateway |  |  |  |  | Network |  | MM Terminal C |  |
| 12 | exchange terminal capabilities open logical channels (H.245) |  |  |  |  |  |  |  |  |

#### 11.6.4.2	Failing case of transfer with rerouting (without Secondary call)

Scenario Description from user point of view:

User A (transferring party):

communicating with B;selects C; Request B to connect to C (Media inherited); unsuccessful transfer; communicating with B

User B (transferred party):

communicating with A;receives request for transfer; unsuccessful transfer; communicating with A;

User C (transferred to party):

idle; unsuccessful call establishment from B; idle

TABLE 5/H.450.2

Failing case of transfer with rerouting (without Secondary call)

| Row No | User/ Application action | a)Primitive b) State c) Timer | Note | H.225, H.323, H.245, H.450 | Note | a) Primitive b) State c) Timer | User/ Application action |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | MM Terminal A |  | Network |  |  | MM Terminal B/Gateway |  |
| 14 | Active Basic Call between TE A and TE B Capabilities exchanged H.225 connection still exists UDP path for audio open |  |  |  |  |  |  |
| 15 | Request terminal B to transfer call to C enter state: CT-Await- Initiate - Response | a) Transferring TRGSE <CT-Initiate- req> b)CT-Await-Ini-tiate-Response c) CT-T3 |  | H.225 <FACILITY> H4501SupplementaryService APDU: invoke - callTransferInitiate - rerouteingNumber=address C - callIdentity (=empty) |  | a) Transferred TRDSE <CTInitiate.ind> b) CT-Idle c) None | Receive transfer_ request |
| 16 | MM Terminal B / Gateway |  |  | Network |  | MM Terminal C |  |
| 17 | Request for callestablishment to TE C | Establishment of basic call fails |  |  |  |  |  |
| 18 | MM Terminal A |  |  | Network |  | MM Terminal B/Gateway |  |
| 19 |  | a) Transfer ring TRGSE <CTInitiate_ resp_rej> b) CT-idle c) None |  | H.225 <FACILITY> H4501SupplementaryService APDU: returnError - callTransferInitiate |  | a) Transferred TRDSE <CTInitiate.conf_rej> b) CT-idle c) None |  |
| 20 | A communicating with B |  |  |  |  |  |  |

#### 11.6.4.3	Transfer with secondary call (consultation)

Description from user point of view:

User A (transferring party):

communicating with B; B on Hold (optional); User selects C and Media; requests connection to C; Consultation Active; Request B to connect to C; Transfer accepted; idle

User B(transferred party):

communicating with A; receives request for transfer; confirms media; communicating 
 with C

User C (transferred-to party)

idle; receives notification of incoming call; accepts call; confirms media; communicating with A; receives notification of incoming call; accepts call; confirms media; communicating with B

TABLE 6/H.450.2

Transfer with secondary call (consultation)

| Row No | User/ Application action | a) Primitive b) State c) Timer | Note | H.225, H.323, H.245, H.450 | Note | a) Primitive b) State c) Timer | User/ Application action |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | MM Terminal A |  | Network |  |  | MM Terminal B |  |
| 22 | Active Basic Call between TE A and TE B Capabilities exchanged H.225 connection still exists UDP path for audio open |  |  |  |  |  |  |
| 23 | MM Terminal A |  |  | Network |  | MM Terminal C |  |
| 24 | Establishment of Basic Call |  |  |  |  |  |  |
| 25 | Consultation active |  |  |  |  |  |  |
| 26 | Identification of Te C | a) Transferring TRGSE <CTIdentify.req> b)CT-Await_ Identify-Res-ponse c) CT-T1 |  | H.225 <FACILITY> H4501SupplementaryService APDU: invoke - callTransferIdentify | 1 | a) Transferred-to TRTSE <CTIdentify.ind> b) CT-Idle c) None |  |
| 27 | Receive Identification | a) Transferring TRGSE <CTIdentify.conf> b) CT-Idle c) None |  | H.225 <FACILITY> H4501SupplementaryService APDU: returnResult - callTransferIdentify - callIdentity - rerouteingNumber=address C | 2 | a) Transferred-to TRTSE <CTIdentify.resp> b) CT-Await- Setup c) CT-T2 | determine ability of CT enter state: CT-Await-Setup |
| 28 | MM Terminal A |  |  | Network |  | MM Terminal B / Gateway |  |
| 29 | transfer with rerouting (TABLE 4/H.450.2 with non-empty callIdentity) |  |  |  |  |  |  |
| 30 | MM Terminal A |  |  | Network |  | MM Terminal C |  |
| 31 | close logical channels |  |  |  |  |  |  |
| 32 | Receive H.225 release.indication | a) Transferring TRGSE <Release.ind> b) CT-Idle c) None |  | H.225 <RELEASE COMPLETE> |  | a) Transferred-to TRTSE <Release.req> - CT-Idle - None |  |

1	optional
2	optional

# 12	Operations in support of SS-CT

The following operations defined in Abstract Syntax Notation number 1 shall apply:

Call-Transfer-Operations

{itu-t recommendation h 450 2 version1(0)  call-transfer-operations(0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS	OPERATION, ERROR FROM Remote-Operations-Information-Objects

{joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

EXTENSION, Extension{} FROM

Manufacturer-specific-service-extension-definition
{itu-t recommendation h 450 1 version1(0) msi-definition(18)}

notAvailable, supplementaryServiceInteractionNotAllowed,

invalidCallState FROM H4501-General-Error-List

{itu-t recommendation h 450 1 version1(0) general-error-list(1)}

EndpointAddress, PartySubaddress FROM Addressing-Data-Elements 
		{itu-t recommendation h 450 1 version1(0)

addressing-data-elements(9)}

NonStandardParameter FROM H323-MESSAGES  -- see H.225.0

H225InformationElement FROM H225-generic-parameters-definition 
		{itu-t recommendation h 450 1 version1(0)

h225-generic-parameters(6)};

H323CallTransferOperations  OPERATION  ::=

{	callTransferIdentify | callTransferAbandon | callTransferInitiate | callTransferSetup | callTransferUpdate | subaddressTransfer | callTransferComplete | callTransferActive }

callTransferIdentify		OPERATION		::=

{-- sent from transferring endpoint to transferred-to endpoint

ARGUMENT	DummyArg	OPTIONAL TRUE

RESULT	CTIdentifyRes

ERRORS	{	notAvailable | invalidCallState | supplementaryServiceInteractionNotAllowed | unspecified }

CODE	local:7

}

callTransferAbandon	OPERATION	::=

{		-- sent from transferring to transferred-to endpoint

ARGUMENT	DummyArg	OPTIONAL  TRUE

RETURN RESULT  FALSE

ALWAYS RESPONDS  FALSE

CODE	local:8

}

callTransferInitiate		OPERATION	::=

{	-- sent from transfering to transferred endpoint

ARGUMENT	CTInitiateArg

RESULT	DummyRes	OPTIONAL  TRUE

ERRORS	{	notAvailable | invalidCallState | invalidReroutingNumber | unrecognizedCallIdentity | establishmentFailure | supplementaryServiceInteractionNotAllowed | unspecified }

CODE	local:9

}

callTransferSetup	OPERATION	::=

{		-- sent from transferred to transferred-to endpoint

ARGUMENT	CTSetupArg

RESULT	DummyRes	OPTIONAL  TRUE

ERRORS 	{	notAvailable | invalidCallState | invalidReroutingNumber | unrecognizedCallIdentity | unspecified | supplementaryServiceInteractionNotAllowed  }

CODE	local:10

}

callTransferUpdate	OPERATION	::=

{-- may be sent from transferred to  transferred-to endpoint (in SETUP) and

-- vice versa (in CONNECT)

ARGUMENT	CTUpdateArg

RETURN RESULT  FALSE

ALWAYS RESPONDS  FALSE

CODE	local:13

}

subaddressTransfer	OPERATION	::=

{-- may be sent from transferred to the transferred-to endpoint and vice versa

ARGUMENT	SubaddressTransferArg

RETURN RESULT  FALSE

ALWAYS RESPONDS  FALSE

CODE	local:14

}

callTransferComplete	OPERATION  ::=

{ -- sent from GK to the transferred endpoint, if GK performs call transfer

-- (see subclause 10.6.1).

ARGUMENT	CTCompleteArg

RETURN RESULT  	FALSE

ALWAYS RESPONDS  	FALSE

CODE	local:12

}

callTransferActive	OPERATION  ::=

{ -- sent from gatekeeper to the transferred endpoint, in case of transfer ringing

-- after transferred-to endpoint has answered the call (only in case of

-- transfer performed by gatekeeper)

ARGUMENT	CTActiveArg

RETURN RESULT  	FALSE

ALWAYS RESPONDS  	FALSE

CODE	local:11

}

DummyArg	::=	CHOICE

{	extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }

DummyRes	::=	CHOICE

{	extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }

CTInitiateArg	::=	SEQUENCE

{

callIdentity	CallIdentity,

reroutingNumber	EndpointAddress,

argumentExtension	CHOICE

{extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }    OPTIONAL,

…

}

CTSetupArg	::=	SEQUENCE

{

callIdentity	CallIdentity,

transferringNumber	EndpointAddress  OPTIONAL,

argumentExtension	CHOICE

{extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }       OPTIONAL,

…

}

CTIdentifyRes	::=	SEQUENCE

{

callIdentity	CallIdentity,

reroutingNumber	EndpointAddress,

resultExtension	CHOICE

{extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }       OPTIONAL,

…

}

CTUpdateArg	::= 	SEQUENCE

{

redirectionNumber	EndpointAddress,

redirectionInfo	BMPString (SIZE(1..128)) OPTIONAL,

basicCallInfoElements H225InformationElement OPTIONAL,

-- Information element

-- Progress indicator may be conveyed

argumentExtension	CHOICE

{extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }  OPTIONAL,

…

}

SubaddressTransferArg	  ::=	SEQUENCE

{

redirectionSubaddress	PartySubaddress,

argumentExtension	CHOICE

{extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }  OPTIONAL,

…

}

CTCompleteArg	::= SEQUENCE

{

endDesignation		EndDesignation,

redirectionNumber		EndpointAddress,

-- number of new connected user

basicCallInfoElements 	H225InformationElement OPTIONAL,

-- Information element Progress indicator may be conveyed

redirectionInfo		BMPString (SIZE(1..128)) OPTIONAL,

callStatus		CallStatus DEFAULT answered,

argumentExtension		CHOICE

{extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }  OPTIONAL,

…

}

EndDesignation	::= ENUMERATED

{

primaryEnd(0),

secondaryEnd(1),

…

}

CallStatus	::= ENUMERATED

{

answered(0),

alerting(1),

…

}

CTActiveArg	::= SEQUENCE

{

connectedAddress		EndpointAddress,

--number of new connected user

basicCallInfoElements 	H225InformationElement OPTIONAL,

-- Information element Progress indicator may be conveyed

connectedInfo		BMPString (SIZE(1..128)) OPTIONAL,

argumentExtension	CHOICE

{extensionSeq	ExtensionSeq,

nonStandardData	NonStandardParameter }  OPTIONAL,	…

}

CallIdentity	::=	NumericString (SIZE(0..4))

-- size 0 means 'empty'

ExtensionSeq 	::=  SEQUENCE OF Extension{{ExtensionSet}}

ExtensionSet	EXTENSION	::= {...}

-- Actual values defined by individual manufacturers

unspecified	ERROR	::=

{

PARAMETER 	CHOICE

{extension	Extension{{ExtensionSet}},

nonStandard	NonStandardParameter

}

CODE	local:1008

}

invalidReroutingNumber	ERROR	::=

{	-- used when establishment of the new connection fails

-- because the reroutingNumber is not a valid alias address

CODE	local:1004

}

unrecognizedCallIdentity	ERROR 	::=

{	-- used when establishment of the new connection fails because it

-- could not be associated with a SS-CT entity at the

-- transferred-to endpoint

CODE	local:1005

}

establishmentFailure		ERROR	::=

{	-- used when establishment of the new connection fails and

-- no other error applies

CODE	local:1006

}

END	-- of Call-Transfer-Operations

13	Specification and Description Language (SDL) Diagrams

The procedures for call transfer signalling entities are described in SDL form in FIGURE 21/H.450.2 to FIGURE 29/H.450.2.

Note 1: In case of conflict with text the text shall take precedence.

Note 2: The SDLs only show call transfer messages transported on H.225.0 reliable connection.  H.245 procedures (e.g. terminal capability exchange, master / slave determination, opening and closing of logical channels, etc.) are not shown).

Note 3: Specific gatekeeper SDLs for call transfer using call Transfer by join as given in subclause 10.6.1 are not provided.

Note 4 : Within this clause, “ct”  is used as an abbreviation for the term “callTransfer” as part of call transfer operation names  (Example: ctInitiate.Invoke refers to callTransferInitiate invoke APDU).

## 13.1	Call Transfer Block diagram

Figure 19/H.450.2 shows the Block of supplementary service Call Transfer and its interfaces to other blocks.  The Substructure of this Block is given in figure 201H.450.2.

FIGURE 19/H.450.2

Call Transfer Block Diagram

## 13.2	Call transfer Substructure Diagram

The Substructure SS_Call_Transfer consists of the processes TRGSE, TRDSE, and TSTSE.  The interface signals to these blocks are shown in the FIGURE 20/H.450.2.

FIGURE 20/H.450.2

Call Transfer Substructure Diagram

## 13.3	Transferring Signalling Entity SDLs

FIGURE 21/H.450.2

Transferring Signalling Entity (sheet 1 of 3)

FIGURE 22/H.450.2

Transferring Signalling Entity (sheet 2 of 3)

FIGURE 23/H.450.2

Transferring Signalling Entity (sheet 3 of 3)

## 13.4	Transferred Signalling Entity SDLs

FIGURE 24/H.450.2

Primary Signalling Entity (sheet 1 of 3)

FIGURE 25/H.450.2

Primary Signalling Entity (sheet 2 of 3)

FIGURE 26/H.450.2

Primary Signalling Entity (sheet 3 of 3)

## 13.5	Transferred-To Signalling Entity SDLs

FIGURE 27/H.450.2

Secondary Signalling Entity (sheet 1 of 3)

FIGURE 28/H.450.2

Secondary Signalling Entity (sheet 2 of 3)

FIGURE 29/H.450.2

Secondary Signalling Entity (sheet 3 of 3)