ITU Telecommunication Standardization Sector Document APC-1481 Study Group 16 Version 1.0 Q.12-14/16 Rapporteur Meeting June 1998 Torino, 17-20 November 1998 SOURCE*: OzEmail Interline TITLE : Back-End Service Transaction Protocol (Technical Rationale) PURPOSE: Discussion *Contact: Douglas Clowes, dclowes@ozemail.com.au www.interline.aust.com

|  | OzEmail Interline Pty. Limited A.C.N. 076 666 316 |  |
| --- | --- | --- |

Back-End Service Transaction Protocol
(Technical Rationale)

| Version: | V1.0.0 |
| --- | --- |
| Date: | 24th June, 1998 |
| Author: | Douglas F. Clowes |

WARNING:

This document contains information that is proprietary to OzEmail Interline Pty Limited and is the subject of exclusive rights granted pursuant to the laws of copyright.

No part of this publication may be reproduced in any form whatsoever without prior written approval by OzEmail Interline Pty Limited.

OzEmail Interline Pty Limited reserves the right to revise this publication and make changes without obligation to notify any person of such revisions or changes.

# Table of Contents

Table of Contents	3

1	Scope	4

2	References	5

3	Definitions, Symbols and Abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Summary	8

5	Implementation Targets	9

5.1	The Devices	9

5.1.1	The Small Devices	9

5.1.2	The Large Devices	10

5.2	The Developers	10

6	The Transport	11

6.1	Unreliable Transport (UDP)	11

6.2	Reliable Transport (TCP)	11

6.3	The Case for Both	11

7	Abstract Syntax Notation One (ASN.1)	13

8	Packed Encoding Rules (PER)	14

9	Security	15

History	16

# Scope

This document provides the technical rationale behind various aspects of the Back-End Service Transaction (BEST) Protocol.

The document describes the implementation targets, on which the protocol will be deployed. It lists the more significant and viable options available. It omits the more trivial options, which are simply inappropriate, or were rejected early in the selection process. The document attempts to show the decisions that were taken, and the reasons for those decisions. The document does not attempt to justify the contents of the protocol data units.

# References

ITU-T Recommendation H.225.0 (1998), Call Signaling Protocols and Media Stream Packetization for Packet Based Multimedia Communications Systems.

ITU-T Recommendation H.235 (1998), Security and Encryption for H Series (H.323 and other H.245 based) Multimedia Terminals.

ITU-T Recommendation H.245 (1998), Control Protocol for Multimedia Communications.

ITU-T Recommendation H.323 (1998), Packet Based Multimedia Communications Systems.

ITU-T Recommendation X.680 (1994), Information Technology – Abstract Syntax Notation One (ASN.1) Specification of Basic Notation.

ITU-T Recommendation X.691 (1994), Information Technology – ASN.1 Encoding Rules – Specification of Packed Encoding Rules (PER).

ITU-T Recommendation E.164 (1991), Numbering Plan for the ISDN Era.

IETF RFC-768, User Datagram Protocol.

ETF RFC-793, Transmission Control Protocol.

IETF RFC-1825, Security Architecture for the Internet Protocol.

IETF RFC-1826, IP Authentication Header.

IETF RFC-1827, IP Encapsulating Security Payload (ESP).

ISO 4217:1995, Codes for the Representation of Currencies and Funds.

# Definitions, Symbols and Abbreviations

## Definitions

For the purposes of the current document, the following definitions apply:

access control: The process of preventing unauthorised use of a resource, or use of a resource in an unauthorised way.

accounting: The process of recording usage information, in such a way that billing and settlement may later be performed.

authentication: The process of proving identity within context.

authorisation: The process of granting permission based on identity.

address translation: The process of translating a given address into another. In this context, it is usually an H.323 address into an endpoint address with appropriate dialling information.

back-end server: A computer system providing services under this protocol. Not part of the H.323 network.

back-end service: A service which is provided by a back-end server. Not part of the services provided under H.323.

back-end service provider: The operator of one or more back-end servers.

billing: The process of generating a statement of charging information, for payment of account.

caller: The person making the call.

certificate: An unforgeable set of security-related data which can be used to provide identification and authentication services.

confidentiality: The property that prevents unauthorised disclosure of information.

endpoint: An H.323 endpoint. An H.323 terminal, gateway, or MCU. An endpoint can call and be called. This includes an IVR server in the context of this document.

gatekeeper: An H.323 gatekeeper.

gateway: An H.323 gateway.

integrity: The property that prevents unauthorised alteration of information.

Interactive Voice Response: An Interactive Voice Response system, operating as an H.323 endpoint, which participates in providing back-end services.

nonrepudiation: The property that provides that neither party can deny having participated in all or part of a communication.

originating endpoint: The H.323 endpoint which originates the call within the H.323 network. This may not be the ultimate origin of the call.

privacy: A property that provides for only the communicating parties to interpret information.

settlement: The process of resolving amounts owed and owing to produce resultant billing.

telephony service provider: A provider of telephony services.

terminating endpoint: The H.323 endpoint which terminates the call within the H.323 network. This may not be the ultimate destination of the call.

user: The entity to which a call is to be billed. This may be the caller, or some other party.

## Symbols

For the purposes of the current document, the following symbols apply:

<None at present>

## Abbreviations

For the purpose of the current document, the following abbreviations apply:

AH	Authentication Header (RFC-1826)

ASN.1	Abstract Syntax Notation One (ITU-T X.680)

BES	Back-End Server

BESP	Back-End Service Provider

BEST	Back-End Service Transaction

CLID	Calling Line Identification

DH	Diffie-Hellman

DNIS	Dialled Number Identification Service

DSA	Digital Signature Algorithm

DSS	Digital Signature Standard

EP	H.323 Endpoint

ESP	Encapsulating Security Payload (RFC-1827)

GK	H.323 Gatekeeper

GRC	H.323 Gatekeeper Routed Call

GW	H.323 Gateway

IETF	Internet Engineering Task Force

IPSEC	IP Security (RFC-1825)

ISP	Internet Service Provider

ITSP	Internet Telephony Service Provider

ITU	International Telecommunication Union

ITU-T	International Telecommunication Standardisation Sector of the ITU

IVR	Interactive Voice Response

MCU	H.323 Multipoint Control Unit

PER	Packed Encoding Rules (ITU-T X.691)

PIN	Personal Identification Number

SCN	Switched Circuit Network

SHA	Secure Hash Algorithm

SHS	Secure Hash Standard

SSL	Secure Socket Layer

TCP	Transmission Control Protocol (RFC-793)

TLS	Transport Layer Security

UDP	User Datagram Protocol (RFC-768)

UTC	Coordinated Universal Time

VoIP	Voice over IP

# Summary

The BEST protocol implements basic functionality for back-end services for an H.323 network. The protocol provides for authentication, authorisation, address translation, call processing, pricing exchange and usage recording. It operates between H.323 entities, such as a gateway or gatekeeper, and one or more back-end servers. It allows flexibility and growth through vendor specific extensions.

The BEST protocol capitalises on the benefits of existing protocols in the H.323 domain. By sharing characteristics of the existing protocols, many benefits derive from reuse of the tools, techniques and experience of engineering personnel in the field. This lowers the learning curve for H.323 developers and shortens time to market. Significant code reuse may be achieved, offering further savings in development time as well as reductions in the program memory footprint required in production devices.

Like H.323, the BEST protocol does not include the network interface, the physical network or the transport protocol used on the network. By using the same mechanisms in H.323, it can therefore, be used anywhere that H.323 devices are deployed. Because of their widespread familiarity and extensive use in the VoIP domain, IP related terms are used for examples and descriptions. This is not to be taken as implying that this protocol is limited to IP networks.

The BEST protocol is a compact protocol, offering low data memory requirements for message staging and transmission buffering. It also imposes low demands for bandwidth on network links, without the added overhead of expensive compression and decompression processes.

By using an unreliable transmission channel, the BEST protocol provides an efficient and flexible mechanism to support the required functionality. Fallback to a reliable transmission channel backs that up with reliable alternatives during periods of degraded network reliability, and provides a mechanism for effective bulk transfers.

# Implementation Targets

This section describes the implementation targets that have been considered; this includes both the devices, and the people who develop them. An objective of the protocol is to minimise the impact on the implementation targets. Many of the design decisions taken in developing this protocol, are predicated on the presumptions made in this section.

The protocol design aims to minimise the impact on the devices and developers, and capitalise on the substantial investment in people, tools and software.

## The Devices

The target implementation platforms for this protocol will be:

- H.323 Endpoints (EP), which may be:

- Terminals which may be:

- Personal Computers, or

- Single Use Devices;

- Gateways (GW),

- Multipoint Control Units (MCU)

- H.323 Gatekeepers (GK),

- Back-End Servers (BES), and

- any other devices that may want to participate.

The characteristics and numbers of these devices, and the ratios of their deployment, will vary greatly. Devices may be small, of limited memory and limited capability, or they may range to huge arrays of interconnected computer systems.

It is recognised that the tendency is that as the size of a device increases, the number of other devices with which that device interacts will also, typically, increase. It is also recognised that the commercial reality is that the size, cost and complexity of each device ought to be minimised while achieving its design goals.

Recognising that the scale of devices is so broad, it behoves us design a protocol that will not only scale from the smallest to the largest, but will have:

- the smallest impact on the smallest devices, which are of limited capacity, and

- the smallest impact on the largest devices, which will have the most interactions.

The number of large devices is expected to be small, compared to the number of small devices. While the number of interactions between large devices may be large, the number of channels connecting large device to large devices will be relatively small and may be the subject of special consideration.

Little consideration can be given to the “other devices,” not knowing what they are. It is to be expected that the same considerations given to the known devices, whether large, small, or somewhere in between, will likely satisfy them too.

### The Small Devices

The smallest devices that are expected to use the BEST protocol are “Single Use Devices” and low capacity gateways, perhaps with as few as one PSTN line. Personal Computers, while not large in the H.323 sense, are expected to have many megabytes of RAM for program and data storage, have high-speed processors, and are not considered as small devices.

Small devices will have limited memory. Programs will probably be stored in ROM of some type, with a minimum amount of RAM for the H.323 purposes. The addition of the BEST protocol to these devices should seek to minimise the amount of both types of memory used on these devices.

Small devices will have limited processing power. While these devices may have special hardware for signal processing, manufacturers will seek to minimise the processing power required for other purposes, such as H.323 protocol and network processing, and so keep costs down. The addition of the BEST protocol to these devices should seek to minimise the amount of processing used on these devices.

It is therefore advantageous to develop a compact protocol, which can reuse code fragments that can be found on a typical small device.

### The Large Devices

Devices in an H.323 and BES network are likely to increase in size from the largest currently available. It is envisaged that large devices will include “carrier class” gateways with many thousands of SCN channels, capable of thousands of protocol transactions per second at their peak rates; and may also include gatekeepers with responsibilities for zones with large numbers of gateways, and other endpoints, of various sizes. Large devices will also include back-end servers, which are providing support for significant numbers of front-end devices that range in size from the very small to the very large.

Large devices will, typically, have large amounts of memory and processing power, relative to the smaller devices. They will also, typically, have many more protocol interactions than small devices, and that means more to store, more to process and more to transport through their network interfaces.

Large front-end devices, such as “carrier class” gateways and gatekeepers, will probably share the same types of code fragments as the small devices. These code fragments may be highly optimised, because of the large numbers of interactions with other devices, which are happening in the H.323 domain.

Back-end servers may be assumed to be a “clean slate” in terms of code fragments, having only IP (or equivalent), and related protocols like UDP and TCP. It should not be assumed that back-end servers have any higher level protocol capability in designing a protocol to interact with the other, H.323 capable, devices.

Back-end servers will benefit from the compact and efficient nature of the BEST protocol in the same way as the large gateways and gatekeepers.

## The Developers

The BEST protocol is designed with thought for the people who will be required to implement it. In the main, these people will be H.323 engineers who are already familiar with the protocol aspects of the H.323 group of standards. In particular, they will already be familiar with protocols described in Abstract Syntax Notation One (ASN.1), and encoded using the Packed Encoding Rules (PER). These engineers will have available to them techniques, tools and libraries, which are currently used for H.323 related software development.

For current H.323 implementations, there will already be a substantial investment in software code that implements the various functions of H.323. For new H.323 projects, the development engineers will need to be recruited, trained, and equipped to build those functions.

The protocol aims to capitalise on the significant investment involved in those developers, their knowledge, experience, tools and libraries.

For developers using externally sourced H.323 protocol stacks, their stack suppliers will have the experience, techniques, tools and libraries.

# The Transport

Transport channels can be either unreliable or reliable. In the IP world, UDP would be used for the unreliable channel and TCP would be used for the reliable channel. Each has advantages in certain circumstances and disadvantages in others.

Mechanisms exist to give UDP the characteristics of TCP, but almost universally, TCP would be a better choice if all of those characteristics were desired.

## Unreliable Transport (UDP)

The UDP transmission channel is an unreliable channel in that the service provides “best-effort” delivery of the data in packets. Transmitted packets are transferred in their entirety, with packet boundaries preserved. Packets may be lost, duplicated or received out of sequence. This is the mechanism specified for the RAS channel in H.323. For a protocol that involves multiple concurrent, independent, request-response transactions, such as BEST, UDP is ideal.

The UDP mechanism is well suited to protocols that have small message sizes and operate in a request-response mode, or where packet loss can be tolerated. Packet loss can be offset, by retransmission of a request when a response has not arrived within a reasonable time. This may cause packet duplication, however, so a mechanism must be implemented to counterbalance this effect. These mechanisms are well understood and are already employed in devices using the H.323 RAS protocol.

The sequence of transactions on a channel is not significant, and if a packet is lost for one transaction, other transactions on the same channel are not delayed while timeout and recovery takes place, as would be the case with TCP.

Significant transmission and delay savings can be made with UDP where connections are transient. A request-response with UDP is just that, a request in one direction and a response in the other. By contrast, TCP incurs a significant overhead in construction and destruction of connections for such short and simple exchanges as those found in BEST.

Protocols that use UDP include NTP, RADIUS, and DNS.

## Reliable Transport (TCP)

The TCP transmission channel is a reliable channel in that the service provides “guaranteed” delivery of the data as a stream. Transmitted data may be buffered and is transferred as a stream, without boundaries, with mechanisms to ensure that data is received in the same order as it was transmitted, without loss or duplication. This is the mechanism specified for the Call Signalling channel in H.323.

The TCP mechanism is well suited to protocols that transfer large volumes of data and where the sequence of the data is significant. Data stream buffering, packetisation, sequencing, timeout, and recovery of lost packets is all handled by TCP, which can efficiently and effectively transfer large volumes of data.

Protocols that use TCP include Telnet, HTTP, and DNS.

## The Case for Both

Some protocols are defined to use both UDP and TCP.  The DNS protocol is one that is specified on both UDP and TCP. Initially, UDP is used for the benefits that it brings to this request-response protocol, and TCP is used as a fallback mechanism when the size of the response is too big for UDP to carry reliably, or at all.

Typically, what happens is that requests for information are made over UDP, and in most cases, that works fine. Occasionally, information in the UDP response is truncated or lost and retries are unsuccessful. If this happens, a TCP channel is opened and the request is reissued.

For the BEST protocol, UDP is normally ideal. However, in the event of serious network quality degradation, it may become difficult to get messages through. It would be expected that the impact of such network degradation would make the media channels effectively unusable before it became a serious problem for the BEST protocol.

For some message exchanges, the volume of data could be such that it would not fit in a single UDP packet. These may include large price lists, or bulk transfers of usage information as might occur in a periodic settlement transaction. Price lists can be fragmented with the recipient told to ask for further information. Usage information can be transferred one record at a time.

An alternative mechanism is to use TCP as the transport mechanism. In this mechanism, a TCP channel is opened and the price list requested, or the usage data transmitted. The information flowing, in terms of the data in the requests and the responses, is the same. Acknowledgment processing and duplication control is still required, however, for recovery from TCP transport channel failures.

# Abstract Syntax Notation One (ASN.1)

The BEST protocol will be used in the H.323 engineering domain, where the protocols are specified using the standard Abstract Syntax Notation One (ASN.1 - ITU-T X.680). It is therefore to be expected that implementation engineers will already be familiar with interpreting protocol descriptions, and implementing systems, using ASN.1.

Abstract Syntax Notation One (ASN.1) provides a notation for defining the syntax of information data. It is a data specification language, which defines a number of simple data types, and specifies a notation for referencing these types, and for specifying values of these types.

ASN.1 allows the unambiguous specification of complex data structures including those with variable-length fields, optional fields, and recursion.

The ASN.1 notations can be applied whenever it is necessary to define the abstract syntax of information, without constraining in any way, how the information is encoded for transmission. It is particularly, but not exclusively, applicable to application layer protocols.

All of the protocols within the H.323 umbrella are specified using the ASN.1 notation. The BEST protocol is also specified in this manner.

# Packed Encoding Rules (PER)

The BEST protocol will be used in the H.323 engineering domain, where the protocols are encoded using the Packed Encoding Rules (PER - ITU-T X.691). It is therefore to be expected that implementation engineers will already be familiar with implementing systems using the Packed Encoding Rules.

The Packed Encoding Rules (PER) describes a set of encoding rules that can be applied to values of all ASN.1 types. Application of these encoding rules produces a compact representation of the values in a data unit described by ASN.1.

The protocols of H.225.0, including RAS, specify encoding of all ASN.1 using the aligned variant PER encoding. The BEST protocol specifies the same encoding as RAS.

# Security

The security mechanism for this protocol is optional, as it is for H.323. If provided, it shall be provided in accordance with H.235, as is specified for H.323.

If security is to be implemented in the H.323 device, it will be implemented in accordance with H.235 for the H.323 protocols. That H.235 implementation will also apply to the BEST protocol.

ITU-T Recommendation H.235 has the principle objectives of providing authentication, privacy and integrity for H.323 implementations.

The BEST protocol provides for the digital signing of transaction information. This provides for authentication of the signer of the information, and the information that has been signed. This provides a level of nonrepudiation for transactions with the BEST protocol. The legal implications of this verification and nonrepudiation may vary between different legal jurisdictions.

Where firewalls are used in an H.323 implementation, the firewall will almost certainly be configured to pass H.323 protocols, including RAS. This protocol is capable of being entirely encapsulated within the nonStandardData of the RAS messages.

# History

| Document History |  |  |
| --- | --- | --- |
| 0.0.1 | 03-Jun-98 | Initial Draft |
| 0.0.2 | 05-Jun-98 | Second Draft |
| 1.0.0 | 24-Jun-98 | Revised Edition |