ITU - Telecommunication Standardisation Sector                      	Document      Q09-16-san-rpt

STUDY GROUP 16			                    	Original : English

Study Group Meeting                                                                          _

Santiago May 17-28 1999

Question 9/16

SOURCE: Gunnar Hellström, Ericsson, Sweden

TITLE:      Q9/16 Report from meeting in Santiago 990517 -- 990528

PURPOSE:    Information

This is the meeting reports from the meeting of    Q9/16 “Accessibility to Multimedia for people with disabilities.”

After each title, relevant documents are mentioned. The documents are currently available at

//sunsite.dcc.uchile.cl/web/SG16M

This report takes the outcome of the study group plenary into account.

# Opening

Q9/16 had a meeting of its own one afternoon during the SG16 meeting. The rest of Q9/16 related work was achieved in joint meetings and by operation in other questions.

# Documents

TD 1-25 	Q9/16 Agenda and schedule

TD Gen-29 	Status report

TD 1-9        	Liaison to Q9/16 from Q17/2.

D289 	Minor Corrections to V.18

D288 	Marker for missing text in T.140

D282 	H.323 Annex G Text Conversation and Text SET

D290 	Application Profile; Sign Language and Lip reading use of low bitrate
video coding.

D291 	Minor Corrections to H.323 Annex F

TD Gen-32 	Modifications to H.224 for T.140

Gen-xxx	Overview of Total Conversation (Powerpoint Presentation)

TD Plen-8	V.18 Implementer’s guide.

TD Plen-10rev	T.140 Addendum

TD Plen-11	H.224 modifications

TD Plen-75	Copyright issues for Video sequence Irene.

# Administrative matters

## Preparations for next study period.

The Rapporteur, Gunnar Hellström announced that he currently has an agreement to continue this task on behalf of LM Ericsson through the feb. 2000 SG16 meeting.

Next study period starts later during 2000.

The group agreed that a continuation of actions for accessibility is of importance and should be proposed to the study group planning. A supportive letter has arrived from Gallaudet University (A University for Deaf Students in USA) commending the work done by the study group and announcing the intent of Gallaudet University to contribute to the work.

Currently viewed future work items are:

- Maintenance of the text telephone modem Recommendation V.18.

- Maintenance of the text conversation presentation Recommendation T.140.

- Service definition of Total Conversation ( as support to Q1/16)

- Finish establishment of Total Conversation and text telephony in the IP environment.

- Interworking between Total Conversation in all environments and with text telephony.

- Accessibility issues in IMT-2000.

- Alternative representation of media, and media conversion of interest for accessibility reasons.

- Accessible user control over Multimedia services.

- General information and awareness of needs and solutions for disabled and elderly people in the Multi media environment.

## Liaison from Q17/2.(TD 1-9).

This Liaison has arrived with answers on our earlier communication to Q17/2. It comments on the efforts of Q1/16 and Q9/16 to establish a service description for Total Conversation. Q17/2 comments that terminals for that service may be complicated to operate. Experience from Total Conversation implementations contradict this.

The liaison also comments that Total Conversation terminals may be expensive. That may be initially true, but should not hold us back from completing a service description. Q9/16 will encourage Q1/16 to continue work on conversation services including text, video and voice.

# Work status and work in other groups

## Review    of status report (TD Gen-29)

Status Report TD-Gen-29 was reviewed and accepted.

## Text conversation in H.320 (TD Gen-32, COM 16-86, plen-)

In H.320 multimedia systems for ISDN, a text conversation transport for T.140 is in preparation in the form of a new Client ID in H.224. The modifications to H.320 are for decision in this meeting, defined in COM 16-86, and the modifications to H.224 are handled by Q11/16 and are defined in TD-Gen-32. They were accepted as a modification to H.323 in Plen-11, and the revised H.224 was determined. (White doc should be proposed before Sep. 30 and decision is planned for Feb. 00)

## Text conversation in H.323 (D-282)

In H.323 multimedia systems, a text conversation transport for T.140 is in
preparation in the form of a channel set up by H.245. The additions to H.323, formed into H.323 Annex G, Text Conversation, and the determination in this meeting was accepted. This H.323 Annex G also contains a simple IP textphone description called “Text SET”. It is specified in D-282 and is handled in Q13/16. See Q13/16 report for the result of the discussions.

## Interworking of text conversation in different networks. (D286, TD 2-58 with Q14/16, pl-59)

Having text conversation now defined in many networks, next important topic is to make sure that interworking is achieved between them. Work should go into H.246, IETF MEGACO and H.gcp. Informal contributions have been accepted by MEGACO and entered in D286. No formal contributions have arrived for this topic in this meeting. The topic is handled by Q14/16. Text conversation is possible in the H.gcp structure for gateways in (Pl-29). Media conversion between text and voice is also mentioned. Continued work in this area from Q9/16 is urgently needed and included in the workplan.

The final report from wp2 specifies this as a topic for future work in sec 3.8.7: “New annexes in H.246 for data support (demod-remod for transit networks), including text telephony (total conversation).” (pl-59).

## Sign language and lip reading application profile    (D290 with Q15/16, plen-75)

A common requirement and information document created together with Q15/16 is now proposed for approval by the Study Group. A digital video sequence “Irene” is to be published together with it. The ITU was granted copyright by the Swedish Educational Broadcasting corporation who owns the sequence. Q9/16 and Q15/16 approved the document, as well as the Study Group. It goes now to publication.

## V.18 and H.324 in multi function terminals. (V.8 and V.8 bis, With Q4/16).

There was much discussion in the last SG16 meeting on small deficiencies in V.8bis that makes it hard to design devices with both V.18 and H.324 functionality. Clarifications were expected to this meeting, but no contributions have been entered. It should be a joint issue with Q.4/16.

## Multi modal conversational service definition containing Video, Text and Voice, (F.MVCS, Plen-65    with Q1/16).

Q1/16 has now put priority on the work with conversation service descriptions F.MVCS.    In a joint meeting, a draft multimedia conversation service description was reviewed. Details for Total Conversation, video requirements for sign language and lip-reading and for text telephony are to be entered by the Q1/16 rapporteur assisted by Q9/16. Goal: determination Feb –00. A new draft with accessibility aspects was presented by Q1-Rapporteur in Pl-65 and is now the base for further adjustments.

# Text telephone issues : V.18 Maintenance (D289, Plen-8)

D289 proposes minor modifications to two areas of V.18 Interworking procedures for modems in the text telephone mode. Firstly, a few corrections of the codes in the conversion table between 5-bit and T.50. Secondly, a corrected byte in the initialization of Minitel.

An unresolved problem is discovered with some plain Minitel terminals (the NON text telephone types of Minitel). There is a need to modify the XCI signal of V.18, but there is not yet any firm proposal. The current XCI does not stimulate all Minitels to respond. This is reported in a problem report. 
The modifications were accepted by the group, and were entered in the V.18 implementer’s guide and approved by the study group. (Pl-8)

# Text conversation presentation protocol: T.140 Maintenance (D288, Plen-10 rev)

The work with text conversation in IP has given rise to a need to define an indicator for missing text in T.140. That is to be used by a receiving channel that discovers that data is missing and cannot be recovered. A plain Unicode character “Lightning” is selected. The proposal was accepted, but editorial enhancements were proposed to the document D288. The result is proposed as a T.140 amendment (Plen-10rev) and the determination was approved by the study group.

# Workplan and future work

Future work in Q9/16 is proposed to include:

- Completetion of text conversation in H.323 (with Q13/16)

- Completion of text conversation in H.320 (with Q11/16)

- Completion of Sign Language and Lip Reading application Profile document.

- Maintenance on established text conversation modem Recommendation V.18, including its environment V.8 bis, V.8 and V.25x. A revision should be produced soon.

- Maintenance of text presentation protocol T.140.

- Conversation service definition containing Video, Text and Voice F.MVCS (with Q1/16).

- End to end interworking of Total Conversation including Video, Text and Voice.(With Q14/16)

- Any other accessibility issue related to SG16 issues.

Present status of Draft documents with Q9 as main responsible.

| Rec. No. | New Or Rev. | Title | Status in this meeting | Text in | Editor and question in co-operation | Pri |
| --- | --- | --- | --- | --- | --- | --- |
| T.140 Amendment | N | T.140 amendment | For approval | Plen-10-rev | Gunnar Hellström | 1 |
| V.18 impe-mentor´s guide | N | V.18 implementor´s guide | For approval | Plen-8 | Alistair Farquharson | 1 |
| H-series supplement | N | H-series supplement on Sign Language and Lip Reading | For Approval | D.290 | Gunnar Hellström Q15/16 | 1 |

Joint work and work in other groups with Q9 input.

| Rec. No. | New Or Rev. | Title | Status | Text in | Editor and co-operating group | Pri |
| --- | --- | --- | --- | --- | --- | --- |
| H.323 Annex G | N | Text Conversation | For Determination | D-282 | Gunnar Hellström, Q13/14 | 1 |
| F.MCVS | N | Multimedia Conversation Services | Draft | Working paper | Robin Champigneul, Q1/16 | 1 |
| H.224 revised | R | Low delay transport | For Determination | Plen-11 | Mickey Nasiri, Q11/16 | 1 |
| Megaco | N | Accessibility requirement to IETF Megaco and H.gcp | To be detailed | D.286 and TD2-58 | Q14/16 | 1 |

# Other matters

Events in and from other meetings and Questions:

A presentation of the status of Q9/16 work was made to a joint working party session. The slide show is available from the document storage in the Gen area. (WDOC-GEN.PPT)

One overview page from this presentation is attached to this report.

All changes needed in H.320 for H.263 V.2, developed by Q11/16, were decided in this meeting. This is an important event of interest for Q9/16, because it makes it possible to create good systems for Sign Language and Lip Reading down to 128 kbit/s. (see D-290)

H.323 Annex F, the Audio SET, is a specification of a simple IP voice phone. It was decided in this meeting. The Text SET in D-282 is based on that important specification. (COM 16-73 + Pl-52)

H.323 Version 3, is the base where many new items are added. Annex G – Text Conversation and Text SET is among them. H.323 Version 3 was determined in this meeting. (Pl-57)

The Gateway decomposition H.gcp was determined in this meeting. It does not specifically mention text conversation and text telephony to be supported, but being a general structure, it does not include any restrictions that could make support impossible. (Pl-29)

# Closing

No meetings in Q9/16 are planned before next Study Group meetings. Work related to Q9/16 is supposed to be progressed by members of the group in Q13/16, Q14/16 and possibly in Q4/16.

Attachment: Overview over Total Conversation Recommendations. (from WDOC_gen.ppt)

Text - aware Gateway

Ready

Ready

Ready

On the way

On the way

In requirements proposal

PSTN

V.18

Text

telephony Ready

Trans

parent

Equal-

izers

T.140

PSTN

H.223

V.34/V.80

AL1

H.245

H.324

T.140

Voice

and

video

ISDN

H.221

Network

access

H.320

H.224

T.140

Voice

and

video

IP

H.323

H.225.0

Network

access

T.140

TCP

H.245

Voice

and

video

MOBILEE

H.324

Mobile system

H.223

Mobile

transmission

Voice

and

video

AL1

H.245

T.140

MOBILE

IP

TCP

H.323

Mobile system

H.245

H.225.0

Mobile

transmission

Voice

and

video

T.140

DATA

CONF.

T.124

GCC

T.134

T.120 Data

conferencing

T.123

T.140

Total Conversation -
a growing family

Contact: Gunnar Hellström,  Ericsson Sweden
Tel +46 751 100 501  Fax +46 556 002 06 		e-mail  gunnar.hellstrom@omnitor.se