| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | TD xxx (WP2/16) |
|  |  | English only Original: English |
| Question(s): | 1/16 | Geneva, 13th June 2006 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Editors H.324 Annex K |  |
| Title: | Draft new H.324 Amendment [1]: “New Annex K: Media Oriented Negotiation Acceleration Procedure” (for Consent) |  |

# General

This new amendment 1/H.324 contains the text for a new draft Annex K/H.324 along with some associated changes to Annex J/H.324 (ASN.1 OIDs defined in H.324). The technical content remains unchanged from the document presented in TD271/Plen that was technically frozen at the April 2006 SG16 Plenary. TD271/Plen was based on D.172, D.207, and D.293. Some clarifications and changes of editorial nature were made during the review process and they are summarized as follows:

- Update of H.245 OID numbers

- Corrected referencing mistakes, particularly clause K.8

- Replaced reference of Section X.y.z by clause X.y.z

- Referenced Figures and Tables in line with ITU-T styling (Table X.x/H.324)

- Terminology is made consistent for SPC and MOS

- Language employed in some areas is made more precise, particularly in K.10

# AAP Summary

ITU-T H.324 Annex K describes a procedure comprising complementary methods for significantly reducing delay in H.324 call setup. It provides a fast channel setup mechanism which does not wait for capability exchange but requires a fallback if the initial media transmission attempts do not succeed.  It provides a flexible accelerated channel setup method that depends on an initial exchange of preferences and the execution of a common inference algorithm.  It provides an accelerated H.245 setup method as a simple and reasonably fast technique in cases where the other methods are not suitable. Interoperation with legacy terminals is also preserved across the procedures.

Draft new Amendment [1] to H.324

# 1.  Addition of new OIDs to Annex J/H.324 as introduced by Annex K

The following OIDS for newly defined generic parameters in Annex K/H.324 are being added to Table J.1/H.324 as follows:

[Begin Correction]

| { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) mona(2) } | K.10.1 |
| --- | --- |
| { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) mona(2)mos(1) } | K.8.3 |
| { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) mona(2)mosack(2) } | K.8.3 |

[End Correction]

# 2.  New Annex K: Media Oriented Negotiation Acceleration Procedure

Annex K

Media Oriented Negotiation Acceleration Procedure

K.1 	Abstract

This Annex specifies an optional enhancement to the H.324 call setup procedure, to be used only in conjunction with Annex C, which allows media channels to be established in a fast and flexible manner.

## K.2 	General

When the optional Media Oriented Negotiation Acceleration Procedure is in use, there is an initial transmission of messages called MONA Preference Messages that are used to transmit preferences relevant to initial setup of the media channels.  The details of media channel setup fall to one of several established signalling methods, depending on the channel setup capabilities and preferences of the terminals as exchanged in the Preference Messages. Terminals may utilize a fast channel setup which does not wait for capability exchange, but which requires fallback in the case where the initial channel transmission attempts do not succeed.  Terminals may also utilize a flexible accelerated channel setup method that depends on an initial exchange of preferences and the execution of a common inference algorithm on both sides.  All terminals are capable of a reasonably fast fallback mechanism implemented as a minor change to the existing H.245 procedures.

## K.3 	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

- ITU-T Recommendation H.223 (2001), Multiplexing protocol for low bit-rate multimedia communication.

- ITU-T Recommendation H.245 (1999), Control protocol for multimedia communication.

- ISO/IEC 14496-2:1999, Information technology – Coding of audio-visual objects Part 2: Visual

- ITU-T Recommendation H.264 (2003), Advanced video coding for generic audiovisual services.

- ETSI TS 126 071 V6.0.0 (2004-12) Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); AMR speech Codec; General description (3GPP TS 26.071 version 6.0.0 Release 6)

- ETSI TS 126 171 V6.0.0 (2004-12) Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); AMR-WB speech Codec; General description (3GPP TS 26.171 version 6.0.0 Release 6)

## K.4 Definitions and format conventions

### K.4.1 Definitions

Inferred common mode (ICM):  The unique non-conflicting media mode determined by both terminals based on media preferences of local and peer MOS profile requests (always the same for both terminals).  ICM is applicable to MOS only.

Normal multiplexer level operation (NMLO): The normal operation of the H.223 multiplexer on the bearer channel. This is Phase E of H.324.

### K.4.2 Symbols and Abbreviations

FEA		Frame Emulation Avoidance procedure

FI		Frame Information

LCN		Logical Channel Number

MONA		Media Oriented Negotiation Acceleration

MTE		Multiplexer Table Entry

MOS		Media Oriented Setup

MPC		Media Preconfigured Channel

MUX		Multiplexer

OLC		Open Logical Channel

PDU		Protocol Data Unit

PSR		Payload Segmentation and Reassembly

SDU		Service Data Unit

SPC		Signalling Preconfigured Channel

SPP		Signalling Preconfigured Channel Preference

### K.4.3 Format Conventions

The numbering, field mapping and bit transmission conventions used are consistent with those used in 3.2/H.223 (clause 3.2 of ITU-T H.223 Recommendation).

## K.5 Terminal Procedures

The steps for provision of communication are as listed in clause C.5/H.324 with the following modifications:

Phase D:  MONA phase, as specified in this annex, is inserted during the level set-up procedure.

## K.6	 MONA Signalling

The initial exchange of preferences between MONA-capable terminals is performed using Preference Messages, short messages that include signalling to accelerate the establishment of multimedia sessions.  These messages shall include the information of the supported method for initial media channel creation.

### K.6.1 Framing

The MONA Preference Message frames are octet aligned and have the structure shown in Table K.1/H.324.

Table K.1/H.324 – Structure of a MONA Preference Message frame

Frame Information (FI) (1 octet)

Payload

(0 or more octets up to 150 octets)

CRC (2 octets)

Payload Length (PL) (1 octet)

Reserved (Always 0x00) (1 octet)

The Frame Information (FI) bit allocation is shown in Table K.2/H.324.   Bit 8 is reserved and shall be set to 1. Bit 7 represents the Last Segment (LS) flag, and the three following bits represent the Segment Sequence Number (SSN).  The three least significant bits are reserved and shall be set to 0.

Table K.2/H.324 – Structure of the MONA Frame Information (FI) field

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LS | SSN3 | SSN2 | SSN1 | 0 | 0 | 0 |

The Payload Length (PL) field indicates the payload size in octets before the application of the Frame Emulation Avoidance (FEA) procedure.

The Payload consists of the Preference Message Capability Description, as defined below.

The CRC (cyclic redundancy check) field is 16 bits and is determined by applying the CRC described in 8.1.1.6.1/V.42 to the entire frame, excluding the MONA Synchronization Flags and the CRC field, and before FEA.

On detecting a CRC error or undefined Frame Information or undefined Reserved bits, the corresponding MONA Preference Message frame shall be discarded.

The MONA Synchronization Flag is defined in Table K.3/H.324.

Table K.3/H.324 – Structure of the MONA Synchronization Flag

1  0  1  0    0  0  1  1

0xA3

0x35

0  0  1  1    0  1  0  1

One MONA Synchronization Flag shall be inserted immediately before and after each Preference Message Frame.  Only one MONA Synchronization Flag shall exist between two consecutive Preference Message Frames.

Segmentation and Reassembly of Preference Message frames is done using a modified version of the CCSRL procedure defined in C.8.1/H.324.  The following modifications are made:

- The LS flag shall be used in place of CCSRL LS.   LS shall be set to 1 on the PDU containing the last segment of an SDU.   It shall be set to 0 otherwise.

- The SSN shall be set to 0 for the first segment and monotonically incremented for each segment, the maximum value of SSN shall be 6.  The value 7 is reserved.

A Flag Emulation Avoidance (FEA) procedure shall be performed against synchronization flags for all multiplexer levels of H.324 before transmitting a MONA Preference Message frame onto the bearer.  Frame Information, Payload Length, Payload and CRC are included in the FEA procedure.  All octets with values 0xA3, 0x35, 0xE1, 0x4D, 0x1E, 0xB2, 0x19, 0xB1, 0x7E and 0xC5 shall have an octet with value 0xC5 inserted immediately preceding them.

### K.6.2 Payload

The Preference Message payload contains the information of the initial accelerated call setup method capabilities. These capabilities indicate which methods may be used for establishment of media channels.  The payload is defined in Table K.4/H.324 and Table K.5/H.324.

Table K.4/H.324 – Bit fields defining Preference Message Capabilities

| 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VER |  | SPC | MPC-RX |  |  |  |  |  |  |  |  |  |  |  |  |
| ACK |  | SPP | MPC-TX |  |  |  |  |  |  |  |  |  |  |  |  |
| SPC-ML |  |  |  |  | reserved |  |  |  | EXT-LEN |  |  |  |  |  |  |

Table K.5/H.324 – Preference Message Capability Definition

| Capability name: | Description: |
| --- | --- |
| Version (VER) | MONA version number (2 bits). It shall be set to 0 for the current version. The value 3 is reserved. |
| Signalling Preconfigured Channel (SPC) Support | Set to 1 if the MONA terminal supports negotiation of logical channels using the Signalling Preconfigured Channel, set to 0 otherwise. |
| Media Preconfigured Channel Receive bits (MPC-RX) | (13 bits) Describes which Media Preconfigured Channel configurations the MONA terminal is capable of receiving. The bit numbers (from 1 to 13) as shown in the table correspond exactly to the Mux Code values in the Media Preconfigured Channel configuration (See Table K.15/H.324). |
| Acknowledgement State (ACK) | The MONA terminal shall set ACK in its outgoing Preference Messages as follows: 00 – MONA terminal has not successfully received any incoming Preference Messages 01 – MONA terminal acknowledges receiving at least one incoming Preference Message containing an ACK value of 00. 10 – MONA terminal acknowledges receiving at least one incoming Preference Message containing an ACK value of 01 11 – Reserved. |
| Signalling Preconfigured Channel Preference (SPP) | Set to 1 if the MONA terminal prefers negotiation of logical channels using the Signalling Preconfigured Channel, set to 0 otherwise. |
| Media Preconfigured Channel Transmit Bits (MPC-TX) | (13 bits) Describes which Media Preconfigured Channel configurations the MONA terminal is capable of sending. The bit numbers (from 1 to 13) as shown in the table correspond exactly to the Mux Code values in the Media Preconfigured Channel configuration (See Table K.15/H.324). |
| Signalling Preconfigured Channel Multiplex Level (SPC-ML) | (5 bits) Represents the SPC multiplexer level preference of the terminal. SPC-ML shall only be used when MOS is used. First 3 MSB bits indicate initial multiplexer level. 4th bit indicates using H.223 Annex A double flag mode. 5th bit indicates using H.223 Annex B optional header mode. Operating multiplexer level shall be determined as defined in C.6.2/H.324 but without transmission, recognising the SPC-ML in the transmitted and received Preference Message as the starting points. |
| Extension Length (EXT-LEN) | Length of additional capability information, in octets. |

Terminals shall support at least one Preconfigured Channel receive capability.

NOTE: This requirement may be met by setting SPC to 1, or by setting at least one non-zero bit within MPC-RX.

The SPC field indicates whether the terminal supports channel negotiation using the procedures defined in clause K.8. The SPP field indicates whether the terminal prefers channel negotiation using the procedures defined in clause K.8.  Interpretation of these fields is given in clause K.7.

Signalling Preconfigured Channel Multiplex Level (SPC-ML) is used to signal the multiplex level used in the case where the media channels are negotiated using the procedures of clause K.8.

The ACK bits are used by the terminal to signal the acknowledgement state of the peer’s Preference Message.

Future protocol versions may add extra fields to the end of the capability information defined in Table K.4/H.324.  The length of such extra information in octets is provided in the EXT-LEN field. For compatibility with such future versions, a terminal that receives a Preference Message with VER set to an unknown version should discard the extra information.

Preference Message frames may be used to encapsulate media per the definition in clause K.9.3 and encapsulate signalling per the definition of K.9.4.

## K.7 Channel establishment method negotiation

## An informative overview of MONA Transmission is provided in Figure K.1/H.324.   Whenever there is a conflict between the diagram and the text, the text applies.

Figure K.1/H.324: Overview of MONA Transmission procedures

An informative overview of MONA Reception is provided in Figure K.2/H.324. Whenever there is a conflict between the diagram and the text, the text applies.

Figure K.2/H.324: Overview of MONA Reception procedures

## A MONA-capable terminal begins the session establishment procedure by transmitting at least ten repetitions of the Preference Message which contains information about its channel establishment capabilities and preferences.  All outgoing Preference Messages sent from a terminal during a particular session shall contain identical information in the Preference Message Capability payload, with the exception of the Acknowledgement State (ACK).

The ACK bits are used by the terminal to signal an acknowledgement of the peer’s Preference Message.  These bits are set to 00 in the initial transmission.  After receiving at least one incoming Preference Message, the ACK bits in subsequent outgoing Preference Messages shall be set to 01 to acknowledge the reception of the incoming Preference Message.  After receiving at least one incoming Preference Message with the ACK set to 01, the ACK bits in subsequent outgoing Preference Messages shall be set to 10.  After the terminal has received an incoming Preference Message with ACK containing value 10, the terminal shall stop sending outgoing Preference Messages.

After sending the initial Preference Message repetitions, but before receiving incoming Preference Messages from the remote end, the MONA-capable terminal may make outgoing transmissions with any combination of the following:

- It may send media data on one or more Media Preconfigured Channels (per definition in clause K.9.3)

- It may send session signalling data on the Signalling Preconfigured Channel (per the definition in clause K.9.4)

- It may send multiplexer level setup flags (per the definition in clause K.7.1)

The MONA-capable terminal shall not transmit media on any Preconfigured Channels not indicated in the MPC-TX bits of the outgoing Preference Messages sent by the terminal.  The terminal shall continue to send at least one Preference Message between each pair of outgoing Preconfigured Channel  PDUs until the stopping criterion (defined previously) is reached.

Upon the successful receipt of an incoming Preference Message, the MONA-capable terminal shall examine the received MPC-RX bits to determine which outgoing Media Preconfigured Channels were not successfully established.  The transmission of any such unsuccessful channels shall be immediately dropped.

At this point, the channel negotiation behaviour of the terminal is determined as follows:

- If both terminals have indicated support for the procedures of clause K.8 (SPC) and at least one of the two terminals indicates preference for those procedures (SPP), then all outgoing Media Preconfigured Channels shall be dropped, and channel negotiation shall proceed using the procedures of clause K.8.

NOTE: In this case, the Media Preconfigured Channels are never considered to be successfully established.

- If both terminals have indicated support for the procedures of clause K.8 (SPC) and a comparison of the MPC sending and receiving capabilities (expressed as the MPC-RX and MPC-TX bits exchanged between both sides) indicates that no successful Preconfigured channels can be established in the current session, then channel negotiation shall proceed using the procedures of clause K.8.

NOTE: such determination may be made by logical AND-ing of the local MPC-RX bits with the remote MPC-TX bits, and likewise AND-ing of the remote MPC-RX bits with the local MPC-TX bits.

- In all other cases, the terminal shall establish the missing outgoing media channels for each media type using one of the following procedures:

  - If examination of the received MPC-RX bits indicates that a Media Preconfigured Channel can be successfully established for the given media type, the terminal may begin transmitting media on the appropriate Preconfigured Channel.

NOTE: Successful channel establishment is guaranteed in this case.  If this is used for fallback following a failed transmission attempt, then it is equivalent to the MPC-Fallback procedure defined in clause K.9.3.

  - The terminal may establish the outgoing channel using the procedures of clause K.10.

- If the procedures of clause K.8 (SPC) are used to establish media channels, the following apply:

  - SPC negotiation procedures exchange Media Oriented Setup (MOS) request messages in order to establish media channels.

  - The SPC channel negotiation will make use of any MOS request messages previously exchanged using the SPC signalling Preconfigured Channel.  The initial exchange of MOS request messages is independent of the exchange of Preference Messages.

  - If one or both sides have not sent initial SPC signalling Preconfigured Channel transmissions, they shall begin to do so as soon as the decision to use SPC has been made through the exchange of Preference Messages.

### K.7.1 Multiplexer Flag Interleaving

A terminal should insert stuffing flags of its multiplexer level, as described in C.6.1/H.324, between adjacent Preference Messages and/or Preconfigured Channel PDUs.  No more than 20 stuffing flags shall be inserted.  Interleaving should be halted upon reception of a Preference Message.

### K.7.2 Legacy interworking

When communicating with a legacy terminal, a MONA terminal will encounter standard multiplexer level flags.  In order to have negligible impact on setup time with the legacy terminal a MONA terminal should try and detect the standard multiplexer level setup as soon as possible and revert to legacy behaviour, cease all MONA transmissions and follow normal start up procedures as defined in H.324 Annex C.  Either of the following conditions shall initiate fallback to legacy behaviour:

- More than 20 valid consecutive multiplexer level stuffing flags are detected, as described in C.6/H.324.

- A normal start up procedure with detection of a normal H.245 TerminalCapabilitySet message as the first non-empty H.223 MUX-PDU at an agreed initial multiplexer level.

## K.8 Channel Establishment via the Signalling Preconfigured Channel (SPC)

### K.8.1 MOS Profile Exchange

#### K.8.1.1 Procedure

Once the bearer is established, if a terminal supports the SPC, it shall send its MOS Request (mos) using the SPC (see Table K.6/H.324).  The MOS Request transmissions should be repeated until a MOS requestAck (see Table K.14/H.324) is detected, or one of the conditions in clause K.8.2 is fulfilled.  For the latter case, the procedure in clause K.8.2 shall be followed.

When a MOS Request is detected and decoded successfully, from the MOS SPC, the terminal accepts it by beginning the transmission and processing of media data as determined by the ICM at NMLO using the agreed mobile level.  MOS requestAck shall be sent on receiving every MOS Request.

If MOS is completed successfully, H.245 message exchange is skipped and opened logical channels operate immediately. The procedure is shown in Figure K.3/H.324.

NOTE: The channels established through the MOS procedure operate immediately on their determined multiplexer entries and require no special framing.

Figure K.3/H.324 – MOS call flow

For the master-slave determination, when the terminalType (Table K.7/H.324) fields differ, the terminal which has higher terminalType value shall be the master.  When the terminalType fields in the MOS Request of the two terminals are identical, and the two terminals having different values of caller (Table K.11/H.324) field, the caller shall be the master; if the caller fields are identical, the terminalType and statusDeterminationNumber (Table K.12/H.324) fields in the MOS Request of the two terminals are used according to the Master-slave determination procedure in C.2/H.245 and in an inferred manner without additional H.245 signalling.

Unexpected MOS-SDUs shall be discarded.

#### K.8.1.2 Logical Channels

A terminal indicates its requested logical channels by listing H.245 OpenLogicalChannel (OLC) requests according to an order of preference in mediaProfile (Table K.8/H.324).  The requests shall be processed in the same order.

Logical channel numbers (LCNs) are assigned by the message originator.  OLC requests with the same LCN indicate alternative media capabilities for the logical channel.  For bidirectional logical channels, the reverse LCN shall be the same as the forward LCN.  If a reverse LCN is already assigned, the next available LCN shall be assigned.  The highest LCN shall be 13, and any OLC requests that lead to LCN exceeding 13 shall be ignored.

If ICM contains an H.223 adaptation layer type not supported by a terminal, the terminal shall fallback as described in clause K.8.2.

#### K.8.1.3 Multiplex Table Entries

The logical channel number shall be mapped to H.223 multiplex entry index.  For example, if logical channel 1 is opened, multiplex entry index 1 will be associated to this logical channel as “{LCN1, RC UCF}”.  For a reverse logical channel, the logical channel number shall be mapped to multiplex entry index at the H.223 demultiplexer.

Explicit multiplex table entries may be set using additionalInfo (Table K.10/H.324) parameter.

Alternate multiplex entries may be signalled similarly as assigning LCNs for alternative media capabilities in clause K.8.1.2.

NOTE – Outgoing LCNs specified in explicit multiplex table entries for transmission are not expected to be changed.

NOTE – e.g. for alternative logical channels of {AMR, G.723.1} with LCN 3 and {H.263, H.261} with LCN 2, additional multiplex entries may be set as follows:

Index 5: (empty); Index 5:{LC 3, RC 22}, {LC2, RC UCF}

Index 7: {LC 3, RC 32}, {LC2, RC UCF}; Index 7: {LC 3, RC 25}, {LC2, RC UCF}

Index 8: {LC 3, RC 7}, {LC2, RC UCF}

This represents when AMR is selected, multiplex entries are: Index 7: {LC 3, RC 32}, {LC2, RC UCF}; Index 8: {LC 3, RC 7}, {LC2, RC UCF}; when G.723.1 is selected, multiplex entries are: Index 5:{LC 3, RC 22}, {LC2, RC UCF}; Index 7: {LC 3, RC 25}, {LC2, RC UCF}.

### K.8.2 Fallback Procedure

A fallback procedure shall be used by a MOS terminal to switch to the next phase of normal operation mode as described in K.7.

MONA Fallback is specified in K.7.2, the following additional conditions shall also initiate a fallback from MOS:

- A normal H.245 TerminalCapabilitySet message with empty genericControlCapability containing MOS OID after completion of the MOS procedure.

- A terminal does not detect a valid MOS request, or does not accept the ICM, within a multiple of the network round trip delay (RTD) period. Typically, 3 RTDs is adopted.

### K.8.3 MOS Messages

Table K.6/H.324: MOS Capability Identifier

| Capability name: | mos |
| --- | --- |
| Capability class: | Control capability. |
| Capability identifier type: | Standard. |
| Capability identifier value: | { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) mona(2) mos(1)} |

Table K.7/H.324: MOS Parameter - terminalType

| Parameter name: | terminalType |
| --- | --- |
| Parameter description: | Terminal type as defined in 7.4/H.324. |
| Parameter identifier value: | 2 |
| Parameter status: | Mandatory |
| Parameter type: | unsignedMax |
| Supersedes: | - |

Table K.8/H.324: MOS Parameter - mediaProfile

| Parameter name: | mediaProfile |
| --- | --- |
| Parameter description: | One or more H.245 OpenLogicalChannel structures specifying media channels in order of preference. |
| Parameter identifier value: | 4 |
| Parameter status: | Optional |
| Parameter type: | octetString |
| Supersedes: | - |

Table K.9/H.324: MOS Parameter - mediaSymmetric

| Parameter name: | mediaSymmetric |
| --- | --- |
| Parameter description: | When set, all media capabilities are symmetric as per H.245. When this parameter is absent, all media capabilities are non-symmetric as per H.245. |
| Parameter identifier value: | 5 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

Table K.10/H.324: MOS Parameter - additionalInfo

| Parameter name: | additionalInfo |
| --- | --- |
| Parameter description: | One or more H.245 MultimediaSystemControlMessage such as UserInputIndication, MultiplexEntrySend and TerminalCapabilitySet. OpenLogicalChannel should not be included. No H.245 response shall be generated for request messages interpreted as commands. Only messages with settings within the mandatory limit of the recommendation shall be sent as receivers, receivers shall ignore messages outside these limits. Response messages are meaningless. Values specified in this parameter take precedence to inferred values. An exception is to MultiplexEntrySend such that inferred multiplex entry indice take precedence. If a TerminalCapabilitySet is supplied, OPTIONAL fields such as multiplexCapability, capabilityTable and capabilityDescriptors are optional. Capabilities already inferred from mediaProfile parameter should not be included. Additional capabilities such as receiveUserInputCapability may be appended to capabilityTable. |
| Parameter identifier value: | 6 |
| Parameter status: | Optional |
| Parameter type: | octetString |
| Supersedes: | - |

Table K.11/H.324: MOS Parameter - caller

| Parameter name: | Caller |
| --- | --- |
| Parameter description: | Indication that the terminal is a caller. When not specified, the terminal is a callee. |
| Parameter identifier value: | 7 |
| Parameter status: | Mandatory |
| Parameter type: | Logical |
| Supersedes: | - |

Table K.12/H.324: MOS Parameter - statusDeterminationNumber

| Parameter name: | statusDeterminationNumber |
| --- | --- |
| Parameter description: | A random number as defined in B.1.1/H.245. |
| Parameter identifier value: | 8 |
| Parameter status: | Mandatory |
| Parameter type: | Unsigned32Max |
| Supersedes: | - |

Table K.13/H.324: MOS Ack Capability Identifier

| Capability name: | mos Ack |
| --- | --- |
| Capability class: | Control capability. |
| Capability identifier type: | Standard. |
| Capability identifier value: | { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) mona(2) mosack(2)} |

Table K.14/H.324: MOS Ack Parameter - requestAck

| Parameter name: | requestAck |
| --- | --- |
| Parameter description: | Acknowledge receipt of MOS message. One Ack message shall be sent for every MOS message received. |
| Parameter identifier value: | 10 |
| Parameter status: | Mandatory |
| Parameter type: | Logical |
| Supersedes: | - |

## K.9 Preconfigured Channel Establishment

This section specifies a procedure by which media channels may be established before capabilities and intents are exchanged.  Specifically, a MONA terminal is allowed to send media before receiving any capability or other information from the opposite terminal.  This allows for fastest possible setup of channels, but limits the setup options to a small set of fixed channel configurations.

### K.9.1	 General

When the procedures of the current section are in use, several of the H.223 multiplex table entries are defined to reference default configurations for the multiplex table, codec configuration, and other related parameters.  Such default operation points may be used to quickly establish channels for data flow, called Preconfigured Channels.  Preconfigured Channels may be used to carry media data, or they may be used to exchange signalling data in order to negotiate regular logical channels.  Once a Preconfigured Channel is established, it may be used for the duration of the session.  Media Preconfigured Channels may be replaced through H.245-based negotiation of a regular logical channel of the same media type.  If the procedures of clause K.7 determine that SPC-based negotiation will not be used to establish channels in the current session, then any current transmission of SPC data shall be dropped.

### K.9.2	 Channel Configurations

The following combinations shown in Table K.15/H.324 of codec, LCN, and multiplex table configuration shall be used within the MPC establishment procedure in order to establish Preconfigured Channels.

Table K.15/H.324 - Channel Configurations

| Codec | Mux Code | LCN | Multiplex Table Entry |
| --- | --- | --- | --- |
| Reserved (H.245) | 0 | - | - |
| AMR [5] | 1 | 1 | {1 ucf} |
| AMR-WB [6] | 2 | 2 | {2 ucf} |
| H.264 [4] | 3 | 3 | {3 ucf} |
| MPEG-4 [3] | 4 | 4 | {4 ucf} |
| Reserved | 5..11 |  |  |
| Unspecified, left for operator use | 12..13 |  |  |
| Signalling Preconfigured Channel (SPC) | 14 | 14 | {14 ucf} |
| Reserved (WNSRP) | 15 | - | - |

Each of the media codec options is associated with fixed configuration information.  The configuration information consists of the whole state pertaining to a logical channel, as if that channel would have been opened using the normal Open Logical Channel procedure of Rec. H.245.

#### K.9.2.1 AMR Speech (mux code 1)

Codec configuration:

maxBitRate = 12.2 kbit/s

maxAL-SDU-Frames = 1

H223 configuration:

AL2 with sequence numbers

(non-segmentable)

#### K.9.2.2 AMR-WB Speech (mux code 2)

Codec configuration:

maxbitRate = 23.85

maxAl-sduFrames = 1

octetAlign = TRUE

modeSet = all modes available

modeChangePeriod = anytime

modeChangeNeighbour = FALSE

crc = FALSE

H223 configuration:

AL2 with sequence numbers

(non-segmentable)

#### K.9.2.3 H.264 Visual (mux code 3)

Codec configuration:

Sequence and Picture Parameter sets shall be inferred to be set as if the following, base64 encoded bit stream were received at the decoder:

Base64: AAAAASdC4AqVoLE6Af1AAAAAASjOBmo=

Hex: 00 00 00 01 27 42 e0 0a 95 a0 b1 3a 01 fd 40 00 00 00 01 28 ce 06 6a

H223 configuration:

AL2 with sequence numbers

(segmentable)

NOTE:  The above base64 coded bitstream corresponds to a single sequence parameter set with ID 0, and a single picture parameter set with ID 0.  The properties are set to a widely accepted operation point of H.264, which can be summarized as baseline profile observing main profile constraints, level 1.0, QCIF picture format, 8 bit frame_num, one reference picture, and constrained intra prediction on.

#### K.9.2.4 MPEG-4 Visual (mux code 4)

Codec configuration:

QCIF only

maxBitRate = 64 kbit/s

profileAndLevel = 8

object = 1

decoderConfigurationInformation:

Base64: AAABsAgAAAG1CQAAAQAAAAEgAIRdTCgsIJCijw==

Hex: 00-00-01-b0-08-00-00-01-b5-09-00-00-01-00-00-00-01-20-00-84-5d-4c-28-2c-20-90-a2-8f

H223 configuration:

AL2 with sequence numbers

## (segmentable)

### K.9.3 	Sending Procedure – Media Preconfigured Channels

MONA terminals may use Preconfigured Channels to establish the initial outgoing audio and visual connections to the remote terminal.  The timing for the initiation of outgoing media Preconfigured Channel transmissions is restricted by the following rules (see also clause K.7):

- Media Preconfigured Channel transmission may not start until the initial outgoing Preference Message repetition requirement has been met.

- Media Preconfigured Channel transmission may not begin after the decision has been made to negotiate media channels using the procedures of clause K.8 (SPC).

- Preconfigured Channel transmission for a particular media type shall not begin after an outgoing channel of the same media type has been already established (whether via a different Media Preconfigured Channel or by any other means).  Equivalently, a Preconfigured Channel may only be used for the initial establishment of a channel for a given media type.

Media Preconfigured Channels shall be formatted according to the following rules:

- Until the receipt of at least one incoming Preference Message with ACK set to 10, outgoing Preconfigured Channel media PDUs shall be encapsulated within MONA Preference Messages.

  - The framing for Preference Messages is defined in clause K.6.1.

  - The payload for Preference Messages consists of the Preference Message Capability payload defined in clause K.6.2, immediately followed by:

    - One octet where the Mux Code is carried in the least significant four bits.  The Mux Code is taken from the appropriate Media Preconfigured Channel configuration defined in clause K.9.2.

    - Media data for the PDU.

  - NOTE: the PSR and FEA procedures normally used for Preference Messages are applied as well to Preference Messages which encapsulate media payloads.

- After the receipt of at least one incoming Preference Message with ACK set to 10, outgoing Preconfigured Channel media PDUs are sent as standard H.223 Level 2 PDUs, using the appropriate mux code and configuration described in clause K.9.2.   Note that the support for other multiplex levels during the call is possible only using the dynamic level change procedure described in clause C.7/H.324.

The initial MPC transmission results in the successful establishment of Preconfigured Channels only if the remote terminal supports receipt of the specific configurations transmitted.  The remote terminal signals this in its outgoing MPC-RX field.   The sending terminal discovers whether the initial outgoing Preconfigured Channels were established upon receipt of the first incoming Preference Message from the remote terminal.  Upon learning the remote terminal’s capabilities, the sending terminal shall drop all current unsuccessful transmissions.   Each unsuccessful transmission may be replaced via a new Preconfigured Channel transmission that the remote terminal is known to support.  This procedure is known as “MPC-Fallback”.  Alternately the Accelerated H.245 Procedures described in clause K.10 may be used to establish a successful channel.  More details on the selection of negotiation procedure are given in clause K.7.

Once established, a Media Preconfigured Channel shall be treated as though it were negotiated using H.245 logical channel procedures.  H.245 messages which reference the channel shall do so using the appropriate LCN defined in clause K.9.2.

Any terminal which begins MPC transmission before the receipt of the first incoming Preference Message may need to switch its outgoing codec, since such a switch may be necessary in the event that the initial transmission does not lead to successful channel establishment.

For media codecs that employ cross-AL SDU prediction (e.g. video codecs employing inter picture prediction), it is recommended to send decoder refresh points (e.g. intra frames) frequently during the session setup.

Terminals implementing the procedures of this section should be capable of responding to the H.245 videoFastUpdatePicture command.

### K.9.4 Sending Procedure –Signalling Preconfigured Channel

MOS messages as defined in K.8.3 are transmitted over the Signalling Preconfigured Channel.   The SPC shall use H.223 AL2 as a segmentable channel.  Each MOS request shall be repeated until an acknowledgment is received via the SPC Acknowledge bit.

PDUs carrying MOS messaging within the Signalling Preconfigured Channel shall be encapsulated using the MONA Preference Message frame structure as defined in clause K.6.1, and following PSR Procedure and FEA according to K.8.2.

### K.9.5 	Receiving Procedure

Terminals which support Media Preconfigured Channel establishment shall search for incoming Preconfigured Channel PDUs for which receive capability was indicated in the MPC-RX field of the outgoing Preference Messages.  When acceptable incoming Preconfigured Channels are detected, the terminal shall begin to decode the received audio and/or video data.  Incoming Preconfigured channel data for unknown or unsupported codec configurations shall be ignored by the receiver.

A terminal which is capable of Media Preconfigured Channel data shall be prepared to accept new incoming audio and/or video Preconfigured Channels at any time until one of the following conditions are met:

- The terminal determines (per the decision algorithm in clause K.7) that the remote terminal will not use the procedures of this section to establish a Preconfigured Channel of the given media type.

- An incoming channel of the given media type is successfully established, whether via a Media Preconfigured Channel or by any other means.

K.10 Accelerated H.245 Procedures

### K.10.1 Accelerated H.245 Signalling

Terminals shall signal MONA parameters by specifying the mona capability identifier, OID { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) mona(2) }, in the genericInformation.messageIdentifier field of the TerminalCapabilitySet message.

Master/Slave status shall be determined once the terminal has received the MasterSlaveDetermination message or the MasterSlaveDeterminationAck message from the remote terminal.  When such status is determined, the terminal shall select the most preferred channels and shall send the corresponding OpenLogicalChannel messages.  The terminal should start sending media without waiting to receive acknowledgement messages for its outgoing TerminalCapabilitySet, MasterSlaveDetermination or OpenLogicalChannel messages. Accelerated H.245 procedures are shown in Figure K.4/H.324.

### K.10.2 The MONA Capability Definition

Table K.16/H.324 defines the capability identifier for the mona Capability. Table K.17/H.324, Table K.17/H.324 and K.19/H.324 define the associated parameters.

Table K.16/H.324: MONA Capability Identifier

| Capability name: | Mona |
| --- | --- |
| Capability class: | Control capability. |
| Capability identifier type: | Standard. |
| Capability identifier value: | { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) mona(2) } |

Table K.17/H.324: MONA Parameter - mediaBuffering

| Parameter name: | mediaBuffering |
| --- | --- |
| Parameter description: | A terminal indicating this parameter's value as 1 is able to buffer incoming media which arrives before the relevant OLC message is received from the remote terminal, allowing faster call setup. |
| Parameter identifier value: | 3 |
| Parameter status: | Shall be present once for capability exchange |
| Parameter type: | unsignedMin, with the value 0 or 1 |
| Supersedes: | - |

Table K.18/H.324: MONA Parameter - audioEntry

| Parameter name: | audioEntry |
| --- | --- |
| Parameter description: | This parameter indicates which multiplex entry shall be reserved in the call for an audio channel established using the Accelerated H.245 Procedures. |
| Parameter identifier value: | 4 |
| Parameter status: | May be present only once for capability exchange |
| Parameter type: | unsignedMin, with the value 1 to 15 |
| Supersedes: | - |

Table K.19/H.324: MONA Parameter - videoEntry

| Parameter name: | videoEntry |
| --- | --- |
| Parameter description: | This parameter indicates which multiplex entry shall be reserved in the call for a video channel established using the Accelerated H.245 Procedures. |
| Parameter identifier value: | 5 |
| Parameter status: | May be present only once for capability exchange |
| Parameter type: | unsignedMin, with the value 1 to 15 |
| Supersedes: | - |

### K.10.3 Opening Accelerated Channels

The procedures of the current section may be used to establish outgoing channels once the remote terminal’s TerminalCapabilitySet is known.  The procedures below are only required for opening channels when no existing channels of the same media types have previously been established successfully, as described in clause K.7.

Channels opened using this procedure shall always be unidirectional channels.

The terminal can start sending media on an accelerated video channel and on an accelerated audio channel simultaneously with sending the OpenLogicalChannel messages.  The entry used for the transmission of the media shall be the one specified by the videoEntry or audioEntry parameter in the mona capability found in the outgoing TerminalCapabilitySet.  The receiver terminal shall infer the media type of the incoming media from the received OpenLogicalChannel messages. The receiver terminal shall ignore or buffer incoming media MUX-SDUs on the accelerated video and on the accelerated audio channel until the corresponding OpenLogicalChannel message is received.

If no outgoing video or audio channels are currently opened, and the relevant entry in the multiplexing table is not defined or is defined for a closed logical channel, then a terminal may send an OpenLogicalChannel message and simultaneously start sending the corresponding media.

The multiplex table may be later reconfigured by sending a MultiplexEntrySend message for channels opened using this procedure.

Figure K.4/H.324: Accelerated H.245 procedure for both audio and video channels

### K.10.4 Handling OpenLogicalChannelReject

The terminal receiving an OpenLogicalChannel using the accelerated H.245 procedure may reject the proposed channel and ignore any media received for this channel.  In this case, the terminal shall not buffer any incoming media for this media type until a new OpenLogicalChannel is received for it. Channel rejection procedures are shown in Figure K.5/H.324.

The terminal which has received the OpenLogicalChannelReject message should reopen a channel with a different media type using the regular OpenLogicalChannel procedure.

Figure K.5/H.324: Channel rejection during accelerated H.245 procedure

___________________