ITU - Telecommunication Standardization Sector		Temporary Document         (PLEN)

STUDY GROUP 16					Original: English

-------------------------------------------------

Geneva, 14-25 September, 1998

Question: 13/16

- SOURCE: 		Rapporteur for Q.13/16 (D. Skran)

TITLE: 		Draft new Recommendation H.450.6 (for determination)

Call Waiting Supplementary Service for H.323

Abstract:

This Recommendation describes the procedures and the signalling protocol for the Call Waiting supplementary service in H.323.

H.450.6 (CW) supplementary service permits a served user while being busy to be informed of an incoming call with an indication. The user then has the choice of accepting, rejecting or ignoring the waiting call. The user calling the busy party is informed about the call waiting condition.

This Recommendation makes use of the "Generic functional protocol for the support of supplementary services in H.323" as defined in Recommendation H.450.1.

- CONTACT:

- Markku Korpi, Tel. +1 408 492 7328, Email: korpim@sbs.com

- Karl Klaghofer, Tel.: +49 89 722 31488, Email: karl.klaghofer@vs.siemens.de

History of changes:

Changes made to APC-1330 Yokosuka 4/98 ‘Proposal for H.450.CallWaiting Supplementary Service for H.323’:

Clause 3: Terms and definitions added

Clause 4 – 7: Editorial changes

Clause 8.2: Added interaction with other supplementary services

Clause 5 and clause 9.4: CW-Timer value changed from 30-120 sec. to min. 30 seconds.

Clause 5: Adding statement that SS-CW applies to the complete H.323 call. The SS-CW for T.120 is f.f.s.

Changes made to APC-1378 Cannes 6/98 according to the Cannes discussions:

Clause 7.1.1 Added parameter to callWaiting Invoke APDU.

Clause 7.1.1 Removed statement regarding fastStart (comment from Nokia)

Clause 7.1.2 If T-CW expires,  ReleaseCompleteReason ´destinationRejection` shall be sent.

Clause 9.2 Added Primitives, parameters and call states for served endpoint.

Clause 9.3 Added Primitives, parameters and call states for calling endpoint.

Clause 11 Added SDLs.

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Draft H.450.6

TELECOMMUNICATION	(September, 1998)
STANDARDIZATION  SECTOR	
OF  ITU

SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Systems and terminal equipment for audiovisual services

Call Waiting Supplementary Service for H.323

ITU-T  Draft Recommendation  H.450.6

Draft Recommendation H.450.6

CONTENTS

1	Scope	7

2	References	7

3	Terms and definitions	7

4	Abbreviations and Acronyms	8

5	Description	8

6	Messages and Information elements	9

7	Procedures	9

7.1	Actions at the served endpoint B	9

7.1.1	Normal procedures	9

7.1.2	Exceptional procedures	10

7.2	Actions at the Calling endpoint C	10

7.3	Gatekeeper actions	10

8	Interworking considerations	10

8.1	Interworking with SCN	10

8.2	Interworking with other Supplementary Services	10

8.2.1	H.450.3 Call Diversion	10

8.2.2	H.450.Hold	10

8.2.3	H.450.Park+Pickup	10

9	Dynamic Description	10

9.1	Operational Models and Signalling Flows for SS-CW	10

9.2      Communication between Served endpoint and Served endpoint User	12

9.2.1	Table of primitives	12

9.2.2	Primitive definition	12

9.2.3	Parameter Definition	12

9.2.4	Call states	12

9.3	Communication between Calling endpoint and Calling endpoint User	12

9.3.1	Table of primitives	12

9.3.2	Primitive definition	13

9.3.3	Parameter Definition	13

9.3.4	Call states	13

9.4	Timers	13

10	ASN.1 Operations in support of  SS-Call Waiting	13

11	Specification and Description Language (SDL) Diagrams	15

11.1	Served endpoint SDLs	16

11.2	Calling Endpoint SDLs	18

SUMMARY

This Supplementary Service describes the procedures and the signalling protocol for the Call Waiting supplementary service (SS-CW) in H.323 (Packet Based Multimedia Communications Systems) networks.

SS-CW permits a served user while being busy to be informed of an incoming call with an indication. The user then has the choice of accepting, rejecting or ignoring the waiting call. The user calling the busy party is informed about the call waiting condition.

This Recommendation makes use of the "Generic functional protocol for the support of supplementary services in H.323" as defined in Recommendation H.450.1.

Draft Recommendation H.450.6

Call Waiting Supplementary Service for H.323

(Geneva, 19xx)

# Scope

This Recommendation specifies the Call Waiting supplementary service (SS-CW), which is applicable to various basic services supported by H.323 Multimedia Endpoints.

# References

The following ITU-T Recommendations, and other references contain provisions which, through reference in this text, constitute provisions of the Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

ITU-T Recommendation H.323 Version 2 (1998), Packet based Multimedia Communications Systems.

ITU-T Recommendation H.225.0 Version 2 (1998), Media Stream Packetization and Synchronization on Non-Guaranteed Quality of Service LANs

ITU-T Recommendation H.245 Version 3 (1998), Control Protocol for Multimedia Communication

ITU-T Recommendation H.450.1 (1998), Generic functional protocol for the support of supplementary services in H.323

# Terms and definitions

For the purposes of this Recommendation, the following definitions apply.

Busy, busy condition

User B is engaged with one or more other calls (“realtime busy”). In this busy condition, a further incoming call can only be accepted after freeing some resources, e.g. by releasing an existing call or by putting an existing call on hold.

The maximum number of calls (active calls, held calls, alerting calls, etc.) an endpoint can handle before a busy condition is encountered is implementation dependent and is therefore out of the scope of this recommendation.

As an option, a busy condition may be encountered if a user is engaged with other applications like writing e-mail. This kind of busy condition is also known as “workflow-busy”.

Camp-on

An incoming call while being busy is indicated to the served user.

Endpoint; terminal; user

See Rec. H.323

H.323 Call

Refer to Rec. H.323.

User B, served user

User B is a user who has activated the Call Waiting supplementary service for his endpoint.

User C, calling user

User C is the user who has originated a call to user B that causes the SS-CW to be invoked.

User A

User A is a user who is engaged in a call with user B.

# Abbreviations and Acronyms

For the purpose of this Recommendation the following abbreviations are used.

| ASN.1 | Abstract Syntax Notation No. 1 |
| --- | --- |
| APDU | Application Protocol Data Unit |
| GK | Gatekeeper |
| NFE | Network Facility Extension |
| PISN | Private Integrated Services Network |
| SCN | Switched Circuit Network |
| SDL | Specification and Description Language |
| SS-CFB | Supplementary Service Call Forwarding Busy |
| SS-CW | Supplementary Service Call Waiting |

# Description

The CW supplementary service (SS-CW) permits a busy user B to be informed of an incoming call while being engaged with one or more other calls.  I.e. SS-CW operates in case of an incoming call when a busy condition within the endpoint is encountered. As an option, a busy condition may also be encountered if the user is busy with workflow applications (e.g. writing e-mails).

When a user C (calling user) attempts to call a busy user B, user B is given an appropriate indication of the waiting call.

The calling user C may be informed about SS-CW being invoked at the destination by being provided with an appropriate indication.

After receiving the call waiting indication, user B has the choice of accepting, rejecting or ignoring the waiting call.

During the call waiting condition, the calling user C has the option to release the call or to invoke other supplementary services, e.g. automatic callback (f.f.s.).

The maximum number of calls that can be handled (e.g. active, held, alerting, waiting) for each endpoint is an implementation option.

Provision and Withdrawal:

SS-CW shall either be provided on a subscription basis or shall be generally available to all users without subscription.  SS-CW can be withdrawn at the request of the user or for administrative reasons.

Note: The maximum number of active calls that can be handled by an endpoint is an implementation matter. SS-CW occurs only when an attempt is made to exceed these limits.

As an option, call waiting may be offered with the following subscription option (see table 1).

| Subscription option | Value |
| --- | --- |
| Calling user receives an indication that their call is waiting due to SS-CW being invoked at the served endpoint | No Yes |
| Maximum number of calls which can be waiting | One N, where N is more than one |
| Optional Call Waiting timer (T-CW) Note 1) | min. 30 seconds |

TABLE 1/H.450.6

Subscription options

Note 1: H.225.0 Timer T301 (min. 180 seconds) operates at the calling user side.

SS-Call Waiting applies to the complete H.323 call for which the supplementary service is being invoked. SS-Call Waiting for a T.120 call is for further study.

# Messages and Information elements

The operations specified for SS-CW in clause 10 shall be sent within h4501SupplementaryService APDUs contained within H.225.0 ALERTING message.

When conveying the Invoke APDU of operations defined in clause 10, the destinationEntity data element of the NFE shall contain the value "endpoint".

When conveying the Invoke APDU of operations defined in clause 10, the

Interpretation APDU shall contain the value “discardAnyUnrecognizedInvokePdu”.

# Procedures

## Actions at the served endpoint B

### Normal procedures

When an incoming H.225.0 SETUP message from user C arrives, encounters a busy condition and SS-CW is provided for the served user, the served endpoint B shall return an ALERTING message towards the calling user C and optionally start timer T-CW.

Before sending ALERTING, a SETUP ACKNOWLEDGE or CALL PROCEEDING message (in accordance with H.225.0 basic call procedures) may be sent.

Depending on the subscription option of user B a callWaiting Invoke APDU may be included within the ALERTING message. The callWaiting Invoke APDU may be accompanied with argument nbOfAddWaitingCalls and with manufacturer specific information.

The served endpoint shall locally provide a call waiting indication to the user B.

The busy User B can free resources to accept a waiting call by:

- releasing an existing call according to the procedures of recommendation H.225.0;

- using the call hold supplementary service on an existing call according to the procedures of Rec. H.450.Hold.

- using the SS-PARK supplementary service on the existing call according to the procedures of Rec. H.450.Park+Pickup.

If the served user B accepts the waiting call, the served endpoint shall stop timer T-CW, if running, send CONNECT message to the calling user and proceed with normal call establishment procedures as described in recommendation H.323/H.225.0.

### Exceptional procedures

After the ALERTING message has been sent, this new incoming call may be rejected by user B by the sending of a RELEASE COMPLETE message with the ReleaseCompleteReason set to ´destinationRejection´.

If the optional timer T-CW is running and T-CW expires the call shall be cleared towards the calling user by the sending of a RELEASE COMPLETE message with ReleaseCompleteReason set to ´destinationRejection´.

If the calling user C clears the call attempt before the call has been established, then call clearing of the waiting call shall be performed in accordance with H.225 basic call clearing.

## Actions at the Calling endpoint C

On receipt of an ALERTING message from endpoint B containing a callWaiting Invoke APDU in response to a SETUP message, the calling endpoint may provide a call waiting indication to the calling user.

The calling user in this situation has the following options:

- wait until the waiting call gets accepted (connected) by the served user B

- release the call

- invoke other supplementary services, e.g. message waiting callback (for further study)

- perform other actions which are out of the scope of this recommendation (e.g. sending e-mail)

## Gatekeeper actions

In the case of a gatekeeper routed model, the gatekeeper shall pass on SS-CW operations transparently.

# Interworking considerations

## Interworking with SCN

SS-CW may interwork with corresponding supplementary services as defined by other standards making use of gateway interworking functions. The specification of detailed gateway interworking procedures is out of the scope of this recommendation and will be specified within other recommendations.

## Interworking with other Supplementary Services

### Call Diversion (H.450.3)

If the served user has subscribed to SS-CW and also has SS-CFB active, then SS-CFB shall take precedence.

### Call Hold (H.450.4)

The served user may invoke SS-Hold on the existing call, in order to be able to accept the waiting call.

### Call Park and Call Pickup (H.450.5)

The served user may invoke SS-Park on the existing call, in order to be able to accept the waiting call.

A waiting call may be subject to SS-PICKUP.

# Dynamic Description

## Operational Models and Signalling Flows for SS-CW

FIGURE 1/H.450.6 - Operational Model

FIGURE 2/H.450.6 Signalling Flow

## Communication between Served endpoint and Served endpoint User

### Table of primitives

| Generic name | Type |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Request (req) | Indication (ind) | Response (resp) | Confirm (conf) |
| callWaiting | PARAMETERS | - 1), 2) | - 3) | Not defined 4) |

TABLE 2/H.450.6

Primitives at the Served endpoint

- "-" means no parameters (manufacturer specific parameters may be present).

- Local indication to Served user.

- Local response from served user (callWaiting.resp_ack or callWaiting.resp_rej).

4)   means that this primitive is not defined

### Primitive definition

The callWaiting.Request primitive is used to request the sending of a callWaiting Invoke APDU to the calling User C.

The callWaiting.Indication primitive is locally sent to the served user to indicate to the served user that a call is camped-on.

The callWaiting.Response primitive is locally sent to the served user to indicate acception or rejection of the waiting call.

### Parameter Definition

callWaiting.Request parameters:

nbOfAddWaitingCalls: Indicates to the calling user the number of waiting calls at served User B in addition to the call to which this APDU applies (e.g. if the user C is the only call which camps on at user B, the value ´0´ shall be sent).

### Call states

Call_Waiting_Idle: The served endpoint is enabled for SS-CW, but SS-CW is not in progress.

Call_Waiting_Invoked: An incoming call has met User B busy, and User C has camped on User B.

## Communication between Calling endpoint and Calling endpoint User

### Table of primitives

| Generic name | Type |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Request (req) | Indication (ind) | Response (resp) | Confirm (conf) |
| callWaiting | Not defined 1) | PARAMETERS | Not defined | Not defined |

TABLE 3/H.450.6

Primitives at the Calling endpoint

1)  means that this primitive is not defined

### Primitive definition

The callWaiting.Indication primitive is used to indicate to the calling user C that SS-CW has been invoked at the served endpoint.

### Parameter Definition

callWaiting.Indication parameters:

Refer to clause 9.2.3 Parameter Definition for primitives at the served endpoint.

### Call states

Call_Waiting_Idle: The calling endpoint has been enabled to inform the calling user about SS-CW being invoked for this call at the served endpoint.

## Timers

T-CW	Call Waiting Timer (value: min. 30 seconds)

The timer is started in the served endpoint when an ALERTING message with a callWaiting Invoke APDU is sent. The Timer is stopped when a CONNECT message is sent or when the call is released.

If timer T-CW expires, call clearing shall occur by the sending of a RELEASE COMPLETE message with the ReleaseCompleteReason set to ´destinationRejection`.

# ASN.1 Operations in support of  SS-Call Waiting

Call-Waiting-Operations

{itu-t recommendation h 450 6 version1(0)  call-waiting-operations(0)}

DEFINITIONS  AUTOMATIC TAGS  ::=

BEGIN

IMPORTS	OPERATION, ERROR  FROM  Remote-Operations-Information-Objects

{ joint-iso-itu-t remote-operations (4) informationObjects (5) version1(0) }

EXTENSION, Extension { } FROM Manufacturer-specific-service-extension-definition

{ itu-t recommendation h 450 1 version1(0) msi-definition(18)}

MixedExtension  FROM Call-Hold-Operations {itu-t recommendation h 450 4

version1(0) call-hold-operations(0)};

CallWaitingOperations	 OPERATION ::=

{callWaiting | … }

callWaiting	OPERATION  ::=

{-- sent from served endpoint to the calling endpoint in ALERTING message

ARGUMENT	CallWaitingArg	OPTIONAL TRUE

RETURN RESULT		FALSE

ALWAYS RESPONDS  		FALSE

CODE	local: 105

}

CallWaitingArg ::= SEQUENCE

{ 	nbOfAddWaitingCalls	INTEGER (0..255) OPTIONAL,

-- indicates the number of waiting calls at the served user

-- in addition to the call to which this operation applies.

extensionArg	SEQUENCE SIZE (0..255) OF MixedExtension OPTIONAL,

… }

END	-- of Call-Waiting-Operations

# Specification and Description Language (SDL) Diagrams

The procedures for Call Waiting signalling entities are described in SDL form from FIGURE 4/H.450.6 to FIGURE 6/H.450.6.

The SDLs only show SS-CW specific information transported on an H.225.0 connection.  H.245 procedures (e.g. terminal capability exchange, master / slave determination, opening and closing of logical channels, etc.) are not shown.

ROSE APDUs sent via the network are indicated using bold letters with the following abbreviations:

( .inv)		Invoke APDU

For primitives and their meaning refer to clauses 9.2 and 9.3.

In case of a conflict between SDLs and the text within the previous clauses, the text shall take precedence.

The symbols used in the following SDLs are defined in FIGURE 3/H.450.6 SDL Symbols.

FIGURE 3/H.450.6

SDL Symbols

## Served endpoint SDLs

FIGURE 4/H.450.6

Served endpoint SDLs (1/2)

FIGURE 5/H.450.6

Served endpoint SDLs (2/2)

## Calling Endpoint SDLs

FIGURE 6/H.450.6

Calling endpoint SDLs (1/1)