Template for of AAP Comment Resolution table:

Summary of handling of (choose one)

[ X ] LAST CALL

[   ] ADDITIONAL REVIEW

comments to Draft Amendment 1 to  H.248.9 (2005).

1. Comments Submitted by Huawei Technologies Co. Ltd.

The table below summarizes how the comments from Huawei Technologies have been reflected to Draft Amendment 1 to H.248.9 (2005).

| # | Comments | Disposition |
| --- | --- | --- |
| 1 | It was highlighted on the IETF Megaco list that the introduction of two signal parameters with identity “st” caused implementation problems as the Signal Token is also encoded “st”. There would be a problem decoding H.248 messages if the parameter ids are adopted. A simple solution is to rename the ids. The offending signal parameters are below. - 13.3.1.1.2 Segment Type ParameterID: st (0x0002) - 13.3.2.1.2 Script Type ParameterID: st (0x0002) | As the proposal fixes an error, the changes to the ParameterIDs should be adopted. The ParameterIDs should read: - 13.3.1.1.2 Segment Type ParameterID: sgt (0x0002) - 13.3.2.1.2 Script Type ParameterID: sct (0x0002) |