ITU - Telecommunication Standardization Sector	APC-xxxx

STUDY GROUP 16

Q.12-14/16 Rapporteurs Meeting

Portland OR, USA, 21-25 August 2000

SOURCE*:		ALCATEL

TITLE:	Modification to H.323 annex H

__________________

Abstract:

In order to allow a smooth migration from the current H.323 and the H.323 with mobility support we propose to add the following requirement:

The procedures should allow an H.323 terminal without mobility function located in an H.323 domain without mobility function to reach a mobile H.323 terminal.

This contribution proposes to add this new requirement to the current chapter 7 and some modification to the current text related to this new requirement.

# Rational

To allow a smooth migration from the current  implementations of H.323 to the support of H.323 mobility it is mandatory to allow that a “non mobile” terminal can call a “mobile” terminal.

If both are located in the same domain, it shouldn’t have much impact since the procedure for the terminal won’t change as specified in the current chapter 10.6 of the mobility annex. But if the terminals are located in two different domains, there are two cases :

- Both domains supports mobility

- Only the destination domain ( where the mobile terminal is located ) supports mobility

The current specification supposes that we are in the first case. In this case we have a vlf in the originating domain (see fig19 of chapter 10.6.2).

We propose to limit the current specification to the second case and to wait to see if there is a good reason to add specific procedures for the second case. Note that the procedures that we will define will work in both cases, He, who can do more,  can do less …

# Proposition

## New chapter 7.1.X

The procedure should allow a terminal located in a domain without support of mobility to call a mobile terminal

## Modification to chapter 10.6.2

Since I don’t use the same tools as the editor, I’m not able to change the picture.

We should replace the VLF #1  from the fig 19 with a Border Element and to add the corresponding border Element in the second domain.

The Border element in the destination domain should be responsible to locate the destination gatekeeper via the HLF, and the VLF.

The interface between the two BE should be as described in annex G.