ITU Telecommunication Standardization Sector				Document AVD-2093

Study Group 16

Q.D,F,G,2-5/16 Rapporteur Meeting

Launceston, 5 - 9 March 2001

Question(s):	Q.2/16

Source*:	Cisco Systems, Inc.

Title:		H.341 State of Affairs

Purpose:	Discussion

___________________

# Introduction

While H.323 has been successful in developing services for the VoIP market, the network management aspect, specifically the SNMP MIBs, has been neglected.  The current state of H.341 standard has many issues in vary degrees of severity impacting users with large-scale deployments.

Listing of some of the current issues:

 Index (Layering) model

As defined in section 8.2 of H.341 (May 1999) specification:

In order to accommodate of an H.323 or H.320 device within a managed system, the MIB modules in this recommendation use ifIndex from the Interface MIB (RFC1573, 2233) group to delineate between the instances.

The problem is most of the MIBs do not have a direct correlation between the protocol being managed and the ifStackTable.  This is a response provided by Keith McClorgie, author of RFC 1573 and 2233.

The ifTable is defined as for use by interface sub-layers beneath

the internetwork-layer.  For example, it would not be appropriate

to represent the interface between TCP and IP by a row in the

ifTable, nor the interface between any application and the

transport protocol (TCP/UDP) it uses.  Doesn't a Gatekeeper sit

in the protocol stack above a transport protocol ?

Another way to think of this is to ask the question:

how would you represent this ifEntry in the ifStackTable ?

Keith.

The solution is to use an index for “applications providing network services”, provided by RFC 2788.

 MIB object duplication

An example is h323GwSystemUptime, which is defined in RFC 1213 MIB II as sysUpTime.

 Incorrect object descriptions

An example being connectionsReleaseCompleteReason with a description of “The data an time of the start of this call.”

 Ambiguous MIB object definitions

The MIB object description does not provide enough information on how the object interacts with the system or what the object represents, for example h323GwConfigurationPstnTotalPorts.

 Missing clauses

Missing UNITS, REFERENCE, and DEFVAL clauses for objects.

 Out-of-date with respect to the current standard

There is no network management support of RAI, T.38 fax, tunneling, etc.

The list of issues is incomplete, but should convey the need for updating all of the MIBs defined within H.341.

# Proposal

The entire set of MIBs defined by H.341 need to be re-examined and revised to support the current feature set defined by H.323.  This document is provided to describe the need.  At a minimum a discussion group needs to be formed from all interested parties to start the process of updating H.341.

# Migration Proposal

The standard method for migrating MIBs to newer a new standard is to allow older definitions to be accepted for an extended period of time in conjunction with the new standard.  This is accomplished by deprecating the affected MIB objects.  Eventually these objects are marked ‘obsolete’ when support is no longer considered necessary, in other words, enough time has passed to allow implementations to convert to the current standards and be widely used.

Upon agreeing on updated H.341 MIBs, the above method can be applied.

___________________