| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2001 - 2004 | PCM01-006 Jan/2001 Original: English |

ITU – Telecommunication Standardization Sector

STUDY GROUP 16

Las–Vegas, USA, 22–24 Jan. 2001

Question(s):	11/SG16

SOURCE: Cisco Systems

TITLE: Modem Transport Over IP Using SPRT

PURPOSE: Draft Standard

Abstract

This document provides a draft standard proposal for modem transport over IP networks utilising Simple Packet Relay Transport (SPRT) protocol.

| * Contact: Herb Wildfeuer Mario Garakani | Cisco System 150 Castilian Drive, Santa Barbara, CA 93105 | Tel: (805)961-3620 (805)961-3640 Fax: (805)962-3600 |
| --- | --- | --- |

Attention: This is not a publication made available to the public, but an internal ITU-T Document intended only for use by the Member States of the ITU, by ITU-T Sector Members and Associates, and their respective staff and collaborators in their ITU related work. It shall not be made available to, and used by, any other persons or entities without the prior written consent of the ITU-T.

Modem Over IP (MoIP) using a Simple Packet Relay Transport

1.0	Overview

This document provides interoperability guidelines for suppliers of modem-relay products.

The following items are included in this specification to provide interoperability:

- V.42/V.42bis/V.44 Termination: Modem relay implementation requires peer gateways to terminate physical layer and V42 locally (only exception is treatment of V42 breaks, which is described below). V.42bis/V.44 may also be terminated optionally. (See section 5.0) Termination for other protocol and compression algorithms TBD.

- Transport Layer.  To be interoperable the peer gateways must use Simple Packet Relay Transport (SPRT) for reliable transport of data and in-band signaling over the IP network (see Section 2.0).

- Inter-gateway Signaling/Control Messages.  Peer gateways require modem relay in-band and out-of-band signaling messages used for setup, control, and termination of modem relay (see Section 3.0).

- Treatment of Breaks.  V42 breaks are relayed to the remote end.  There is limited processing of various kinds of V42 breaks by the gateways.  The procedure for handling breaks is summarized in Section 4.0

The following items are outside the scope of this document

- Provision for non-error corrected connections

- Physical layer establishment

2.0 Transport Layer (Simple Packet Relay Transport)

SPRT is a simple packet based protocol layered on UDP/IP, which provides reliable in-sequence delivery of modem relay data across the IP network.   It is intended to result in a very small implementation .  There are a number of simplifications that have been made to keep the implementation small:

- No provision for opening and closing SPRT transport channels is included within SPRT.  This eliminates the need to maintain various associated states.  It is presumed that on transition into modem relay processing for the gateway channel, the needed SPRT channels have been opened already (external to SPRT) on both peer gateways and will be closed by the gateways on terminating the modem relay session.

- No provision to negotiate parameters associated with SPRT protocol is provided in this specification.  Peer SPRT gateways are required to have compatible parameters for max message size (bytes) and window size (i.e., number of packets transmitted without any acknowledgements).  There is no provision within the SPRT protocol to negotiate these parameters.  These SPRT parameters can be optionally negotiated using out of band mechanisms such as SDP packets in SIP/MGCP or H.245 packets for H.323.  SPRT parameter negotiation is outside the scope of this specification  Section 2.2.1 defines the default SPRT parameters which are used by the gateway in the absence of out of band negotiation.  All compliant implementations of SPRT MUST support (at a minimum) these default parameters since the out of band negotiation of SPRT parameters is optional.

2.1 Packet Structure

The basic packet structure used for in-band communication between a modem relay gateway and its peer is a “t_packet” (transport packet):

IP Header

UDP Header

Transport Sequence Number

C

# of Acks

Window Base Sequence Number

ACK 1 Sequence number

SPRT

Header

C1

ACK 2 Sequence number

C2

ACK 3 Sequence number

C3

Message Type

Session ID

Modem Relay
Content Header

Content (n bytes)

If n odd, padded to even

boundary

SPRT

Payload

16 bits

- 2.1.1 SPRT Header

C (2 bits):  Transport Channel ID:

00: Unreliable transport (plain vanilla UDP)  --- Used for acknowledgements only

01: Standard reliable transport (info and non-expedited break t_frames, and 
acknowledgements),

10: Signaling reliable transport (init, destructive breaks, expedited breaks, cap_xchg, connected, disconnect t_frames, and acknowledgements),

11: Not used by modem relay.

Transport Sequence number (14 bits):  Private identifier used by transport layer for sequencing and acknowledgements

# of Acks (2 bits):  Identifies number of ACK fields included (piggy back) in this t_frame

Window base sequence number (14 bits):  Base transport sequence number of packets not processed on receive end yet.  This identifies the sequence number for the next packet in the “ingress” direction to be processed (i.e., permanently removed from transport queue) by this gateway on the transport channel “C”.

ACK fields:  Zero to three Acknowledgements fields, each acknowledging receipt of a packet with the given transport sequence number on the receive end. C1, C2, and C3 identify the Transport Channel that the given sequence number being acknowledged corresponds to.

- 2.1.2 Modem Relay Content Header

Message Type:

0   acknowledgement              		// content is: empty

1   init					// content is:  Session ID after init,

2   info 		// content is: HDLC Address (one or two bytes), followed by I-frame content without any other data link layer header information

3   cap_xchg                   	// content is: Type (1 for v42, other types TBD)
// followed by negotiation posture in XID packet
// format for IP leg

4   connected	// content is: modulation type, error correcting 
// protocol type, additional info TBD

5   disconnect				// content is: none

64 v42_destructive break                 	// content is: new Session ID after break, and destructive break (expedited) v42 frame with v42 header

65 v42_expedited_break	// content is: expedited-break (non-destructive) 
v42 frame with v42 header

66 v42_non-expedited-break 	// content is: non-expedited v42 frame with v42 header

Session ID:  Identifies a particular epoch within this modem relay connection time-line.

Message Type space is broken into the following regions, with the possibility to add additional generic or vendor specific messages later:

- 0 through 63:  Generic modem relay layer specific

64 through 127:  Generic protocol (e.g., V42) layer specific

- 127 - 255:  Vendor specific assigned in blocks per vendor

2.2 Transport Mechanism

Information frames received from each modem, as well as gateway-to-gateway control/management messages require reliable, sequenced delivery.  This is provided by Simple Packet Relay Transport (SPRT) with runs over plain vanilla UDP.

T_frames are sequenced at the sending end, and acknowledged at the receiving end.  The acknowledgments can be sent using dedicated t_frames or can piggy-back ride (up to 3 ACKs on a single t-frame), if there is a flow in the opposite direction.  Acknowledgements will be sent on the earliest of the following three conditions (applies per transport channel C) in  order below:

- whenever there is a t_frame in the opposite direction and there is at least one received t_frame that has not been acknowledged yet,

- whenever there is at least “N1” received t_frames that have not been acknowledged yet,

- after “T1” milli-seconds if there is at least one or more t_frames that have been received but not acknowledged yet,

- after “T2” milli-seconds if no frames (acknowledgement or other type of frames) have been transmitted (to update peer with current base window sequence number).

- 2.2.1 Required Parameters for SPRT for V42 modem relay

For the peer SPRT entities to be interoperable, it is required for them to have identical window size (receive and transmit) as well as maximum SPRT payload size for each transport channel.

By default, each SPRT transport entity shall be configured with the following set of parameters for V42 modem relay:

- Standard Reliable Transport Channel (C=01) max SPRT payload size = 132 bytes

- Standard Reliable Transport Channel (C=01) max received buffered messages = TBD packets

- Standard Reliable Transport Channel (C=01) max transmitted buffered messages = TBD packets

- Signaling Reliable Transport Channel (C=10) max SPRT payload size = 132 bytes

- Signaling Reliable Transport Channel (C=10) max received buffered messages = 8 packets

- Signaling Reliable Transport Channel (C=10) max transmitted buffered messages = 8 packets

EDITORS NOTE: (We are in process of proposing in the MGCP/SIP/H323  mechanisms for negotiation of these parameters between the two gateways using the SDP packet, etc. However, such external negotiation is optional and if one or the other gateway does not chose to participate, they can do so and default values specified in Section 2.2.1 shall be used or else there will be no interoperability, which also implies all gateways must be prepared to support default values.  The parameters in Section 2.2.1 mirror N401 and k parameters used in V42, and the proposed approach uses same mechanism as V42, which is the values are negotiable, the rules of negotiation are the same as specified in V42 (which is one side proposes and the other side can chose the proposed value or a value closer to default inclusive),  the negotiation is optional and if it does not occur, then default values can be used.

3.0 Inter-gateway Signaling/Control Messages

- 3.1 Action on Session ID Changes and Signaling to Peer Gateway

Session ID is initially zero (first session), and shall not be increases before starting with end-to-end data transfer phase.  Subsequently (after data transfer phase has started) Session ID shall be increased by a modem relay gateway on following events:

- When receiving a Destructive Break on local modem line,

- When receiving a SABME command on local modem line during data transfer phase (optionally gateway may terminate the call) .

In both of the above cases, the new session ID shall be selected according to the following rule: calling leg gateway shall increase Session ID to next larger even integer; called leg gateway shall increase to next larger odd integer (this is necessary to avoid collision).  Furthermore, the gateway shall notify its peer of a change of Session ID because of a local event as follows:

- In case of a local V42 Destructive Break, the gateway  is required to send its peer a “v42_destructive_break” t_frame with its new Session ID,

- In case of a SABME command on local modem line, the gateway is required to send its peer an “init” t_frame with its new Session ID.

When receiving an “init” or “v42_destructive_break”  t_frame from its peer gateway, a gateway is required to compare its local Session ID with the new Session ID specified in the t_frame from its peer, and if the local Session ID is a smaller integer than the new Session ID from peer, the gateway is required to update its Session ID (i.e., make local Session ID equal to new Session ID specified in the t_frame).

All t_frames transmitted by a gateway shall include the gateway’s current Session ID in Modem Relay Content Header.

Furthermore a gateway is required to:

- Discard any “v42_destructive_break” or “init” t_frames from peer gateway that has a New Session ID lower than the gateway’s current Session ID,

- Discard any other t_frames from peer (other than the three types listed above) with a Session ID  lower than the gateway’s Session ID,

- Process all other t_frames as specified in Section 3.2.1 below.

3.2 In-band Signaling

Various types of t_frames are sent to peer for in-band control and management of the modem relay session.  For interoperability the gateways shall send appropriate t_frames to peer and process received t_frames as specified in Section 3.2.1, assuming the received t_frame does not have to be discarded according to the requirements in Section 3.1.

- 3.2.1 Message Types

- 3.2.1.1 Acknowledgment t_frame

Send to peer to acknowledge receipt of up to three t_frames.  The sequence number of received t_frames to acknowledge are listed in SPRT header.  Acknowledgement t_frames do not have a valid “Transport Sequence Number” (always set to zero).  They shall be processed on arrival and not be queued in sequence with other t_frames that are waiting in transport queues or to arrive later.  An Acknowledgement t_frame received on one channel (say Transport Channel “01”) may contain acknowledgements for t_frames received on the other channel (say Transport Channel "10”).

The “Base Sequence Number” in an acknowledgement t_frame transmitted on Channel C=00 is invalid (always set to zero).  Otherwise, the “Base Sequence Number” in an acknowledgement t_frame (or any other t_frame) refers to the ingress queue associated with channel “C” in SPRT header of the gateway which transmitted this t_frame.

It should be noted that the Acknowledgement t_frames provide for two separate functions, which is to acknowledge received t_frames as well as to update the peer gateway with the current value of the “Base Sequence Number”.  The later function is needed so the gateway receiving the t_frame knows whether and how many more t_frames it can source on this channel in the other direction without exceeding the transport window requirements.

Message Type = 0,

Content Length = 0,

SPRT Payload Content = null (empty).

- 3.2.1.2 Init t_frame

Send to peer to request initializing data link layer on peer.  On receiving this message a receiving gateway is required to:

- initialize its data link layer, send a SABME command to its local modem, and change its Session ID to the New Session ID, if New Session ID in t_frame is larger than gateway’s current Session ID, or

- discard this message without any further action, if New Session ID is not larger than gateway’s Session ID (see Section 3.1)

Message Type = 6,

Content Length = 2 bytes,

SPRT Payload Content = New Session ID.

- 3.2.1.3 Info t_frame

Contains data link layer HDLC address field (1 or 2 bytes) followed by I-Frame content (does not include the fields containing V42 NS or NR).  Format for non-V42 protocols TBD.

Message Type = 2,

Content Length = 2 to 130,

SPRT Payload Content = HDLC address (one or two bytes) followed by I-frame content up to 128 bytes.

- 3.2.1.4 cap_xchg t_frame

Informs peer of local negotiation posture.  This is the posture taken by each modem relay gateway, in the XID format.  The XID directional parameters have same reference as the view of the originating client modem (i.e., transmit direction refers to the direction of flow from the originating client modem towards the answering client modem).

Message Type = 3,

Content Length = 2 to 128,

SPRT Payload Content = 2 byte Type (Type=1 for V42) followed by capability in V42 XID format.

Format of non-V42 protocol connections TBD.

- 3.2.1.5 connected t_frame

Informs peer gateway that error corrected connection is up.  The gateway receiving this indication from peer shall notify its local modem that “own-receiver-busy” condition no longer exists and proceed into end-to-end data transfer phase immediately if a local establishment phase is not in progress or after successful completion of local establishment phase (if in progress).

Message Type = 4,

Content Length = 4,

SPRT Payload Content = 2 byte Modulation Type (Type=1 for V34) followed by 2 byte Error Correcting Protocol Type (Type=1 for V42).

Additional compression parameters TBD.

- 3.2.1.6 disconnect t_frame

Informs peer gateway that error corrected connection has failed.  The gateway receiving this indication from peer shall release the line.

Message Type = 5,

Content Length = 0,

SPRT Payload Content = null (empty).

- 3.2.1.7 v42_destructive_break t_frame

Informs peer gateway that a destructive break was received on the modem line.  The t_frame would contain New Session ID and v42 frame for the destructive (expedited) break.  The gateway receiving this t_frame from peer shall:

- update its Session ID according to rules specified in Section 3.1 and forward the v42 break frame to its local modem, if New Session ID in t_frame is larger than gateway’s current Session ID, or

- discard this message without any further action, if New Session ID is not larger than gateway’s Session ID (see Section 3.1)

Message Type = 64,

Content Length = n bytes,

SPRT Payload Content = 2 bytes for New Session ID followed by v42 destructive break including v42 header.

- 3.2.1.8 v42_expedited_break t_frame

Informs peer gateway that a expedited break (non-destructive) was received on the modem line.  The t_frame would contain v42 frame for the expedited (non-destructive) break.  The gateway receiving this t_frame from peer shall forward the v42 break frame to its local modem.

Message Type = 65,

Content Length = n bytes,

SPRT Payload Content = v42 expedited break including v42 header.

- 3.2.1.9 v42_non_expedited_break t_frame

Informs peer gateway that a expedited break (non-destructive) was received on the modem line.  The t_frame would contain v42 frame for the non-expedited (non-destructive) break.  The 
gateway receiving this t_frame from peer shall forward the v42 break frame to its local modem.

Message Type = 66,

Content Length = n bytes,

SPRT Payload Content = v42 non-expedited break including v42 header.

( EDITORS NOTE: Sections 3.2.2 through 3.2.6 describe an example for a modem relay gateway which use the SPRT protocol. Compatible implementations can extend beyond the described behavior, so long as the following general guidelines continue to be observed)

3.2.2 ADP/ODP Detection

The Detection Phase is entered immediately after physical layer bring up.  Depending on the “role” specified, the gateway shall take the following action:

- If the gateway is on the calling leg, the gateway shall be in the “answerer” mode and hence upon successful establishment of physical connection shall generate 1-bits (mark) until termination of detection phase, receipt of ODP, or detection of the start of the protocol phase (i.e., receiving continuous flags or of a LAPM frame).  If in the detection phase the ODP is not observed after the allowed time, gateway shall take this as an indication that local modem is not capable of V42 operation and shall implement fallback algorithm which is TBD. If ODP is observed within the given period, gateway shall take this to mean that the local modem is capable of V42 operation and shall immediately respond by sending ADP patterns at least 10 times as described in V.42.

- If the gateway is on the called leg, the gateway shall be in the “originator” mode and hence upon successful establishment of physical connection shall generate ODP patterns for the required time specified in V.42, or until an ADP pattern from the local modem is observed which indicates V42 capability.  If the ADP is not detected during the detection time, the gateway shall take this as an indication that the local modem is not capable or does not desire to have V42 operation and shall implement fallback algorithm which is TBD. If ADP is observed within the given period, the gateway shall take this to indicate that the local modem is capable of V42 operation and shall stop transmitting ODP.  Gateway shall examine the type of ADP that was received.   For the purpose of this paragraph, the only ADP that is relevant is one that indicates V42 capability by the local modem (receiving other types of ADP’s shall not result in proceeding into Establishment Phase below).

If the gateway succeeds in the detection phase, it shall proceed to Establishment Phase below.

- 3.2.3 Establishment Phase Activities for V.42 Operation

The action of the gateway is different depending on whether it is in the “answerer mode” (calling leg) or “originator mode” (called leg).  These are described in Sections 3.2.4 and 3.2.5 below

The following parameters are negotiable between the gateways over the IP leg using the cap_xchg t_frame, with the following values used as the default in case of no negotiation:

- “Maximum length of information field (N401): transmit direction” set to 128

- “Maximum length of information field (N401): receive direction” set to 128

- “V42bis P0:PV”: 11 (requesting compression in both directions)

- “V42bis P1:PV: Number of codewords” : TBD

- “V42bis P1:PV: Maximum String Length”:  TBD

An implementation which chooses not to perform compression layer transcoding would match these parameters to those used over the modem leg.

The above “compression” parameters constitute the negotiation posture of each gateway on the IP leg, which may or may not be the same as what is negotiated between a gateway and its associated client modem.

For example, although the default negotiation posture for N401 is 128 bytes, each gateway can use a smaller frame size on it’s own telephony leg.

The default compression parameters (code words and string length) are intentionally set at low values to be compatible with the widest number of client modems.  Higher values may be used by each gateway if they are mutually negotiated through t_frames (message type 3).

- 3.2.4 Calling Leg Establishment Phase for V.42 Operation

If the gateways have agreed to perform “inband negotiation ” (such indication is provided through out of band signaling such as H323, SIP, or MGCP), the compression  posture for the IP leg is exchanged using the “cap_xchg” t_frame.   “Calling” leg gateway always initiates the exchange of “cap_xchg” t_frame, to which the “called” leg gateway shall respond with a “cap_xchg” t_frame.  In regards to V42/V42bis, the main parameters that must be included in this exchange are V42 bis parameters indicating on which direction compression is to be performed, the size of dictionary, and the string length.  The content of a “cap_xchg” t_frame is in a typical XID format and would include the usual V42/V42bis parameters.  However, XID parameters related to V42 are informational (not required), but V42bis parameters MUST  be included. All directional parameters included in “cap_xchg” t_frame are with respect to the view point of the “originator” (not “answerer”) client modem.  The gateways may use different compression values for their telephony and IP legs if they wish to perform compression transcoding function, otherwise compression parameters must be matched on their telephony and IP legs.

When a SABME command is received from the local modem, the gateway shall respond to local modem with a UA followed by an RNR command with P=0, set V(S), V(R ), V(A) = 0, clear peer_receiver_busy condition and all exceptions, and proceed to notify its peer of its readiness by sending a “connected” t_frame.

After this, the gateway shall look for “connected” t_frames from its peer.   While waiting for “connected” t_frames, the gateway shall keep the local V42 session alive by responding to any supervisory poll commands from the local modem by sending an RNR response frame with F=1 and N(R ) = 0.  As soon as it received peer’s “connected” t_frame, gateway shall clear local_receiver_busy condition, start T403 (idle) timer, notify local modem by sending an RR command with P=0 and N(R ) = 0 and proceed to the data transfer phase.

Non V.42 operation TBD.

- 3.2.5 Called Leg Establishment Phase for V.42 Operation

If the gateways have agreed to perform “inband negotiation ” (such indication is provided through out of band signaling such as H323, SIP, or MGCP), the calling leg gateway shall send a “cap_xchg” t_frame to its peer indicating its desired  compression posture over the IP leg and wait for a response to this “cap_exchg” from the “called” leg gateway indicating the desired compression  posture by the peer gateway.   The gateways may use different compression values for their telephony and IP legs if they wish to perform compression transcoding function, otherwise compression parameters must be matched on their telephony and IP legs.

After XID exchange with local modem, the gateway shall  send SABME and once it receives the appropriate response, it shall proceed to notify its peer of its readiness by sending a “connected” t_frame.

Gateway shall look for “connected” t_frames from its peer (that will arrive or have been queued already).   While waiting for “connected” t_frames, gateway shall keep the local V42 session alive by responding to any supervisory poll commands from the local modem by sending an RNR response frame with F=1 and N(R ) = 0.  As soon as it received peer’s “connected” t_frame, gateway shall clear local_receiver_busy condition, start T403 (idle) timer, notify local modem by sending an RR command with P=0 and N(R ) = 0 and proceed to the data transfer phase.

Non V.42 operation TBD.

- 3.2.6 Data Transfer Phase Activities for V.42 Operation

During data transfer phase, gateway shall send the content of I-frames received from the local modem to its peer encapsulated in “info” t_frames and using transport services (SPRT).  The “info” t-frames are transported using standard transport channel (C=01).  Other types of t_frames that are used for control/management shall be sent using signaling transport channel C=10 (the exception to the later statement being “non-expedited-break” t_frame which shall use C=01).

Any “SABME” command received during the data transfer phase from the local modem shall reinitialize the local V42 stack, and result in transmission of an “init” t_frame to the peer gateway.  On receiving the “init” t_frame, the peer gateway shall send a SABME command to its modem and initialize its own V42 stack (alternatively the line can be disconnected on SABME during data transfer phase).

After “init” or “retrain” transport entities shall assign a new SESSION_ID and discard data in transit.

Non V.42 operation TBD.

- 3.3 Session Control

- 3.3.1 Out-Of-band Signaling

Other standards will cover Out-Of-Band signaling used between gateways to achieve transition into and out of modem relay operation.  These would include various extensions to H323, MGCP, and SIP required to support the described modem relay approach.

3.3.2 Failure toTransition into V42 Modem Relay State

If after placement in modem relay state, either gateway determines either of the two modems to be incapable of V.42 support, it may terminate the call or may follow a TBD fallback procedure.

.

3.3.3 Transition out of Modem Relay State

At the end of the modem session, the gateways shall release the line.

3.3.4 UDP Port Assignment for SPRT

Same port assignment as used for Voice Channel (i.e., RTP flow).

- 4.0 Transfer of Breaks (Optional)

Breaks are not processed by the local V42 stack.  They will be forwarded (relayed) to the remote modem.  Below is the treatment for the three types of breaks:

- Destructive/Expedited.  This MUST result in a change of SESSION_ID and destruction of data in transit.  The Destructive Break t_frame MUST be transported using the signaling transport channel (C=10).

- Non-Destructive/Expedited.  This MUST not result in a change of SESSION_ID or destruction of data in transit.  The Expedited Break t_frame MUST be transported using the signaling transport channel (C=10).

- Non-Destructive/Non-Expedited.  This MUST not result in a change of SESSION_ID or destruction of data in transit.  This break MUST be delivered to recipient modem after all “info” t_frames that are ahead of it have been delivered and not awaiting acknowledgement. 
Non-Expedited Breaks MUST use standard transport channel (C=01) and MUST be queued behind any “info” t_frames in transit.

5.0 Supported Configurations

Modem Relay Connection Using Link Layer Gateways

Modem Relay Connection between Link Layer and Compression Layer Gateways

Modem Relay Connection Using Compression Layer Gateways