Question: Q.13 interim Ad-hoc meeting Dec., 08th 1999

SOURCE*: Alcatel	                Contribution MTD-107b

TITLE: Annex-H (User, Terminal and Service Mobility in H.323)

SUBJECT: Clear Split of functions between Call Control and (radio) Access networks

# Introduction

Before dealing with the details of H.323 impacts of mobility, the main principles of H.323 relationship with mobility need to be settled: a clear separation of the functions between (H.323) call control functions and (possibly radio) access network functions is needed.

# Discussion

Terminal Mobility may correspond to:

- Discrete Macro Mobility:  this is the mobility linked with change of Network Point of Attachment, e.g. mobility between different network operators or between different access technologies. This kind of mobility applies to any kind of mobile (wired-line, wireless) terminal and is radio technology independent. Macro mobility applies when (radio) access network attachment occur.

- Continuous Micro Mobility: this is mobility between different cells within a radio access technology. This mobility is radio access dependant. This kind of mobility applies only to mobile wireless terminals.

- Clearly the H.323 dedicated functions (e.g. GK) are impacted by Discrete (Macro) Mobility and need to deal with:

- Notification by the user to its new H.323 Point of Attachment of its new Network Point of Attachment. (This is already covered by the Gatekeeper registration).

- Warning the Home network of the new H.323 Point of Attachment of an user

- Retrieval of the subscription information from Home H.323 Point of Attachment by the currently visited H.323 Point of Attachment

- Deletion of a previous attachment of a mobile H.323 user in a previously visited H.323 Point of Attachment

- H.323 service and protocol is concerned with Personal and Discrete (Macro) mobility but should not be concerned with micro-mobility in order to:

- Provide the same solution for both mobile wired-line and wireless access

- not to have to develop H.323 specific parts for each radio technology (GSM, D-AMPS, CDMAOne,  UMTS WB-CDMA, CDMA2000, satellite, …)

- Avoid ITU SG 16 having to interact too much with other bodies (3GPP, 3GPP2,  other ITU SG, …) responsible for the radio access technology.

- Allow different operators / different subsidiaries of an operator to provide the (possibly radio) access service and the (H.323 mobile) call control service.

Hence continuous (micro) mobility is hidden from the GK by the (possibly radio) access network, in order for the GK not to have to deal with access (e.g. radio) specific security, specific identifier, hand-over, etc..

This provides a layered architecture where the access (fixed or mobile) layer can evolve independently from the call control layer (provided by H.323) and from the service layer.

An architecture providing such a separation between (radio) access network (that deals with and hides micro-mobility) and multimedia call control (that is impacted by macro-mobility) has been chosen by 3GPP .

As a consequence, the following concepts are not relevant to H323 mobility because they correspond to radio concepts:

- “Location Area Identity (set of Network of Attachments associated)” and “Temporary Identity”. On the other hand, change of NPOA or of H323 Point of Attachment may need to be dealt with within H323 scope.

- “Paging mechanism”. On the other hand, “Finding the Mobile User for incoming call (e.g. find the serving GK of a mobile user) ” because the need is to retrieve the GK currently serving the mobile. Then  messages (e.g. set-up, LRQ,..) need to be sent to the mobile. The transmission of the first of these  messages may require paging (if the mobile is within the scope of a radio network) but this paging is under (radio) access network responsibility and not H323 scope.

# Proposal

It is hence proposed to modify Annex-H (User, Terminal and Service Mobility in H.323) (current Tdoc MTD-03b) as follows (modifications are underlined by revision marks).

Note that MTD-03b is taken as input for this contribution with all previous modifications accepted and with remarks (“from Red-Bank TD xx”) erased.

# Introduction

The high level Functional Entities needed for H.323 Mobility agreed on H.323 Mobility functional entities and very high level requirements.  New Functions to provide mobility management were identified as Home Location Function (HLF), Visitor Location Function (VLF), Authentication Function (AuF) and Inter-working Function (IWF).

H.323 Annex H

Generic Packet Network layer (e.g., generic IP  network layer framework)

Service/application layer for

- User Mobility

- Terminal Mobility

- Service Mobility

Mobility Management  Functions:

- HLF, VLF, AuF & IWF

Radio Network Access

Specific (W-CDMA, CDMA-2000, EDGE, Wireless-LAN access, etc)

Figure 1: High Level H.323 Mobility Management Functional Layers

Fixed Network Access specific (xDSL,Cable modem, Ethernet etc)

H.323 Annex I

As respective H.323 Mobility Annex H contributors introduced their documents, as listed in section 2 above, a number of requirements were identified for User, Terminal and Service Mobility capabilities:

- The Mobility Management Functional Entities needed to support H.323 Mobility Management are Home Location Function (HLF), Visitor Location Function (VLF), Authentication Functions (AuF) and Inter-working Function (IWF) which reside above the IP layer shown in figure1.

# Reference

Need TEXT

## 2.1	Abbreviations

Need TEXT

# Definitions

User: A person or other entity authorized by a subscriber to use some or all of the services subscribed to by that subscriber.

User Identity: a code or string of a particular prescribed format, uniquely identifying a user across a multi-user, multi-service infrastructure.

User Service Role:  role that a user has in the scope of a particular service.

User Service Role Identity: a code or string of a particular prescribed format, uniquely identifying a user service role instance.

H.323 Mobile Terminal: a Mobile Terminal that implements the H.323 Terminal functions as defined in the H.323 standard.

H.323 Point of Attachment: the gatekeeper with which the H.323 terminal is registered.

Mobile Terminal: a Terminal that may change its Network Point of Attachment and/or H.323 Point of Attachment.

Network Point of Attachment: the address by which packets reach an endpoint.

Services: Different types of services exist, one can distinguish:

- provided at the application layer towards subscribers, hence called applications; the type of services usually referred to within the scope of the mobility problem.

- provided by a lower communication protocol layer towards an upper protocol layer by means of a defined Service Access Point (SAP), typically some type of transport service.

Service Mobility: ability for a user to use the particular (subscribed) service irrespective of the location of the user and the terminal that is used for that purpose.

Subscriber: A person or other entity that has a contractual relationship with a service provider on behalf of one or more users. (A subscriber is responsible for the payment of charges due to that service provider.)

Service Provider Mobility: the ability of a user to change service providers and retain the same user identity.

User Terminal Profile: user-specific information, possibly contained in a multi-user database, indicating which terminals a user is using, both on a temporary or permanent basis.

Terminal Identity: a code or string of a particular prescribed format, uniquely identifying a terminal.

Terminal Mobility: the ability for a terminal to change location, network point of attachment, and H.323 point of attachment and still be able to communicate. Discrete terminal mobility (terminal roaming): The ability for a terminal to make discrete changes of location. I.e., to change location while no media streams are active. Continuous terminal mobility (handover): The ability for a terminal to change location while media streams are active. Handover is further called seamless when terminal location change does not result in delay or loss of communicated data that would ultimately be perceived by the user as degradation of quality of service (note that seamless handovers thus may depend on many factors, including service type and service presentation robustness against data loss at the terminal).

User Mobility: defined as the ability for a user to maintain the same user identity irrespective of terminals and terminal types, and so network point of attachment.

User Service Profile: user-specific information, possibly contained in a multi-user database, indicating which services a to which a user is subscribed and personal configuration data for the respective services.  Note that particular services may use the user terminal profile as input for service operation.

# 4	Scope

Provide recommendations for terminal, user and service mobility for H.323 systems, as well interworking with mobile and fixed networks.

# 5	H.323 Mobility  Functional Requirements

Editor's note:

Section 5.1 and 5.3 is from the Ad-Hoc meeting in Redbank, Following are concepts from contributions which where accepted as basis for progressing the work. Reference to TD16 Red Bank, New Jersey, USA, 18-22 October, 1999.

## 5.1	Basic Concepts that could be used to provide H.323 Mobility

The goal of H.323 mobility is to provide one Universal H323 mobile solution that supports both fixed and (possibly radio) mobile terminals. It should allow different operational situations such as different operators / different subsidiaries of an operator providing the (possibly radio) access service and the H.323 mobile service.

Identifications concepts:

- Global Unique Identity (like User Identity, Subscriber Identity, Service Provider Identity, Terminal Identity) concept

- Network Point of Attachment of a mobile H.323 user

- H.323 Point of Attachment of a Mobile User

Mobility Concepts:

- Gatekeeper discovery mechanisms needed for H.323 Mobility terminal and user to identify the service provider’s network using unique ID like System Identification (SID), preferred SID or HomeZone ID or VisitedZone ID (service provider’s serving area).

Editor's note: Note that HomeZone ID and VisitedZone ID "some members have some objection to agree on these two terms, but we will revisit these terms when we finalize our architectural solution."

- Authentication of terminal and user,

- Authorization of  service profile,

- Change of Network Point of Attachment of a mobile H.323 user

- Change of H.323 Point of Attachment of a Mobile User

- ,

- Finding the Mobile User for incoming call (e.g. find the serving GK of a Mobile User): this corresponds to finding  the GK currently serving the mobile. Then  messages (e.g. set-up, LRQ,..) need to be sent to the mobile. The transmission of the first of these  messages may require paging (if the mobile is within the scope of a radio network) but this paging is under (radio) access network responsibility and not H323 scope.

- Subscriber profile sharing between HLF and VLF as needed

Interworking Concepts:

- UIM (user identity module) used for user mobility and service mobility for existing and future Mobile cellular/PCS subscribers.

- Mapping between the HLF and HLR of PLMN

- Mapping between the AuF and AuC of PLMN

- Mapping between the VLF and VLR of PLMN

## 5.2	Mobility Management Applicable to H.323

H.323 should support the Mobility Management as  presented below:

- It provides authentication of user identification and maintains user location information in the service profile.

- Controls the completion of calls based on user-specified incoming call management contained in the service profile.

- Provides translation between user identification and identification of the terminal currently associated with the user for the completion of calls to the user’s current location.

- Controls the services and features available to the user based on the user’s subscription and in conjunction with the user-specified terminal access configurations.

# 6	Architecture for H.323 Mobility

## 6.0 Basic Architectural Speech between H.323 Mobility and Acces network

To provide a mobile H.323 network architecture independent from the way the H.323 user is accessing the network (wired-line access, terrestrial radio wireless access, satellite radio access …), H.323 is kept independent from (radio) access technology and does not handle concepts such as fixed / temporary radio identifier (e.g. IMSI, TMSI), radio security, radio hand-over…. On the other hand, the Radio access (layer of the) network (e.g. UTRA/ERAN, SGSN, GGSN in UMTS) is unaware of multimedia (H.323) signaling and carries this signaling as pure raw packet traffic. This does not prevent the mobile from requesting to the radio access bearer (layer of the) network, specific bearers adapted to the user plane flow (speech, video, data). These specific bearers are nevertheless not handled by the multimedia call control (e.g. not handled by the GK).

- A Mobile registration procedure consists logically of a bearer level registration and an application level registration. Mobile registration is performed, as an example, at Mobile power up.

- The bearer level registration procedure entails registration with the (radio) access network, according to (radio) access network procedures (e.g. attach in UMTS or e.g. PPP connection establishment with a Remote Access Server  in the wired-line case).The bearer level registration may also require IP level authentication and address assignment in certain network architectures (assignment of a NPOA).

- The application level (i.e. H323 level) registration procedure entails registration with a serving (or “visiting”) GK in order to inform the GK/VLF of the Mobile presence and to allow the GK/VLF to retrieve the user information from the HLF.  The HLF then cancels the registration in the previous serving GK/VLF.

- Bearer level registration and application level registration are considered to be two separate procedures. Bearer level registration completion may trigger in the Mobile the establishment of a (radio) access bearer as a possible means of supporting the application level registration procedure.

- The Mobile location is tracked at the (radio) access network level using (radio) access network mobility management (e.g. routing area update in UMTS), and user mobility is tracked at the application (H323) level through a specific procedure aimed at updating  the user location information maintained by the HLF.”

- HLF keeps track of the user mobility in terms of the current Serving VLF.

# Architecture Model

The architectural decomposition due to the  separation between (radio) access network (that deals with and hides continuous mobility) and multimedia call control (that is impacted by discrete mobility) is as follows

- All kind of access network are possible . In case of UMTS, Access Network part is made of  radio access ( ERAN, UTRAN), and a PLMN backbone (SGSN, GGSN).

- H.323 Multi media part - GK,VLF), HLF, GW (MGC,MG), MCU, R-SGW (signalling IWF)…

- Service part - Service and application,

- Others networks part – Multimedia IP networks, PSTN/legacy, ….

## 6.1	Functional Entities

## 6.2       Reference Points

# 7	 H.323 Roaming Interworking with Non-H.323 Networks

Editor's note: [ This section can identify which Mobile or PSTN networks this Annex will support e.g., GSM, ANSI, GPRS and PSTN from the SS7 MAP and IN perspective to H.323 perspective]

# 8 	Non-H.323 Supplementary Services Interwroking

[ This section can identify the a subsets of  the PSTN and Mobile features this Annex will support e.g., CAMEL,WIN, WAP, WIPP, and LNP from a H.323 perspective]

----- end -----

Editor's Special Note:  The interworking  referred in this annex is the interworking of legacy systems to H.323 systems. Not to be confused with interworking H.323 systems to circuit switched hybrid systems or circuit switched adjuncts. The work proposed therewith, does not impact the legacy systems or impose new requirements to the Legacy systems to support H.323 terminals or H.323 systems.

Need to add more sections to Annex-H  to comply with TOR e.g.,

Interworking:

Network interworking:

- Connections between H.323 systems and mobile networks (e.g., GSM, ANSI 41, ...)

- Connections between mobile H.323 systems and PSTN or other networks.

Terminal interworking:

Support  of non-H.323 mobile terminals (e.g., GSM handset, H.324 terminal, H.320 terminal, etc.) to communicate with H.323 systems.

Tandeming minimisation:

Non-transcoding of media streams

* Contact Person:

Laurent Thiebaut      tel: +33 (0)1 3077 0645

A L C A T E L         e.mail:laurent.thiebaut@alcatel.fr