## 7.3	Q.931 Message details

Note that the lengths of the information elements specified in the tables below refer to messages that are generated by H.323 terminals only. The size of the User-to-User Information Element shown is understood as the size of the user-data structure in H323-UserInformation and does not include the h323-UU-PDU.  The total size of H323-UserInformation is limited to 65,536 octets.  Regardless of the specified sizes, messages forwarded from the SCN side may have different (larger) sizes.

Also note that an information element specified below as mandatory, optional, or forbidden refers only to whether or not H.323 terminals may originate such information elements.

### 7.3.6	User Information

This message may be sent to provide supplementary information.  It may be used to deliver proprietary features.

This message may be sent by an H.323 entity; its processing on receipt is optional.

This message follows Table 3-7/Q.931 with the following modifications:

| Information element | H.225.0 status(M/F/O) | Length in H.225.0 |
| --- | --- | --- |
| Protocol discriminator | M | 1 |
| Call reference | M | 3 |
| Message type | M | 1 |
| Sending complete | O | 1 |
| Display | O | 2-82 |
| Keypad facility | O | 2-34 |
| Signal | O | 2-3 |
| Called party number | O | 2-35 |
| User-to-User | M | 2-131 |

Table 9/H.225.0

Information Message Content

The user-to-user information element contains the UI-UUIE defined in the H.225.0 Message Syntax. The UI-UUIE includes the following:

protocolIdentifier  - set to the version of H.225 supported

callIdentifier - a globally unique call identifier set by the originating endpoint which can be used to associate RAS signaling with the modified Q.931 signaling used in H.225.0

## 7.6	 H.225.0 Common Message Elements

The Endpoint structure is used to indicate backup, redundant, or alternative information about an endpoint:

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

aliasAddress - This is a list of alias addresses, by which other endpoints may identify this endpoint.

callSignalAddress - this is the call signaling transport address for this endpoint.

rasAddress - this is the registration and status transport address for this endpoint.

endpointType - this specifies the type of the endpoint

tokens - tokens associated with this endpoint (i.e., the endpoint described in the Endpoint structure)

cryptoTokens – cryptoTokens associated with this endpoint (i.e., the endpoint described in the Endpoint structure)

priority - used when a SEQUENCE of Endpoints is presented. Endpoints with lower priority numbers are preferred over endpoints with higher priority numbers. Endpoints without priority numbers are equivalent to those with a priority of 0 (highest priority).

remoteExtensionAddress - contains the alias address of an endpoint in cases where this information is needed to traverse multiple gateways.

destExtraCallInfo - contains external addresses for multiple calls.

The AlternateGK structure is used to indicate a list of alternative, or backup, gatekeepers:

rasAddress - the transport address used for RAS signaling.

gatekeeperIdentifier - optionally included to identify the backup or alternative gatekeeper. If it is supplied, it shall be included in future RAS messages sent to the backup gatekeeper.

needToRegister - set to TRUE to indicate that the endpoint must register with the alternative before sending other RAS requests.

priority - indicates the priority of the gatekeeper backup or alternative. A lower number implies a higher priority.

The AltGKInfo structure is used to provide information about alternate gatekeepers:

alternateGatekeeper - sequence of prioritised alternateGatekeeper for gatekeeperIdentifer and rasAddress for client  to retry the request

altGKisPermanent - TRUE if all future RAS signals should be redirected to an address from alternateGatekeeper, FALSE if only the message that caused the Reject should be redirected.

If a system wants to generate a GloballyUniqueID but has no IEEE 802 compliant network card or other source of IEEE 802 addresses, then an alternative method should be used to generate a replacement value for the address. The ideal solution is to obtain a 47 bit cryptographic quality random number, and use it as the most significant 47 bits of the node ID, with the least significant bit of the first octet of the node ID set to 1. This bit is the unicast/multicast bit, which will never be set in IEEE 802 addresses obtained from network cards; hence, there can never be a conflict between GloballyUniqueIDs generated by machines with and without network cards.

### 7.8.3 GatekeeperReject (GRJ)

The GRJ message includes the following:

requestSeqNum - This shall be the same value that was passed in the GRQ.

protocolIdentifier - identifies the vintage of the rejecting gatekeeper.

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

gatekeeperIdentifier -  string to identify gatekeeper that is sending the GRJ.

rejectReason - codes for why the GRQ was rejected by this gatekeeper.

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

### 7.9.3 RegistrationReject (RRJ)

The RRJ message includes the following:

requestSeqNum - This shall be the same value that was passed in the RRQ.

protocolIdentifier - identifies the vintage of the rejecting gatekeeper.

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

rejectReason - the reason for the rejection of the registration

gatekeeper identifier -  string to identify the gatekeeper that has rejected the terminal’s registration.

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

### 7.10.3 UnregistrationReject (URJ)

The URJ message includes the following:

requestSeqNum - This shall be the same value that was passed in the URQ.

rejectReason - the reason for the rejection of the unregistration

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

### 7.11.3 AdmissionReject (ARJ)

The ARJ message includes the following:

requestSeqNum - This shall be the same value that was passed in the ARQ.

rejectReason - reason the admission request was denied

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

callSignalAddress – This is the gatekeeper’s call signaling address returned when the reject reason is routeCallToGatekeeper.

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

### 7.12.3 BandwidthReject (BRJ)

The BRJ message includes the following:

requestSeqNum - This shall be the same value that was passed in the BRQ.

rejectReason - the reason the change was rejected by the gatekeeper.

allowedBandWidth - the maximum allowed at this time in increments of 100 bps including the current allocation.

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

### 7.13.3 LocationReject (LRJ)

The LRJ message includes the following:

requestSeqNum - This shall be the same value that was passed in the LRQ.

rejectReason - reason the location request was denied

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

### 7.14.3 DisengageReject (DRJ)

DRJ is sent by the gatekeeper if the endpoint is unregistered.

The DRJ message includes the following:

requestSeqNum - This shall be the same value that was passed in the DRQ.

rejectReason - The reason the request was rejected.

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

### 7.15.4 InfoRequestNak (INAK)

The INAK message includes the following:

requestSeqNum - This field shall contain the requestSeqNum that was in the IRR

nonStandardData - carries information not defined in this recommendation (for example, proprietary data)

nakReason - reason the IRR was negatively acknowledged.

altGKInfo – optional information about alternative gatekeepers

tokens - This is some data which may be required to allow the operation. The data shall be inserted into the message if available.

cryptoTokens - encrypted tokens

integrityCheckValue - provides improved message integrity/message authentication of the RAS messages. The cryptographically based integrity check value is computed by the sender applying a negotiated integrity algorithm and the secret key upon the entire message. Prior to integrityCheckValue computation this field shall be ignored and shall be empty. After computation, the sender puts the computed integrity check value in the integrityCheckValue field and transmits the message.

## 13.3 G.729

An RTP packet may consist of zero or more G.729 or G.729 Annex A frames, followed by zero or one G.729 Annex B payloads. The presence of a comfort noise frame can be deduced from the length of the RTP payload.

- The first packet of a talkspurt (first packet after a silence period) is distinguished by setting the marker bit in the RTP header.

- The sampling frequency (RTP clock frequency) is 8000Hz.

- The default packetization interval should have a duration of 20ms.

- Codecs should be able to encode and decode 1 - 10 consecutive frames within a single packet.

- A receiver should accept packets representing between 0 and 200ms of audio data.

# 15. ANNEX H - H.225.0 Message Syntax (ASN.1)

H.225.0 defines protocols for RAS (essentially a gatekeeper protocol) and call signaling (essentially protocol data units which reside in a User-to-user Information Element). These protocols are defined together in the following ASN.1 tree. Symantic definitions for the messages and various elements appear in subsequent sections.

H323-MESSAGES DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

SIGNED{},

ENCRYPTED{},

HASHED{},

ChallengeString,

TimeStamp,

RandomVal,

Password,

EncodedPwdCertToken,

ClearToken,

CryptoToken,

AuthenticationMechanism

FROM H235-SECURITY-MESSAGES

Capability,

MultiplexCapability,

OpenLogicalChannel

FROM MULTIMEDIA-SYSTEM-CONTROL;

H323-UserInformation ::= SEQUENCE		-- root for all Q.931 related ASN.1

{

h323-uu-pdu	H323-UU-PDU,

user-data	SEQUENCE

{

protocol-discriminator		INTEGER	(0..255),

user-information		OCTET STRING (SIZE(1..131)),

...

} OPTIONAL,

...

}

H323-UU-PDU ::= SEQUENCE

{

h323-message-body   CHOICE

{

setup			Setup-UUIE,

callProceeding		CallProceeding-UUIE,

connect			Connect-UUIE,

alerting		Alerting-UUIE,

userInformation	UI-UUIE,

releaseComplete	ReleaseComplete-UUIE,

facility		Facility-UUIE,

...,

progress		Progress-UUIE,

empty			NULL		-- used when a FACILITY message is sent,

-- but the Facility-UUIE is not to be invoked

-- (possible when transporting supplementary

-- services messages)

},

nonStandardData	NonStandardParameter OPTIONAL,

...,

h4501SupplementaryService	SEQUENCE OF OCTET STRING OPTIONAL

-- each sequence of octet string is defined as one

-- H4501SupplementaryService APDU as defined in

-- Table 3/H.450.1

}

Alerting-UUIE ::= SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

destinationInfo	EndpointType,

h245Address		TransportAddress OPTIONAL,

...,

callIdentifier		CallIdentifier,

h245SecurityMode	H245Security OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

fastStart		SEQUENCE OF OpenLogicalChannel OPTIONAL,

fastCap			FastCap OPTIONAL

}

CallProceeding-UUIE ::= SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

destinationInfo	EndpointType,

h245Address		TransportAddress OPTIONAL,

...,

callIdentifier		CallIdentifier,

h245SecurityMode	H245Security OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

fastStart		SEQUENCE OF OpenLogicalChannel OPTIONAL,

fastCap			FastCap OPTIONAL

}

Connect-UUIE ::= SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

h245Address		TransportAddress OPTIONAL,

destinationInfo	EndpointType,

conferenceID		ConferenceIdentifier,

...,

callIdentifier		CallIdentifier,

h245SecurityMode	H245Security OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

fastStart		SEQUENCE OF OpenLogicalChannel OPTIONAL,

fastCap			FastCap OPTIONAL

}

UI-UUIE	::=SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

...,

callIdentifier		CallIdentifier

}

ReleaseComplete-UUIE ::= SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

reason			ReleaseCompleteReason OPTIONAL,

...,

callIdentifier		CallIdentifier

}

ReleaseCompleteReason ::= CHOICE

{

noBandwidth		NULL,	-- bandwidth taken away or ARQ denied

gatekeeperResources	NULL,	-- exhausted

unreachableDestination	NULL,	-- no transport path to the destination

destinationRejection	NULL,	-- rejected at destination

invalidRevision	NULL,

noPermission		NULL,	-- called party’s gatekeeper rejects

unreachableGatekeeper	NULL,	-- terminal cannot reach gatekeeper for ARQ

gatewayResources	NULL,

badFormatAddress	NULL,

adaptiveBusy		NULL,	-- call is dropping due to LAN crowding

inConf			NULL,	-- no address in AlternativeAddress

undefinedReason	NULL,

...,

facilityCallDeflection	NULL,	-- call was deflected using a Facility message

securityDenied		NULL,	-- incompatible security settings

calledPartyNotRegistered	NULL,	-- used by gatekeeper when endpoint has

-- preGrantedARQ to bypass ARQ/ACF

callerNotregistered		NULL	-- used by gatekeeper when endpoint has

-- preGrantedArq to bypass ARQ/ACF

}

Setup-UUIE ::= SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

h245Address		TransportAddress OPTIONAL,

sourceAddress		SEQUENCE OF AliasAddress OPTIONAL,

sourceInfo		EndpointType,

destinationAddress	SEQUENCE OF AliasAddress OPTIONAL,

destCallSignalAddress	TransportAddress OPTIONAL,

destExtraCallInfo	SEQUENCE OF AliasAddress OPTIONAL,	-- Note(1)

destExtraCRV		SEQUENCE OF CallReferenceValue OPTIONAL,-- Note(1)

activeMC		BOOLEAN,

conferenceID		ConferenceIdentifier,

conferenceGoal		CHOICE

{

create		NULL,

join		NULL,

invite		NULL,

...,

capability-negotiation		NULL,

callIndependentSupplementaryService	NULL

},

callServices		QseriesOptions  OPTIONAL,

callType		CallType,

...,

sourceCallSignalAddress	TransportAddress OPTIONAL,

remoteExtensionAddress	AliasAddress OPTIONAL,

callIdentifier		CallIdentifier,

h245SecurityCapability	SEQUENCE OF H245Security OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

fastStart		SEQUENCE OF OpenLogicalChannel OPTIONAL,

fastCap			FastCap OPTIONAL,

canOverlapSend		BOOLEAN

}

Facility-UUIE ::= SEQUENCE

{

protocolIdentifier		ProtocolIdentifier,

alternativeAddress		TransportAddress OPTIONAL,

alternativeAliasAddress	SEQUENCE OF AliasAddress OPTIONAL,

conferenceID			ConferenceIdentifier OPTIONAL,

reason				FacilityReason,

...,

callIdentifier			CallIdentifier,

destExtraCallInfo		SEQUENCE OF AliasAddress OPTIONAL,

remoteExtensionAddress		AliasAddress OPTIONAL,

tokens				SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens			SEQUENCE OF CryptoH323Token OPTIONAL,

conferences			SEQUENCE OF ConferenceList OPTIONAL

}

ConferenceList ::= SEQUENCE

{

conferenceID			ConferenceIdentifier OPTIONAL,

conferenceAlias		AliasAddress OPTIONAL,

nonStandardData		NonStandardParameter OPTIONAL,

...

}

FacilityReason ::= CHOICE

{

routeCallToGatekeeper	NULL,		-- call must use gatekeeper model

-- gatekeeper is alternativeAddress

callForwarded		NULL,

routeCallToMC		NULL,

undefinedReason	NULL,

...,

conferenceListChoice	NULL

}

Progress-UUIE ::= SEQUENCE

{

protocolIdentifier		ProtocolIdentifier,

destinationInfo		EndpointType,

h245Address			TransportAddress OPTIONAL,

callIdentifier			CallIdentifier,

h245SecurityMode		H245Security OPTIONAL,

...,

tokens				SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens			SEQUENCE OF CryptoH323Token OPTIONAL,

fastStart			SEQUENCE OF OpenLogicalChannel OPTIONAL,

fastCap				FastCap OPTIONAL

}

TransportAddress ::= CHOICE

{

ipAddress	SEQUENCE

{

ip		OCTET STRING (SIZE(4)),

port		INTEGER(0..65535)

},

ipSourceRoute	SEQUENCE

{

ip		OCTET STRING (SIZE(4)),

port		INTEGER(0..65535),

route		SEQUENCE OF OCTET STRING(SIZE(4)),

routing 	CHOICE

{

strict	NULL,

loose	NULL,

...

},

...

},

ipxAddress	SEQUENCE

{

node	OCTET STRING (SIZE(6)),

netnum	OCTET STRING (SIZE(4)),

port	OCTET STRING (SIZE(2))

},

ip6Address	SEQUENCE

{

ip	OCTET STRING (SIZE(16)),

port	INTEGER(0..65535),

...

},

netBios			OCTET STRING (SIZE(16)),

nsap			OCTET STRING (SIZE(1..20)),

nonStandardAddress	NonStandardParameter,

...

}

EndpointType ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

vendor			VendorIdentifier OPTIONAL,

gatekeeper		GatekeeperInfo OPTIONAL,

gateway			GatewayInfo OPTIONAL,

mcu			McuInfo OPTIONAL,	-- mc must be set as well

terminal		TerminalInfo OPTIONAL,

mc			BOOLEAN,		-- shall not be set by itself

undefinedNode		BOOLEAN,

...

}

GatewayInfo ::= SEQUENCE

{

protocol		SEQUENCE OF SupportedProtocols OPTIONAL,

nonStandardData	NonStandardParameter OPTIONAL,

...

}

SupportedProtocols ::= CHOICE

{

nonStandardData	NonStandardParameter,

h310			H310Caps,

h320			H320Caps,

h321			H321Caps,

h322			H322Caps,

h323			H323Caps,

h324			H324Caps,

voice			VoiceCaps,

t120-only		T120OnlyCaps,

...,

nonStandardProtocol	NonStandardProtocol

}

H310Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H320Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H321Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H322Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H323Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H324Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

VoiceCaps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

T120OnlyCaps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

NonStandardProtocol ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix,

...

}

McuInfo ::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...

}

TerminalInfo ::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...

}

GatekeeperInfo ::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...

}

VendorIdentifier ::= SEQUENCE

{

vendor			H221NonStandard,

productId		OCTET STRING (SIZE(1..256)) OPTIONAL,	-- per vendor

versionId		OCTET STRING (SIZE(1..256)) OPTIONAL,	-- per product

...

}

H221NonStandard ::= SEQUENCE

{	t35CountryCode		INTEGER(0..255),	-- country, as per T.35

t35Extension		INTEGER(0..255),	-- assigned nationally

manufacturerCode	INTEGER(0..65535),	-- assigned nationally

...

}

NonStandardParameter ::= SEQUENCE

{

nonStandardIdentifier	NonStandardIdentifier,

data			OCTET STRING

}

NonStandardIdentifier		::=CHOICE

{

object			OBJECT IDENTIFIER,

h221NonStandard	H221NonStandard,

...

}

AliasAddress ::= CHOICE

{

e164		IA5String (SIZE (1..128)) (FROM (“0123456789#*,")),

h323-ID		BMPString (SIZE (1..256)),	-- Basic ISO/IEC 10646-1 (Unicode)

...,

url-ID		IA5String (SIZE(1..512)),	-- URL style address

transportID	TransportAddress,

email-ID	IA5String (SIZE(1..512)),	-- rfc822-compliant email address

partyNumber	PartyNumber

}

PartyNumber ::= CHOICE

{

publicNumber PublicPartyNumber,

-- the numbering plan is according to

-- Recommendations E.163 and E.164.

dataPartyNumber NumberDigits,

-- not used, value reserved.

telexPartyNumber NumberDigits,

-- not used, value reserved.

privateNumber PrivatePartyNumber,

nationalStandardPartyNumber NumberDigits,

-- not used, value reserved.

...

}

PublicPartyNumber ::= SEQUENCE

{

publicTypeOfNumber	PublicTypeOfNumber,

publicNumberDigits	NumberDigits

}

PrivatePartyNumber ::= SEQUENCE

{

privateTypeOfNumber	PrivateTypeOfNumber,

privateNumberDigits	NumberDigits

}

NumberDigits ::= IA5String (SIZE (1..128)) (FROM (“0123456789#*,”))

PublicTypeOfNumber ::= CHOICE

{

unknown			NULL,

-- if used number digits carry prefix indicating type

-- of number according to national recommendations.

internationalNumber	NULL,

nationalNumber		NULL,

networkSpecificNumber	NULL,

-- not used, value reserved

subscriberNumber	NULL,

abbreviatedNumber	NULL,

-- valid only for called party number at the outgoing

-- access, network substitutes appropriate number.

...

}

PrivateTypeOfNumber ::= CHOICE

{

unknown			NULL,

level2RegionalNumber	NULL,

level1RegionalNumber	NULL,

pISNSpecificNumber	NULL,

localNumber		NULL,

abbreviatedNumber	NULL,

...

}

Endpoint ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

aliasAddress		SEQUENCE OF AliasAddress OPTIONAL,

callSignalAddress	SEQUENCE OF TransportAddress OPTIONAL,

rasAddress		SEQUENCE OF TransportAddress OPTIONAL,

endpointType		EndpointType OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

priority		INTEGER(0..127) OPTIONAL,

remoteExtensionAddress	SEQUENCE OF AliasAddress OPTIONAL,

destExtraCallInfo	SEQUENCE OF AliasAddress OPTIONAL,

...

}

AlternateGK ::= SEQUENCE

{

rasAddress		TransportAddress,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

needToRegister		BOOLEAN,

priority		INTEGER (0..127),

...

}

AltGKInfo ::= SEQUENCE

{

alternateGatekeeper	SEQUENCE OF AlternateGK,

altGKisPermanent	BOOLEAN,

...

}

SecurityServiceMode ::= CHOICE

{

nonStandard		NonStandardParameter,

none			NULL,

default			NULL,

...				-- can be extended with other specific modes

}

SecurityCapabilities ::= SEQUENCE

{

nonStandard		NonStandardParameter OPTIONAL,

encryption		SecurityServiceMode,

authenticaton		SecurityServiceMode,

integrity		SecurityServiceMode,

...

}

H245Security ::= CHOICE

{

nonStandard		NonStandardParameter,

noSecurity		NULL,

tls			SecurityCapabilities,

ipsec			SecurityCapabilities,

...

}

FastCap ::= SEQUENCE

{

caps		SEQUENCE OF Capability OPTIONAL, -- Capability structure from H.245

multiplexCap	MultiplexCapability OPTIONAL, -- MultiplexCapability structure from H.245

...

}

QseriesOptions ::= SEQUENCE

{

q932Full	BOOLEAN,	-- if true, indicates full support for Q.932

q951Full	BOOLEAN, 	-- if true, indicates full support for Q.951

q952Full	BOOLEAN, 	-- if true, indicates full support for Q.952

q953Full	BOOLEAN, 	-- if true, indicates full support for Q.953

q955Full	BOOLEAN, 	-- if true, indicates full support for Q.955

q956Full	BOOLEAN, 	-- if true, indicates full support for Q.956

q957Full	BOOLEAN, 	-- if true, indicates full support for Q.957

q954Info	Q954Details,

...

}

Q954Details ::= SEQUENCE

{

conferenceCalling	BOOLEAN,

threePartyService	BOOLEAN,

...

}

GloballyUniqueID	::=	OCTET STRING (SIZE(16))

ConferenceIdentifier	::=	GloballyUniqueID

RequestSeqNum		::=	INTEGER (1..65535)

GatekeeperIdentifier	::=	BMPString (SIZE(1..128))

BandWidth		::=	INTEGER (0.. 4294967295)	-- in 100s of bits

CallReferenceValue	::=	INTEGER (0..65535)

EndpointIdentifier	::=	BMPString (SIZE(1..128))

ProtocolIdentifier	::=	OBJECT IDENTIFIER

-- shall be set to

-- {itu-t (0) recommendation (0) h (8) 2250 version (0) 2}

TimeToLive		::=	INTEGER (1..4294967295)		--in seconds

CallIdentifier ::= SEQUENCE

{

guid		GloballyUniqueID,

...

}

EncryptIntAlg ::= CHOICE

{	-- core encryption algorithms for RAS message integrity

nonStandard		NonStandardParameter,

isoAlgorithm		OBJECT IDENTIFIER,	-- defined in ISO/IEC 9979

...

}

NonIsoIntegrityMechanism ::= CHOICE

{	-- HMAC mechanism used, no truncation, tagging may be necessary!

hMAC-MD5		NULL,

hMAC-iso10118-2-s	EncryptIntAlg,	-- according to ISO/IEC 10118-2 using

-- EncryptIntAlg as core block encryption algorithm

-- (short MAC)

hMAC-iso10118-2-l	EncryptIntAlg,	-- according to ISO/IEC 10118-2 using

-- EncryptIntAlg as core block encryption algorithm

-- (long MAC)

hMAC-iso10118-3	OBJECT IDENTIFIER,  -- according to ISO/IEC 10118-3 using

-- OID as hash function (OID is SHA-1, RIPE-MD160,

-- RIPE-MD128)

...

}

IntegrityMechanism ::= CHOICE

{	-- for RAS message integrity

nonStandard		NonStandardParameter,

digSig			NULL,	-- indicates to apply a digital signature

iso9797			OBJECT IDENTIFIER,	-- according to ISO/IEC 9797 using OID as

-- core encryption algorithm (X-CBC MAC)

nonIsoIM		NonIsoIntegrityMechanism,

...

}

ICV ::= SEQUENCE

{

algorithmOID	OBJECT IDENTIFIER,  -- the algorithm used to compute the signature

icv		BIT STRING	-- the computed cryptographic integrity check value

-- or signature

}

FastStartToken ::= ClearToken (WITH COMPONENTS {..., timeStamp, dhkey, generalID -- set to ‘alias’ –- })

EncodedFastStartToken ::= TYPE-IDENTIFIER.&Type (FastStartToken)

CryptoH323Token::= CHOICE

{

cryptoEPPwdHash SEQUENCE

{

alias		AliasAddress, -- alias of entity generating hash

timeStamp	TimeStamp, -- timestamp used in hash

token 		HASHED	{ EncodedPwdCertToken –- generalID set to ‘alias’ -- }

},

cryptoGKPwdHash  SEQUENCE

{

gatekeeperId	GatekeeperIdentifier, -- GatekeeperID of GK generating hash

timeStamp	TimeStamp, -- timestamp used in hash

token		HASHED 	{ EncodedPwdCertToken  -- generalID set to Gatekeeperid -- }

},

cryptoEPPwdEncr	ENCRYPTED

{ EncodedPwdCertToken –- generalID set to Gatekeeperid --},

cryptoGKPwdEncr	ENCRYPTED

{ EncodedPwdCertToken –- generalID set to Gatekeeperid --},

cryptoEPCert		SIGNED { EncodedPwdCertToken –- generalID set to Gatekeeperid -- },

cryptoGKCert   	SIGNED { EncodedPwdCertToken –- generalID set to alias -- },

cryptoFastStart	SIGNED { EncodedFastStartToken },

nestedcryptoToken	CryptoH323Token,

...

}

DataRate ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

channelRate		BandWidth,

channelMultiplier	INTEGER (1..256) OPTIONAL,

...

}

SupportedPrefix ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

prefix			AliasAddress,

...

}

RasMessage ::= CHOICE

{

gatekeeperRequest		GatekeeperRequest,

gatekeeperConfirm		GatekeeperConfirm,

gatekeeperReject		GatekeeperReject,

registrationRequest		RegistrationRequest,

registrationConfirm		RegistrationConfirm,

registrationReject		RegistrationReject,

unregistrationRequest		UnregistrationRequest,

unregistrationConfirm		UnregistrationConfirm,

unregistrationReject		UnregistrationReject,

admissionRequest		AdmissionRequest,

admissionConfirm		AdmissionConfirm,

admissionReject		AdmissionReject,

bandwidthRequest		BandwidthRequest,

bandwidthConfirm		BandwidthConfirm,

bandwidthReject		BandwidthReject,

disengageRequest		DisengageRequest,

disengageConfirm		DisengageConfirm,

disengageReject		DisengageReject,

locationRequest		LocationRequest,

locationConfirm		LocationConfirm,

locationReject			LocationReject,

infoRequest			InfoRequest,

infoRequestResponse		InfoRequestResponse,

nonStandardMessage		NonStandardMessage,

unknownMessageResponse		UnknownMessageResponse,

...,

requestInProgress		RequestInProgress,

resourcesAvailableIndicate	ResourcesAvailableIndicate,

resourcesAvailableConfirm	ResourcesAvailableConfirm,

infoRequestAck			InfoRequestAck,

infoRequestNak			InfoRequestNak

}

GatekeeperRequest ::= SEQUENCE --(GRQ)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

rasAddress		TransportAddress,

endpointType		EndpointType,

gatekeeperIdentifier	GatekeeperIdentifier OPTIONAL,

callServices		QseriesOptions  OPTIONAL,

endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,

...,

alternateEndpoints	SEQUENCE OF Endpoint OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

authenticationCapability	SEQUENCE OF AuthenticationMechanism OPTIONAL,

algorithmOIDs		SEQUENCE OF OBJECT IDENTIFIER OPTIONAL,

integrity		SEQUENCE OF IntegrityMechanism OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

GatekeeperConfirm ::= SEQUENCE --(GCF)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

rasAddress		TransportAddress,

...,

alternateGatekeeper	SEQUENCE OF AlternateGK OPTIONAL,

authenticationMode	AuthenticationMechanism OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

algorithmOID		OBJECT IDENTIFIER OPTIONAL,

integrity		SEQUENCE OF IntegrityMechanism OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

GatekeeperReject ::= SEQUENCE --(GRJ)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

rejectReason		GatekeeperRejectReason,

...,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

GatekeeperRejectReason ::= CHOICE

{

resourceUnavailable		NULL,

terminalExcluded		NULL,	-- permission failure, not a resource failure

invalidRevision		NULL,

undefinedReason		NULL,

...,

securityDenial			NULL

}

RegistrationRequest ::= SEQUENCE --(RRQ)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

discoveryComplete	BOOLEAN,

callSignalAddress	SEQUENCE OF TransportAddress,

rasAddress		SEQUENCE OF TransportAddress,

terminalType		EndpointType,

terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

endpointVendor		VendorIdentifier,

...,

alternateEndpoints	SEQUENCE OF Endpoint OPTIONAL,

timeToLive		TimeToLive OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

keepAlive		BOOLEAN,

endpointIdentifier	EndpointIdentifier OPTIONAL,

willSupplyUUIEs	BOOLEAN

}

RegistrationConfirm ::= SEQUENCE --(RCF)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

callSignalAddress	SEQUENCE OF TransportAddress,

terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

endpointIdentifier	EndpointIdentifier,

...,

alternateGatekeeper	SEQUENCE OF AlternateGK OPTIONAL,

timeToLive		TimeToLive OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

willRespondToIRR	BOOLEAN,

preGrantedARQ		SEQUENCE

{

makeCall				BOOLEAN,

useGKCallSignalAddressToMakeCall	BOOLEAN,

answerCall				BOOLEAN,

useGKCallSignalAddressToAnswer	BOOLEAN,

...

} OPTIONAL

}

RegistrationReject ::= SEQUENCE --(RRJ)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

rejectReason		RegistrationRejectReason,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

...,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

RegistrationRejectReason ::= CHOICE

{

discoveryRequired		NULL,	-- registration permission has aged

invalidRevision		NULL,

invalidCallSignalAddress	NULL,

invalidRASAddress		NULL,	-- supplied address is invalid

duplicateAlias			SEQUENCE OF AliasAddress,

-- alias registered to another endpoint

invalidTerminalType		NULL,

undefinedReason		NULL,

transportNotSupported		NULL,	-- one or more of the transports

...,

transportQOSNotSupported	NULL,	-- endpoint QoS not supported

resourceUnavailable		NULL,	-- gatekeeper resources exhausted

invalidAlias			NULL,	-- alias not consistent with gatekeeper rules

securityDenial			NULL

}

UnregistrationRequest ::= SEQUENCE --(URQ)

{

requestSeqNum		RequestSeqNum,

callSignalAddress	SEQUENCE OF TransportAddress,

endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,

nonStandardData	NonStandardParameter OPTIONAL,

endpointIdentifier	EndpointIdentifier OPTIONAL,

...,

alternateEndpoints	SEQUENCE OF Endpoint OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

reason			UnregRequestReason OPTIONAL

}

UnregRequestReason ::= CHOICE

{

reregistrationRequired		NULL,

ttlExpired			NULL,

securityDenial			NULL,

undefinedReason		NULL,

...

}

UnregistrationConfirm ::= SEQUENCE --(UCF)

{

requestSeqNum		RequestSeqNum,

nonStandardData	NonStandardParameter OPTIONAL,

...,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

UnregistrationReject ::= SEQUENCE --(URJ)

{

requestSeqNum		RequestSeqNum,

rejectReason		UnregRejectReason,

nonStandardData	NonStandardParameter OPTIONAL,

...,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

UnregRejectReason ::= CHOICE

{

notCurrentlyRegistered	NULL,

callInProgress		NULL,

undefinedReason	NULL,

...,

permissionDenied	NULL,	-- requesting user not allowed to unregister

-- specified user

securityDenial		NULL

}

AdmissionRequest ::= SEQUENCE --(ARQ)

{

requestSeqNum		RequestSeqNum,

callType		CallType,

callModel		CallModel OPTIONAL,

endpointIdentifier	EndpointIdentifier,

destinationInfo	SEQUENCE OF AliasAddress OPTIONAL,	--Note 1

destCallSignalAddress	TransportAddress OPTIONAL,		--Note 1

destExtraCallInfo	SEQUENCE OF AliasAddress OPTIONAL,

srcInfo			SEQUENCE OF AliasAddress,

srcCallSignalAddress	TransportAddress OPTIONAL,

bandWidth		BandWidth,

callReferenceValue	CallReferenceValue,

nonStandardData	NonStandardParameter OPTIONAL,

callServices		QseriesOptions  OPTIONAL,

conferenceID		ConferenceIdentifier,

activeMC		BOOLEAN,

answerCall		BOOLEAN,	-- answering a call

...,

canMapAlias		BOOLEAN,	-- can handle alias address

callIdentifier		CallIdentifier,

srcAlternatives	SEQUENCE OF Endpoint OPTIONAL,

destAlternatives	SEQUENCE OF Endpoint OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

transportQOS		TransportQOS OPTIONAL,

willSupplyUUIEs	BOOLEAN

}

CallType ::= CHOICE

{

pointToPoint	NULL,		-- Point to point

oneToN		NULL,		-- no interaction (FFS)

nToOne		NULL,		-- no interaction (FFS)

nToN		NULL,		-- interactive (multipoint)

...

}

CallModel ::= CHOICE

{

direct			NULL,

gatekeeperRouted	NULL,

...

}

TransportQOS ::= CHOICE

{

endpointControlled	NULL,

gatekeeperControlled	NULL,

noControl		NULL,

...

}

AdmissionConfirm ::= SEQUENCE --(ACF)

{

requestSeqNum			RequestSeqNum,

bandWidth			BandWidth,

callModel			CallModel,

destCallSignalAddress		TransportAddress,

irrFrequency			INTEGER (1..65535) OPTIONAL,

nonStandardData		NonStandardParameter OPTIONAL,

...,

destinationInfo		SEQUENCE OF AliasAddress OPTIONAL,

destExtraCallInfo		SEQUENCE OF AliasAddress OPTIONAL,

destinationType		EndpointType OPTIONAL,

remoteExtensionAddress		SEQUENCE OF AliasAddress OPTIONAL,

alternateEndpoints		SEQUENCE OF Endpoint OPTIONAL,

tokens				SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens			SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue		ICV OPTIONAL,

transportQOS			TransportQOS OPTIONAL,

willRespondToIRR		BOOLEAN,

uuiesRequested			UUIEsRequested

}

UUIEsRequested ::= SEQUENCE

{

setup			BOOLEAN,

callProceeding		BOOLEAN,

connect			BOOLEAN,

alerting		BOOLEAN,

userInformation	BOOLEAN,

releaseComplete	BOOLEAN,

facility		BOOLEAN,

progress		BOOLEAN,

empty			BOOLEAN,

...

}

AdmissionReject ::= SEQUENCE --(ARJ)

{

requestSeqNum		RequestSeqNum,

rejectReason		AdmissionRejectReason,

nonStandardData	NonStandardParameter OPTIONAL,

...,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

callSignalAddress	SEQUENCE OF TransportAddress OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

AdmissionRejectReason ::= CHOICE

{

calledPartyNotRegistered	NULL,	-- can’t translate address

invalidPermission		NULL,	-- permission has expired

requestDenied			NULL,	-- no bandwidth available

undefinedReason		NULL,

callerNotRegistered		NULL,

routeCallToGatekeeper		NULL,

invalidEndpointIdentifier	NULL,

resourceUnavailable		NULL,

...,

securityDenial			NULL,

qosControlNotSupported		NULL,

incompleteAddress		NULL

}

BandwidthRequest ::= SEQUENCE --(BRQ)

{

requestSeqNum		RequestSeqNum,

endpointIdentifier	EndpointIdentifier,

conferenceID		ConferenceIdentifier,

callReferenceValue	CallReferenceValue,

callType		CallType OPTIONAL,

bandWidth		BandWidth,

nonStandardData	NonStandardParameter OPTIONAL,

...,

callIdentifier		CallIdentifier,

gatekeeperIdentifier	GatekeeperIdentifier,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

answeredCall		BOOLEAN

}

BandwidthConfirm ::= SEQUENCE --(BCF)

{

requestSeqNum		RequestSeqNum,

bandWidth		BandWidth,

nonStandardData	NonStandardParameter OPTIONAL,

...,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

BandwidthReject ::= SEQUENCE --(BRJ)

{

requestSeqNum		RequestSeqNum,

rejectReason		BandRejectReason,

allowedBandWidth	BandWidth,

nonStandardData	NonStandardParameter OPTIONAL,

...,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

BandRejectReason ::= CHOICE

{

notBound			NULL,	-- discovery permission has aged

invalidConferenceID		NULL,	-- possible revision

invalidPermission		NULL,	-- true permission violation

insufficientResources		NULL,

invalidRevision		NULL,

undefinedReason		NULL,

...,

securityDenial			NULL

}

LocationRequest ::= SEQUENCE --(LRQ)

{

requestSeqNum		RequestSeqNum,

endpointIdentifier	EndpointIdentifier OPTIONAL,

destinationInfo	SEQUENCE OF AliasAddress,

nonStandardData	NonStandardParameter OPTIONAL,

replyAddress		TransportAddress,

...,

sourceInfo		SEQUENCE OF AliasAddress OPTIONAL,

canMapAlias		BOOLEAN,	-- can handle alias address

gatekeeperIdentifier	GatekeeperIdentifier OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

LocationConfirm ::= SEQUENCE --(LCF)

{

requestSeqNum			RequestSeqNum,

callSignalAddress		TransportAddress,

rasAddress			TransportAddress,

nonStandardData		NonStandardParameter OPTIONAL,

...,

destinationInfo		SEQUENCE OF AliasAddress OPTIONAL,

destExtraCallInfo		SEQUENCE OF AliasAddress OPTIONAL,

destinationType		EndpointType OPTIONAL,

remoteExtensionAddress		SEQUENCE OF AliasAddress OPTIONAL,

alternateEndpoints		SEQUENCE OF Endpoint OPTIONAL,

tokens				SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens			SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue		ICV OPTIONAL

}

LocationReject ::= SEQUENCE --(LRJ)

{

requestSeqNum		RequestSeqNum,

rejectReason		LocationRejectReason,

nonStandardData	NonStandardParameter OPTIONAL,

...,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

LocationRejectReason ::= CHOICE

{

notRegistered		NULL,

invalidPermission	NULL,	-- exclusion by administrator or feature

requestDenied		NULL,	-- can't find location

undefinedReason	NULL,

...,

securityDenial		NULL

}

DisengageRequest ::= SEQUENCE --(DRQ)

{

requestSeqNum		RequestSeqNum,

endpointIdentifier	EndpointIdentifier,

conferenceID		ConferenceIdentifier,

callReferenceValue	CallReferenceValue,

disengageReason	DisengageReason,

nonStandardData	NonStandardParameter OPTIONAL,

...,

callIdentifier		CallIdentifier,

gatekeeperIdentifier	GatekeeperIdentifier,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

answeredCall		BOOLEAN

}

DisengageReason ::= CHOICE

{

forcedDrop		NULL,	-- gatekeeper is forcing the drop

normalDrop		NULL,	-- associated with normal drop

undefinedReason	NULL,

...

}

DisengageConfirm ::= SEQUENCE --(DCF)

{

requestSeqNum		RequestSeqNum,

nonStandardData	NonStandardParameter OPTIONAL,

...,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

DisengageReject ::= SEQUENCE --(DRJ)

{

requestSeqNum		RequestSeqNum,

rejectReason		DisengageRejectReason,

nonStandardData	NonStandardParameter OPTIONAL,

...,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

DisengageRejectReason ::= CHOICE

{

notRegistered		NULL,	-- not registered with gatekeeper

requestToDropOther	NULL, 	-- can’t request drop for others

...,

securityDenial		NULL

}

InfoRequest ::= SEQUENCE --(IRQ)

{

requestSeqNum		RequestSeqNum,

callReferenceValue	CallReferenceValue,

nonStandardData	NonStandardParameter OPTIONAL,

replyAddress		TransportAddress OPTIONAL,

...,

callIdentifier		CallIdentifier,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

uuiesRequested		UUIEsRequested OPTIONAL

}

InfoRequestResponse ::= SEQUENCE --(IRR)

{

nonStandardData	NonStandardParameter OPTIONAL,

requestSeqNum		RequestSeqNum,

endpointType		EndpointType,

endpointIdentifier	EndpointIdentifier,

rasAddress		TransportAddress,

callSignalAddress	SEQUENCE OF TransportAddress,

endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,

perCallInfo		SEQUENCE OF SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

callReferenceValue	CallReferenceValue,

conferenceID		ConferenceIdentifier,

originator		BOOLEAN OPTIONAL,

audio			SEQUENCE OF RTPSession OPTIONAL,

video			SEQUENCE OF RTPSession OPTIONAL,

data			SEQUENCE OF TransportChannelInfo OPTIONAL,

h245			TransportChannelInfo,

callSignaling		TransportChannelInfo,

callType		CallType,

bandWidth		BandWidth,

callModel		CallModel,

...,

callIdentifier		CallIdentifier,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

substituteConfIDs	SEQUENCE OF ConferenceIdentifier,

pdu			SEQUENCE OF SEQUENCE

{

h323pdu		H323-UU-PDU,

sent		BOOLEAN		-- TRUE is sent, FALSE is received

} OPTIONAL

} OPTIONAL,

...,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

needResponse		BOOLEAN

}

TransportChannelInfo ::= SEQUENCE

{

sendAddress		TransportAddress OPTIONAL,

recvAddress		TransportAddress OPTIONAL,

...

}

RTPSession ::= SEQUENCE

{

rtpAddress		TransportChannelInfo,

rtcpAddress		TransportChannelInfo,

cname			PrintableString,

ssrc			INTEGER (1..4294967295),

sessionId		INTEGER (1..255),

associatedSessionIds	SEQUENCE OF INTEGER (1..255),

...

}

InfoRequestAck ::= SEQUENCE --(IACK)

{

requestSeqNum		RequestSeqNum,

nonStandardData	NonStandardParameter OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

InfoRequestNak ::= SEQUENCE --(INAK)

{

requestSeqNum		RequestSeqNum,

nonStandardData	NonStandardParameter OPTIONAL,

nakReason		InfoRequestNakReason,

altGKInfo		AltGKInfo OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

InfoRequestNakReason ::= CHOICE

{

notRegistered		NULL,	-- not registered with gatekeeper

securityDenial		NULL,

undefinedReason	NULL,

...

}

NonStandardMessage ::= SEQUENCE

{

requestSeqNum		RequestSeqNum,

nonStandardData	NonStandardParameter,

...,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

UnknownMessageResponse ::= SEQUENCE	-- (XRS)

{

requestSeqNum	RequestSeqNum,

...,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL

}

RequestInProgress ::= SEQUENCE	-- (RIP)

{

requestSeqNum		RequestSeqNum,

nonStandardData	NonStandardParameter	OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

delay			INTEGER(1..65535),

...

}

ResourcesAvailableIndicate ::= SEQUENCE --(RAI)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

endpointIdentifier	EndpointIdentifier,

protocols		SEQUENCE OF SupportedProtocols,

almostOutOfResources	BOOLEAN,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

ResourcesAvailableConfirm ::= SEQUENCE --(RAC)

{

requestSeqNum		RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

END		-- of ASN.1

###### 19.1.1.2.2.1 The SRV Resource Record Query

The first solution uses the fact that the gatekeeper is basically a system service, and the transport address of a named system service can be extracted from DNS by using a query for a new type of DNS Resource Record, called SRV (for “service location record”). Given a domain name, an SRV record query will be made for the transport address of the ras service for that domain. The domain name itself, or one returned in the SRV response, is used as the gatekeeperIdentifier.

[editor’s note: The proposal is to register with IANA the name “ras” for the gatekeeper service of H323.]

This simple solution will soon be standard. The problem is that almost no current DNS client or server implementations support the SRV resource record yet. Unless the DNS client knows about the SRV resource record type, it is not possible for it to pass on queries for this resource record. Until this support becomes widespread, there is a reasonable chance that the SRV query will fail.

###### 19.1.1.2.2.2 The TXT record query

All current DNS implementations support the TXT resource record. Basically this is some free text that can be returned for each domain name. It is possible to store many TXT resources for a single domain. The standard stipulates that all TXT records will be returned when a query is made for them.

It is possible to use TXT queries if the SRV queries fail. Assume the same convention for extracting a domain name that was suggested above. Either RFC822 compliant strings (email “-like” names) or IETF RFC1768 compliant strings (URLs) can be used for gatekeeperIdentifiers. In either case the domain name is used to make a DNS TXT query  for the domain name. The returned resource records are lines of free text, and the terminal will then look for lines in the response of the form:

ras [< gk id>@]<domain name >[:<portno>] [<priority>]